Enterprise Network Standards

## Introduction

Welcome to the Network Engineering Department at Sonoran Outdoor Equipment (SOE).

This guide establishes the network engineering standards, naming conventions, documentation requirements, and configuration practices used throughout the organization. As a Junior Network Engineer, you are expected to follow these standards while completing each Enterprise Change Request (ECR).

Enterprise networks should be carefully planned before they are configured. Every change should be designed, implemented, verified, and documented. Unless otherwise directed, all Enterprise Change Requests (ECRs) should follow the standards contained in this guide.

### Related Documentation

The following documents work together to define the standards and procedures used throughout this course:

- Company Overview – Introduces Sonoran Outdoor Equipment and the enterprise environment.

- Enterprise Network Standards Guide – Defines enterprise design standards, naming conventions, and configuration requirements.

- Lab Process & Expectations Guide – Defines lab procedures, documentation standards, submission requirements, and grading expectations.

- Enterprise Interface Addressing Plan – Defines the IPv4 and IPv6 interface addressing assignments used throughout the enterprise.

- Enterprise Change Requests (ECRs) – Provide the business requirements and technical tasks for each implementation.

## Your Role

As a Junior Network Engineer, you are responsible for assisting the Network Engineering Team with planning, implementing, verifying, troubleshooting, and documenting changes to the enterprise network. Network engineers are expected to communicate professionally, follow enterprise standards, and maintain accurate documentation throughout every phase of an implementation.

Throughout this course you will receive Enterprise Change Requests (ECRs) from members of the Information Technology Department. Each request represents a real business need requiring changes to the enterprise network.

Professional documentation and verification are considered part of every implementation.

## Enterprise Locations

The enterprise currently consists of three operational locations:

- Headquarters – Phoenix, Arizona

- Distribution Center – Tucson, Arizona

- Retail Store – Flagstaff, Arizona

Future expansion has been approved for:

- Tubac Adventure Lodge

- Greer Mountain Cabins

The network has been designed to support future growth without requiring major redesigns.

## Enterprise Design Principles

The Network Engineering Department follows these design principles for all enterprise networks.

### Standardization

Whenever practical, use consistent hardware models, naming conventions, and configuration standards throughout the enterprise.

### Scalability

Design networks that can support future departments, additional users, and new business locations.

### Security

Protect enterprise resources using the principle of least privilege while maintaining business functionality.

### Documentation

Maintain accurate network documentation throughout the planning, implementation, verification, and maintenance phases of every Enterprise Change Request (ECR). Enterprise documentation should evolve as the network evolves and accurately reflect the current production environment.

### Reliability

Build networks that are dependable, resilient, and easy to support to ensure reliable business operations.

### Verification

All implementations must be verified before they are considered complete.

## Enterprise Addressing Standards

Sonoran Outdoor Equipment has been assigned a private enterprise IPv4 address space for all internal network operations.

The enterprise has also been assigned an IPv6 Global Unicast prefix and is designed as a dual-stack network. IPv6 services will be implemented through future Enterprise Change Requests.

Unless otherwise specified:

- All IPv4 and IPv6 network assignments must follow the Enterprise IP Addressing Plan.

- All router interfaces, switch management interfaces, WAN interfaces, loopback interfaces, and end-device addressing must follow the Enterprise IP Addressing Plan.

- Address assignments shall support future enterprise growth and comply with the enterprise addressing standards.

### Enterprise Dynamic Address Assignment Standards

The Sonoran Outdoor Equipment enterprise uses centralized Dynamic Host Configuration Protocol (DHCP) services to automatically assign IPv4 and IPv6 addressing information to client devices throughout the enterprise network.

Unless otherwise specified:

- Enterprise DHCP services shall be centrally hosted at Phoenix Headquarters.

- Remote locations shall obtain DHCP services using DHCP relay.

- DHCP pool names shall identify the site and the department or service they support.

| Location | DHCP Pool Name |
| --- | --- |
| Phoenix Executive | HQ-EXECUTIVE |
| Phoenix Human Resources | HQ-HR |
| Phoenix Accounting & Finance | HQ-ACCOUNTING |
| Phoenix Sales & Customer Service | HQ-SALES |
| Phoenix IT | HQ-IT |
| Tucson Warehouse | TUC-WAREHOUSE |
| Flagstaff Sales | FLG-SALES |

- DHCP pool names should remain consistent even if VLAN assignments change in the future.

- The first 10 usable IPv4 addresses of every subnet shall be reserved for network infrastructure and other statically addressed devices.

- The last 5 usable IPv4 addresses of every subnet shall be reserved for future enterprise growth and infrastructure expansion.

- All remaining usable IPv4 addresses shall be available for dynamic assignment through DHCP.

- IPv6 client addressing shall use SLAAC with Stateless DHCPv6, unless otherwise specified by future enterprise requirements.

DHCPv4 and DHCPv6 pools serving the same department or service must use the same enterprise pool name. The configuration context identifies the protocol; therefore, separate naming conventions for IPv4 and IPv6 are not required.

## Enterprise Standard Hardware

The enterprise has standardized on Cisco networking equipment.

### Standard Router

## Cisco 2911 Integrated Services Router

### Standard Switch Models

- Cisco Catalyst 2960

- Cisco Catalyst 3560

- Cisco Catalyst 3650

Recommendation:

Select one switch model and use it consistently throughout your enterprise whenever practical. Consistent hardware simplifies interface naming, documentation, troubleshooting, and configuration.

Note:

Layer 3 switches may be used when appropriate; however, Enterprise Change Requests are designed so they can be completed using traditional routed interfaces unless otherwise specified.

## Device Naming Standards

All enterprise network devices must follow the standardized naming convention shown below. The device name displayed in the network topology and the configured hostname on the Cisco device must match exactly. Consistent naming improves documentation, troubleshooting, and enterprise-wide standardization.

| Device | Required Device Name | Required Hostname |
| --- | --- | --- |
| Phoenix Headquarters Router 1 | HQ-R1 | HQ-R1 |
| Phoenix Headquarters Router 2 (Reserved for Phoenix Headquarters future expansion) | HQ-R2 | HQ-R2 |
| Phoenix Headquarters Switch 1 | HQ-S1 | HQ-S1 |
| Phoenix Headquarters Switch 2 | HQ-S2 | HQ-S2 |
| Tucson Distribution Router | TUC-R2 | TUC-R2 |
| Tucson Distribution Switch | TUC-S3 | TUC-S3 |
| Flagstaff Retail Router | FLG-R3 | FLG-R3 |
| Flagstaff Retail Switch | FLG-S4 | FLG-S4 |

Switch numbering is unique across the enterprise rather than restarting at each location.

Future locations should follow the same naming convention.

### Enterprise End Device Naming Standards

End devices must be named using the following format:

<Device Type>-<Department><Number>

When the same department exists at multiple company locations, prepend the site abbreviation to ensure each device name is unique.

#### Standard Device Prefixes

| Device Type | Prefix |
| --- | --- |
| Personal Computer | PC |
| Laptop | LT |
| Printer | PR |
| IP Phone | PH |
| Tablet | TB |
| Security Camera | CAM |
| IoT Device | IOT |
| Wireless Access Point | AP |
| Server | SRV |

#### Department Abbreviations

| Department | Abbreviation |
| --- | --- |
| Executive | EX |
| Human Resources | HR |
| Accounting & Finance | AF |
| Sales & Customer Service | SCS |
| Information Technology | IT |
| Warehouse | WH |
| Voice Services | VO |
| Guest Wireless | GW |
| Management | MG |

#### Site Abbreviations

| Department | Abbreviation |
| --- | --- |
| Phoenix Headquarters | HQ |
| Tucson Distribution Center | TUC |
| Flagstaff Retail Store | FLG |

#### Examples

Device names must be unique throughout the enterprise.

| Device | Name |
| --- | --- |
| Executive PC | PC-EX1 |
| Human Resources PC | PC-HR1 |
| Accounting & Finance Printer | PR-AF1 |
| IT PC | PC-IT1 |
| IT Laptop | LT-IT1 |
| Warehouse PC | PC-WH1 |
| Sales PC (Phoenix) | HQ-PC-SCS1 |
| Sales PC (Flagstaff) | FLG-PC-SCS1 |
| Sales Printer (Flagstaff) | FLG-PR-SCS1 |
| Security Camera | CAM-WH1 |
| Warehouse IoT Sensor | IOT-WH1 |

#### Required Enterprise Standard

The naming conventions in this guide are required enterprise standards and must be followed exactly. Consistent device names, hostnames, VLAN names, interface descriptions, and other standardized identifiers ensure accurate documentation, simplify troubleshooting, and support consistent enterprise network management.

Assignments and Enterprise Change Requests (ECRs) will be evaluated based on compliance with these standards. Failure to follow the required naming conventions may result in point deductions, even if the network functions correctly.

## Banner Standards

Every router and switch must include a Message of the Day (MOTD) banner. The MOTD banner provides identification and a legal warning for unauthorized users.

The banner must include:

- Junior Network Engineer (Student first and last name)

- Director of Network Engineering (Course Instructor)

- Course identifier (CNT160 ENSA)

- Enterprise warning message (See Example)

### Enterprise Warning Message

***********************************************
Sonoran Outdoor Equipment

Device: HQ-R1

Junior Network Engineer: Bob Samson

Director of Network Engineering: Andrea Schauer

CNT160 ENSA

WARNING: Authorized personnel only. Unauthorized access is prohibited. 
***********************************************

## Interface Description Standards

Interface descriptions are required on all configured and active interfaces unless otherwise directed by an Enterprise Change Request (ECR).

Interface descriptions shall accurately document, as applicable:

- Source device and interface

- Destination device and interface

- Connection purpose

- VLAN(s)

- IPv4 and IPv6 network(s)

- Department or service

All interface descriptions must comply with the documentation standards defined in the Lab Process & Expectations Guide.

## Enterprise Documentation Requirements

Enterprise documentation shall be updated when an Enterprise Change Request changes information represented in that documentation. Not every ECR requires changes to every enterprise document. The Enterprise Network Standards Guide should be updated only when an approved change establishes, modifies, or retires an enterprise standard.

Enterprise documentation should be updated throughout every Enterprise Change Request (ECR), as appropriate, and may include:

- Updated topology

- Addressing information

- Device configurations

- Verification results

- Configuration backups

- Enterprise Documentation Repository (EDR)

## Configuration Standards

Unless otherwise directed:

- Configure device hostnames according to the Enterprise Device Naming Standards.

- Configure and maintain interface descriptions that comply with the Lab Process & Expectations Guide.

- Highlight required configuration items in submitted show running-config outputs when directed by the Enterprise Change Request.

- Configure the approved enterprise passwords.

- Configure the required MOTD banner.

- Disable DNS lookups

- Verify all configurations before implementation is considered complete.

- Save the running configuration to the startup configuration.

- Update the Enterprise Documentation Repository (EDR).

Important:  Enterprise Password Standard

To ensure instructors can access and verify device configurations, all devices must use the standardized passwords defined by the course. Students may not substitute personal passwords.

Do not substitute personal passwords. All Enterprise Change Requests assume these standard passwords unless otherwise directed.

| Configuration | Standard Password |
| --- | --- |
| Console Password | cisco |
| VTY Password | cisco |
| Enable Secret | class |

### Enterprise Security Configuration Standards

Unless otherwise directed by an Enterprise Change Request (ECR):

- Use SSH Version 2 for all remote device management.

- Telnet shall not be used for remote device management.

- Administrative access shall require local or centralized authentication.

- Configure Port Security on designated enterprise access ports.

- Configure Sticky MAC addressing when Port Security is implemented.

- Configure a Port Security violation action according to the approved Enterprise Change Request.

- Configure PortFast and BPDU Guard on enterprise access ports.

- Configure Root Guard where designated by the Enterprise Network Engineering Department.

- Assign unused access ports to the designated Parking Lot VLAN and administratively shut them down.

## Enterprise Access Control Standards

Sonoran Outdoor Equipment uses Access Control Lists (ACLs) to enforce enterprise security policies, protect confidential business resources, secure network infrastructure, and control access to network services. All ACL implementations must follow the standards defined in this guide to ensure consistent deployment, simplify troubleshooting, and support future enterprise growth.

### Reserved Enterprise ACL Numbering Standards

The following numbered Access Control Lists are reserved for enterprise-wide use.

| ACL Number | Type | Purpose |
| --- | --- | --- |
| 10 | Standard IPv4 | Enterprise SSH Device Management |
| 110 | Extended IPv4 | External HTTPS Access Policy |
| 120 | Extended IPv4 | Reserved for Future Enterprise Security Policies |
| 130 | Extended IPv4 | Reserved for Future Enterprise Security Policies |
| 140 | Extended IPv4 | Reserved for Future Enterprise Expansion |

Additional numbered ACLs should follow the enterprise numbering convention and be documented before deployment.

### Enterprise ACL Naming Standards

Named Access Control Lists improve readability and simplify long-term administration. The following ACL names are reserved for enterprise-wide use.

IPv6 ACLs must use descriptive names and provide extended filtering capabilities based on source and destination IPv6 addresses, protocols, and transport-layer information when applicable. IPv6 ACLs do not use the numbered standard and extended ACL ranges used for IPv4 ACLs.

| ACL Name | Type | Purpose |
| --- | --- | --- |
| ACCOUNTING-ACL | Extended IPv4 | Accounting & Finance Protection |
| ACCOUNTING-IPV6-ACL | IPv6 | Accounting & Finance Protection |
| SNMP-ACL | Standard IPv4 | Network Management |
| SSH-IPV6-ACL | IPv6 | Enterprise SSH Device Management |
| WEB-IPV6-ACL | IPV6 | External HTTPS Access Policy |

ACL names should be descriptive, use uppercase letters with hyphens where appropriate, and remain consistent throughout the enterprise.

### Enterprise ACL Placement Standards

Access Control Lists should be placed to efficiently enforce enterprise security policies while minimizing unnecessary network traffic.

- Standard IPv4 ACLs should be placed as close to the destination as practical.

- Extended IPv4 ACLs should be placed as close to the source as practical.

- IPv6 ACLs should follow the same placement principles as IPv4 ACLs whenever applicable.

- Access Control Lists must be applied to the appropriate interface and direction based on the required security policy.

- Remote management restrictions must be applied to the VTY lines of all enterprise routers and switches using the appropriate IPv4 and IPv6 access-class commands.

### Enterprise Access Control Policies

The following enterprise security policies govern network access throughout the Sonoran Outdoor Equipment enterprise.

#### ACL Documentation Requirements

- Every enterprise Access Control List must include a meaningful remark statement documenting the purpose of the ACL.

- ACL remarks must clearly identify the business or security purpose of the ACL.

- Remarks must be concise, professionally written, and provide enough information for another network administrator to understand why the ACL exists.

- ACL remarks must be maintained and updated whenever the purpose or security policy associated with the ACL changes.

#### Enterprise Device Management

- Only the Information Technology department is authorized to establish SSH management sessions to enterprise routers and switches.

- Telnet is prohibited throughout the enterprise.

- Remote management access must be secured using both IPv4 and IPv6 access control policies.

#### Accounting & Finance Protection

- Only Executive Management is authorized to access Accounting & Finance resources.

- All other enterprise departments are denied access to Accounting & Finance resources unless specifically authorized by the Network Engineering Department.

#### External HTTPS Access

- Executive Management, Human Resources, Information Technology, and Sales & Customer Service are authorized to access approved external HTTPS services.

- Warehouse & Inventory workstations are prohibited from accessing approved external HTTPS services unless specifically authorized by the Network Engineering Department.

#### Dual-Stack Security

- Whenever enterprise services support both IPv4 and IPv6, equivalent security policies shall be implemented for both protocol versions.

- Security policies should provide equivalent protection regardless of the IP protocol version being used.

## Enterprise VLAN Standards

| VLAN ID | VLAN Name | Department / Purpose |
| --- | --- | --- |
| 10 | Executive | Executive Management |
| 20 | Human_Resources | Human Resources |
| 30 | Accounting_Finance | Accounting & Finance |
| 40 | Sales_Customer_Service | Sales & Customer Service |
| 50 | IT | Information Technology |
| 60 | Warehouse | Warehouse & Inventory |
| 70 | IoT_Security | IoT & Security Systems |
| 80 | Voice | Voice Services |
| 90 | Guest_Wireless | Guest Wireless Network |
| 99 | Management | Network Management |
| 999 | Parking_Lot | Unused Ports |

### Enterprise VLAN Standards – Implementation Requirements

VLAN 999 (Parking_Lot) is the enterprise Parking Lot VLAN for unused access ports. All unused access ports must be assigned to VLAN 999, appropriately described, and administratively shut down. VLAN 999 must not be assigned to active end devices and must not be permitted across trunk links.

VLAN implementation requirements vary by enterprise location. VLANs identified for initial implementation, reserved future use, or future expansion must be configured according to the standards defined below and the requirements of the applicable Enterprise Change Request (ECR).

### Initial Enterprise Deployment

#### Phoenix Headquarters

Implement:

- VLAN 10 – Executive

- VLAN 20 – Human Resources

- VLAN 30 – Accounting & Finance

- VLAN 40 – Sales & Customer Service

- VLAN 50 – Information Technology

- VLAN 99 – Network Management

- VLAN 999 – Parking_Lot/Unused Ports

Reserved for Future Activation:

- VLAN 70 – IoT & Security Systems

- VLAN 80 – Voice Services

- VLAN 90 – Guest Wireless Network

All enterprise VLANs are created and named on the Phoenix Headquarters switches during the initial deployment.

Interfaces designated for a Reserved VLAN must be preconfigured as access ports and assigned to their designated VLAN. Reserved interfaces must include the required interface description and remain administratively shut down until activated by a future Enterprise Change Request (ECR).

Reserved VLANs must not be permitted across trunk links and should not receive Layer 3 gateway or inter-VLAN routing configuration until activated by a future ECR.

#### Tucson Distribution Center

Initial implementation:

- VLAN 1 – Warehouse & Inventory

- VLAN 999 – Parking_Lot/Unused Ports

Future expansion:

- VLAN 40 – Sales & Customer Service

- VLAN 50 – Information Technology

- VLAN 60 – Warehouse & Inventory

During the initial enterprise deployment, the Tucson Distribution Center operates as a single routed network using VLAN 1. VLANs identified for future expansion are documented for network planning purposes but are not created, assigned to access ports, or configured for inter-VLAN routing until directed by a future Enterprise Change Request (ECR).

Interfaces designated for future expansion must include the required interface description and remain administratively shut down until activated by a future ECR.

#### Flagstaff Retail Store

Initial implementation:

- VLAN 1 – Sales & Customer Service

- VLAN 999 – Parking_Lot/Unused Ports

Future expansion:

- VLAN 40 – Sales & Customer Service

During the initial enterprise deployment, the Flagstaff Retail Store operates as a single routed network using VLAN 1. VLANs identified for future expansion are documented for network planning purposes but are not created, assigned to access ports, or configured for inter-VLAN routing until directed by a future Enterprise Change Request (ECR).

Interfaces designated for future expansion must include the required interface description and remain administratively shut down until activated by a future ECR.

Additional VLANs and networks may be added in future Enterprise Change Requests as business needs evolve.

## Router Interface Standards

The following tables are organized from the perspective of the device identified in each section heading. "Local Interface" refers to the interface on that device. "Connected Device" and "Remote Interface" identify the device and interface at the opposite end of the connection.

### HQ-R1: Phoenix Headquarters Router 1

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| S0/0/0 | TUC-R2 | S0/0/0 |
| S0/0/1 | FLG-R3 | S0/0/0 |
| G0/0 | HQ-S1 | G0/1 |
| G0/1 | ISP Router (Future) | G0/0 |
| G0/2 | Reserved | — |
| Lo0 | Router ID / Management | — |
| Lo10 | Reserved | — |

### HQ-R2: Phoenix Headquarters Router 2

HQ-R2 is reserved for future enterprise expansion and shall not be implemented until directed by an approved Enterprise Change Request (ECR).

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| G0/0 | Reserved – Future Enterprise Expansion | — |
| G0/1 | Reserved | — |
| G0/2 | Reserved | — |

### TUC-R2: Tucson Distribution Center Router 2

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| S0/0/0 | HQ-R1 | S0/0/0 |
| S0/0/1 | FLG-R3 | S0/0/1 |
| G0/0 | TUC-S3 | G0/1 |
| G0/1 | Reserved | — |
| G0/2 | Reserved | — |
| Lo0 | Router ID / Management | — |
| Lo10 | Future Tubac Branch | — |

### FLG-R3: Flagstaff Retail Store Router 3

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| S0/0/0 | HQ-R1 | S0/0/1 |
| S0/0/1 | TUC-R2 | S0/0/1 |
| G0/0 | FLG-S4 | G0/1 |
| G0/1 | Reserved | — |
| G0/2 | Reserved | — |
| Lo0 | Router ID / Management | — |
| Lo10 | Future Greer Branch | — |

## Switch Interface Standards

### Switch Port Configuration Standards

All enterprise access switches must be configured according to the following standards:

- Configure all active switch ports as static access or trunk ports, as appropriate.

- Assign all unused switch ports to VLAN 999 (Unused/Parking VLAN).

- Administratively shut down all unused switch ports.

- Administratively shut down all reserved switch ports until they are activated for their designated purpose by an approved Enterprise Change Request (ECR).

- Activate reserved ports only when directed by an approved Enterprise Change Request (ECR).

### Switch Interface Description Standards

#### Configure interface descriptions according to the following standards:

- Configured switchports – Configure interface descriptions on all active switch ports according to the documentation standards defined in the Enterprise Change Request Process & Expectations Guide.

- Reserved and future expansion switchports – Configure an interface description identifying the port's planned enterprise purpose according to the Standard Switch Port Assignments. Reserved interfaces at Phoenix Headquarters and interfaces designated for future expansion at remote sites must be configured according to the applicable Enterprise Change Request (ECR).

- Unused switch ports – Configure an interface description indicating the port is Unused - shutdown for security. Assign unused ports to VLAN 999 (Unused/Parking VLAN) and administratively shut them down.

### Important: Implemented, Reserved and Future Expansion Interfaces

The Standard Switch Port Assignments identify interfaces used during the current enterprise deployment as well as interfaces designated for future network growth

- Implemented interfaces identify devices and connections that should be installed and configured as part of the current enterprise deployment.

- Reserved interfaces at Phoenix Headquarters identify ports designated for planned enterprise services or expansion. These interfaces are preconfigured according to the applicable VLAN standards but remain administratively shut down until activated by a future Enterprise Change Request (ECR).

- Future expansion interfaces at remote sites identify ports designated for future network growth. Unless specifically directed by an ECR, these interfaces should be documented and administratively shut down but should not be assigned to future VLANs until those VLANs are implemented.

As the enterprise grows throughout the semester, future Enterprise Change Requests will activate additional interfaces, devices, departments, VLANs, and services.

### Standard Switch Port Assignments

To promote consistency throughout the enterprise, switch interfaces have been assigned standardized functions. Unless otherwise specified in an Enterprise Change Request, all switch ports should follow these assignments. Reserving interface ranges for specific purposes simplifies documentation, troubleshooting, grading, and future network expansion.

#### End Device Placement Standards

Enterprise end devices shall be installed in the department or business area they are intended to support. End devices should not remain in the Network Closet after installation unless they are specifically designated as network infrastructure.

Unless otherwise specified by an Enterprise Change Request (ECR):

- Install workstations, laptops, printers, IP phones, wireless access points, and other end-user devices in their assigned office or departmental locations.

- Install servers in the designated server or infrastructure area.

- Only network infrastructure devices, such as routers, switches, and approved management or console workstations, should remain in the Network Closet (MDF).

- Arrange end devices in a logical and professional manner that reflects the physical layout of the enterprise.

Proper device placement is considered part of the enterprise implementation and documentation standards and will be evaluated as part of each Enterprise Change Request (ECR).

#### HQ-S1: Phoenix Enterprise Switch 1

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| G0/1 | HQ-R1 | G0/0 |
| F0/1-F0/3 | HQ-S2 | F0/1-F0/3 |
| F0/4-F0/9 | Unused | — |
| F0/10 | Reserved - Executive Management | — |
| F0/11 | Human Resources End User | NIC |
| F0/12 | Reserved - Accounting & Finance | — |
| F0/13 | Sales & Customer Service End User | NIC |
| F0/14 | Information Technology End User | NIC |
| F0/15 | Reserved - Voice Endpoint | — |
| F0/16 | Reserved - IoT/Security Device | — |
| F0/17 | Reserved - Guest Wireless Access Point | — |
| F0/18-F0/19 | Unused | — |
| F0/20–F0/24 | Reserved - Printers, Servers, Infrastructure Devices (Static IP) | Assigned as needed |

#### HQ-S2: Phoenix Headquarters Switch 2

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| G0/1 | Reserved - Future Enterprise Expansion | — |
| F0/1-F0/3 | HQ-S1 | F0/1-F0/3 |
| F0/4-F0/9 | Unused | — |
| F0/10 | Executive Management End User | NIC |
| F0/11 | Human Resources End User | NIC |
| F0/12 | Accounting & Finance End User | NIC |
| F0/13 | Sales & Customer Service End User | NIC |
| F0/14 | Information Technology End User | NIC |
| F0/15 | Reserved - Voice Endpoint | — |
| F0/16 | Reserved - IoT/Security Device | — |
| F0/17 | Reserved - Guest Wireless Access Point | — |
| F0/18-F0/19 | Unused | — |
| F0/20–F0/24 | Reserved - Printers, Servers, Infrastructure Devices (Static IP) | Assigned as needed |

#### TUC-S3: Tucson Distribution Center Switch 3

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| G0/1 | TUC-R2 | G0/0 |
| F0/1-F0/3 | Reserved for EtherChannel | — |
| F0/4-F0/9 | Unused | — |
| F0/10–F0/12 | Warehouse Workstations | NIC |
| F0/13-F0/17 | Future Expansion | — |
| F0/18-F0/19 | Unused | — |
| F0/20–F0/24 | Future Expansion - Printers, Servers, Infrastructure Devices (Static IP) | Assigned as needed |

#### FLG-S4: Flagstaff Retail Store Switch 4

| Local Interface | Connected Device | Remote Interface |
| --- | --- | --- |
| G0/1 | FLG-R3 | G0/0 |
| F0/1-F0/3 | Reserved for EtherChannel | — |
| F0/4-F0/9 | Unused | — |
| F0/10–F0/12 | Sales & Customer Service End User | NIC |
| F0/13-F0/17 | Future Expansion | — |
| F0/18-F0/19 | Unused | — |
| F0/20–F0/24 | Future Expansion - Printers, Servers, Infrastructure Devices (Static IP) | Assigned as needed |

## Enterprise Packet Tracer Standards & Topology

### Enterprise Packet Tracer File Standard

All Enterprise Change Requests (ECRs) are completed using a single cumulative Cisco Packet Tracer Activity (.pka) file.

Throughout the semester, you will continue building the same enterprise network. Each Enterprise Change Request builds upon the previous implementation, resulting in a complete enterprise network by the end of the course.

Unless otherwise instructed, submit both your cumulative .pka file and the completed ECR documentation required for the assignment.

### Physical Workspace Standard

The Physical Workspace is the official implementation environment for this course.

All devices should be placed, cabled, named, and configured in the Physical Workspace according to the Enterprise Network Standards Guide and Enterprise IP Addressing Plan.

The Physical Workspace is the version that will be evaluated during grading.

### Logical Workspace

The Logical Workspace automatically represents the same enterprise network.

You may use the Logical Workspace to:

- Visualize the enterprise topology

- Review device connectivity

- Troubleshoot network issues

- Better understand logical network relationships

Because both workspaces represent the same network, no separate Packet Tracer file is required.

### Enterprise Topology

The enterprise network consists of three company locations connected by a wide area network (WAN):

- Phoenix Headquarters (HQ)

- Tucson Distribution Center (TUC)

- Flagstaff Retail Store (FLG)

The Internet connection is located at Phoenix Headquarters.

Build and maintain the enterprise topology as shown in the course documentation unless an Enterprise Change Request specifically instructs you to make changes.

### In-Person Cisco Networking Environment

Students enrolled in an in-person section will build and maintain the enterprise network in both Cisco Packet Tracer and the physical Cisco networking environment.

The cumulative Packet Tracer Activity (.pka) serves as the primary enterprise network model and should always reflect the current state of your enterprise.

Designated Enterprise Change Requests (ECRs) will also require you to implement the same configuration changes on the physical Cisco routers and switches in the classroom.

Unless otherwise instructed:

- Complete the Enterprise Change Request in your cumulative Packet Tracer Activity.

- Implement the required configuration on the assigned Cisco equipment.

- Verify connectivity in both environments when required.

- Keep your Packet Tracer enterprise synchronized with your physical Cisco network throughout the semester.

### Enterprise File Naming

Save your cumulative Packet Tracer Activity using the naming convention specified by the assignment.

Examples:

- ECR-001 – Your Name.pka

- ECR-002 – Your Name.pka

- ECR-003 – Your Name.pka

Continue updating the same cumulative enterprise file throughout the semester.

### Save and Backup Requirements

Save your Packet Tracer Activity frequently while working.

Before beginning each new Enterprise Change Request, create a backup copy of your cumulative .pka file. Maintaining backup copies allows you to recover your work if a file becomes damaged or an error occurs during implementation.

## Enterprise Standards Addendum — First-Hop Redundancy

Jr Network Operations Engineer-Developed Enterprise Standard

As part of an approved Enterprise Change Request, you may be directed to establish a new enterprise standard for first-hop gateway redundancy. Use this addendum to document the standard developed from your completed implementation and verification.

Write the standard as an enterprise-wide policy that can be applied to future network deployments, rather than as a description of a specific router, VLAN, or site.

Your standard should address, as applicable:

- Approved first-hop redundancy protocol

- Virtual default gateway addressing convention

- Relationship between the virtual gateway address and physical router interface addresses

- Group numbering convention

- Active and standby router roles

- Priority and preemption requirements

- IPv4 and IPv6 redundancy requirements

- Required verification procedures

Enterprise First-Hop Redundancy Standard:

Enter the approved standard below.

## Version 1: Updated 08/22/2026