ECR-005

## Secure Enterprise Network Access

| Requested By | Network Engineering Department |
| --- | --- |
| Assigned To | Junior Network Engineer |
| Priority | High |
| Change Type | Normal |
| Implementation Window | Estimated Implementation Time: 1–3 hours See established due date. |
| Business Impact | Medium – Security enhancements may temporarily affect network access or device management if implemented incorrectly, requiring careful validation after deployment. |

## Business Justification

Following the successful deployment of the enterprise routing infrastructure, the Network Engineering Department has initiated the next phase of the enterprise network modernization project by standardizing security across the access layer. As the enterprise continues to expand, consistent security configurations are required to protect network devices, reduce the risk of unauthorized access, improve network resiliency, and establish standardized configuration practices across all enterprise switching infrastructure. This change implements the Enterprise Security Configuration Standards, strengthens remote device management, enhances Layer 2 resiliency, and improves the overall security posture of the Sonoran Outdoor Equipment network.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

- Implement an enterprise EtherChannel using Cisco Port Aggregation Protocol (PAgP) to increase bandwidth and provide link redundancy between headquarters switches.

- Configure enterprise Spanning Tree Protocol (STP) enhancements, including Root Bridge priority, PortFast, BPDU Guard, and Root Guard, to improve Layer 2 resiliency and protect the switching infrastructure.

- Secure enterprise access ports by implementing Port Security, Sticky MAC addressing, Parking Lot VLANs, and administrative shutdown of unused switch ports.

- Configure secure remote device management using SSH Version 2, local authentication, RSA encryption keys, and secure VTY access on enterprise routers and switches.

- Verify enterprise network connectivity using IPv4 and IPv6 connectivity tests following implementation of enterprise security enhancements.

- Validate enterprise configurations using Cisco IOS show commands and interpret the operational status of EtherChannel, STP, Port Security, and SSH.

- Verify compliance with the Enterprise Network Standards Guide through analysis of device configurations and operational output.

- Document and submit an enterprise validation package demonstrating successful implementation of the requested infrastructure security improvements.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Successfully complete ECR-001 through ECR-004.

✔ The enterprise network is fully operational using both IPv4 and IPv6 addressing.

✔ OSPFv2 and OSPFv3 are fully implemented and enterprise connectivity has been verified.

✔ The Enterprise Network Standards Guide and Security Configuration Standards are available for reference.

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

- Enterprise Network Standards Guide

- Enterprise IP Addressing Plan

- Enterprise Topology

- Enterprise Change Request (ECR-005)

- Enterprise Change Request Process & Expectations Guide

## Implementation Plan

### Phase 1 – Improve Enterprise Switch Resiliency

Background

As enterprise network traffic continues to grow, the Network Engineering Department has approved the implementation of link aggregation between the headquarters access switches. EtherChannel combines multiple physical links into a single logical connection, increasing available bandwidth while providing redundancy in the event of a link failure. Sonoran Outdoor Equipment has standardized on Cisco's Port Aggregation Protocol (PAgP) for this implementation.

Prepare the Reserved EtherChannel Interfaces

On HQ-S1 and HQ-S2, enable the previously reserved FastEthernet0/2 and FastEthernet0/3 interfaces. Verify that the interfaces are operational before configuring the EtherChannel.

Give the links a moment to come up and negotiate, and then configure F0/1–F0/3 together as Port-Channel 1.

Instructions

☐  Configure Port-Channel 1 between HQ-S1 and HQ-S2 using PAgP according to the Enterprise Network Standards Guide.

At a minimum:

- Configure Port-Channel 1 between HQ-S1 and HQ-S2.

- Configure HQ-S1 to use PAgP desirable mode.

- Configure HQ-S2 to use PAgP auto mode.

- Configure Port-Channel 1 as an 802.1Q trunk and configure VLAN 99 as the native VLAN.

- Update the descriptions on both the physical interfaces and Port-Channel 1 according to the Enterprise Network Standards Guide and the Enterprise Change Request Process & Expectations Guide.

- Verify that Port-Channel 1 is operational and that all member interfaces have successfully joined the EtherChannel.

- Troubleshoot and correct any EtherChannel configuration issues before proceeding.

### Phase 2 – Implement Enterprise Spanning Tree Standards

#### Step 1: Configure Enterprise Spanning Tree Security

Background

Following the implementation of EtherChannel, the Network Engineering Department has approved the next phase of the enterprise access layer modernization project by standardizing the Spanning Tree Protocol (STP) configuration across the enterprise switching infrastructure. Proper STP configuration improves network resiliency, prevents Layer 2 switching loops, protects the enterprise root bridge, and allows end devices to connect more efficiently while reducing the risk of unauthorized or accidental topology changes. Root Guard is used to ensure that designated switches cannot unexpectedly become the Root Bridge, helping maintain a stable and predictable spanning-tree topology.

☐  Configure the enterprise Spanning Tree Protocol (STP) settings according to the Enterprise Network Standards Guide.

At a minimum:

Phoenix Headquarters – HQ-S1 and HQ-S2

- Configure PortFast globally so that enterprise end-device access ports immediately transition to the forwarding state.

- Configure BPDU Guard globally to protect PortFast-enabled access ports.

- Configure HQ-S2 as the Root Bridge for the implemented enterprise VLANs.

- Configure Root Guard on the HQ-S2 Port-Channel interface to protect the intended STP topology.

- Ensure PortFast and BPDU Guard do not affect trunk or infrastructure links.

Important:

Root Guard has not been previously implemented within the enterprise. The Network Engineering Director has approved its use to ensure that HQ-S2 remains the enterprise Root Bridge. Configure Root Guard on the interface connecting HQ-S2 to HQ-S1 using the following command:

HQ-S2(config)# int po1

HQ-S2(config-if)# spanning-tree guard root

Remote Sites – TUC-S3 and FLG-S4

- Configure PortFast globally so that end-device access ports immediately transition to the forwarding state.

- Configure BPDU Guard globally to protect PortFast-enabled access ports.

- Because the G0/1 router uplinks operate as VLAN 1 access links, explicitly disable PortFast and BPDU Guard on TUC-S3 G0/1 and FLG-S4 G0/1 after applying the global settings.

interface g0/1

spanning-tree portfast disable

spanning-tree bpduguard disable

- Verify that PortFast and BPDU Guard remain active on the designated end-device access ports.

#### Verification

- Verify the STP configuration and confirm that HQ-S2 is operating as the Root Bridge for the required VLANs.

- Verify that Root Guard is correctly configured on the HQ-S2 Port-Channel interface.

- Verify that PortFast and BPDU Guard are operating as intended on enterprise end-device access ports and are not applied to infrastructure links.

- Troubleshoot and correct any STP configuration issues before proceeding.

### Phase 3 – Secure Enterprise Access Ports

#### Step 1: Configure Enterprise Port Security

Background

As part of the enterprise access layer hardening initiative, the Network Engineering Department has approved the implementation of standardized port security across the enterprise switching infrastructure. Port Security helps prevent unauthorized devices from connecting to the enterprise network while allowing approved end devices to maintain normal network access.

During the initial enterprise deployment in ECR-001, unused and reserved switch ports were secured according to the Enterprise Network Standards Guide. Unused ports were assigned to the enterprise Parking Lot VLAN (VLAN 999) and administratively shut down, while reserved ports were administratively shut down until needed for an approved future implementation. This phase builds upon those existing security measures by implementing Port Security on active enterprise access ports.

☐  Configure enterprise access ports according to the Security Configuration Standards found in the Enterprise Network Standards Guide.

At a minimum:

- On TUC-S3 and FLG-S4, configure the designated end-device interfaces as static access ports before enabling Port Security. The designated Phoenix Headquarters end-device interfaces were configured as static access ports during the initial enterprise deployment and do not require this additional step.

- Configure Port Security on all designated enterprise access ports.

- Configure each access port to allow a maximum of two secure MAC addresses.

- Configure Sticky MAC Addressing so that dynamically learned MAC addresses are added to the running configuration.

- Configure the Port Security violation mode to shutdown.

- Verify that Port Security is operating correctly on all designated access ports.

- Troubleshoot and correct any Port Security or access port configuration issues before proceeding.

### Phase 4 – Secure Enterprise Device Management

#### Step 1: Configure Secure Remote Device Management

Background

To improve the security of the enterprise network infrastructure, the Network Engineering Department has approved the transition from unsecured remote management to Secure Shell (SSH). SSH provides encrypted remote administrative access to enterprise network devices, helping protect administrator credentials and management traffic from unauthorized interception. As part of this implementation, all enterprise routers and switches will use standardized local authentication, encrypted communications, and secure remote management settings in accordance with the Enterprise Security Configuration Standards.

Important:

Host key verification has not been previously implemented within the enterprise. During the initial SSH connection, administrators should verify the authenticity of the remote device before accepting its SSH host key. This process helps protect against connecting to an unauthorized or spoofed network device. The Network Engineering Director has provided the required configuration and verification procedures below.

☐  Configure secure remote device management according to the Enterprise Security Configuration Standards on all enterprise intermediary devices.

At a minimum:

- Configure the local administrator account using the following enterprise standards:

  - Username: Andrea

  - Privilege Level: 15

  - Secret password: CNT160class

- Configure the enterprise domain name:

  - SOE.com

- Generate 1024-bit RSA encryption keys.

- Configure all enterprise routers and switches to use SSH Version 2.

- Configure the VTY lines to:

  - Use local authentication (login local)

  - Accept SSH connections only

  - Disable Telnet access.

- Verify that secure remote management is functioning correctly by establishing an SSH connection to each enterprise router and switch.

- During the initial SSH connection, verify and accept the remote device's SSH host key when prompted.

- Troubleshoot and correct any SSH connectivity or authentication issues before proceeding.

- Save the running configuration after verifying SSH connectivity.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete command output or ping results.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Verify Enterprise Connectivity

☐ Verify end-to-end enterprise communication.

For each connectivity test:

- Perform and document both an IPv4 and an IPv6 ping

- Include the source device in the screenshot.

- Include the successful ping results.

- Ensure the destination IPv4 and IPv6 address is visible in the screenshot.

⚠ Important:

The Sonoran Outdoor Equipment enterprise operates as a dual-stack IPv4/IPv6 network. Unless otherwise specified, all required connectivity validation must include successful IPv4 and IPv6 connectivity tests. Every screenshot must include the source device, the successful ping results, and the destination IP address so the Director of Network Engineering can verify that enterprise communications remain fully operational.

Phoenix Headquarters

Verify that enterprise communications between departments connected to HQ-S1 and HQ-S2 remain operational following the implementation of the enterprise access layer security enhancements. These tests validate that the EtherChannel and trunk configuration are functioning correctly and that normal business communications have not been disrupted.

At a minimum, perform and document the following connectivity tests:

Connectivity Test

| Source Department HQ-S1 | Destination Department HQ-S2 | Screenshot |
| --- | --- | --- |
| IPv4 Phoenix PC-HR1 | IPv4 Phoenix PC-AF1 |  |
| IPv6 Phoenix PC-HR1 | IPv6 Phoenix PC-AF1 |  |
| IPv4 Phoenix PC-SCS1 | IPv4 Phoenix PC-EX1 |  |
| IPv6 Phoenix PC-SCS1 | IPv6 Phoenix PC-EX1 |  |
| IPv4 Phoenix PC-IT1 | IPv4 Phoenix PC-EX1 |  |
| IPv6 Phoenix PC-IT1 | IPv6 Phoenix PC-EX1 |  |

#### Step 2 – Verify Secure Device Management

☐ Verify that secure remote management has been successfully implemented across the enterprise network.

For each SSH validation:

- Establish a successful SSH connection to each enterprise router and switch using its IPv4 management address.

- Establish a successful SSH connection to each enterprise router and switch using its IPv6 management address.

- Include the source device initiating the SSH session.

- Include the successful SSH login.

- Ensure the destination device is clearly identified.

- During the initial SSH connection, verify and accept the SSH host key if prompted.

⚠ Important:

Your validation package must clearly document the source of each SSH connection. Every screenshot must include both the source device and the successful SSH session so the Director of Network Engineering can verify that secure remote management has been successfully implemented throughout the enterprise.

At a minimum, perform and document the required SSH connectivity tests for all enterprise routers and switches.

| Device | Screenshot |
| --- | --- |
| IPv4 HQ-R1 |  |
| IPv6 HQ-R1 |  |
| IPv4 HQ-S1 |  |
| IPv6 HQ-S1 |  |
| IPv4 HQ-S2 |  |
| IPv6 HQ-S2 |  |
| IPv4 TUC-R2 |  |
| IPv6 TUC-R2 |  |
| IPv4 TUC-S3 |  |
| IPv6 TUC-S3 |  |
| IPv4 FLG-R3 |  |
| IPv6 FLG-R3 |  |
| IPv4 FLG-S4 |  |
| IPv6 FLG-S4 |  |

#### Step 3 – Verify EtherChannel Configuration

☐ Verify the enterprise EtherChannel configuration on Phoenix Headquarters enterprise switches using the required show commands.

### Background

Enterprise network validation requires reviewing information from multiple show commands to confirm that the EtherChannel has been implemented correctly and is operating according to the Enterprise Network Standards Guide.

Important:

Before submitting your validation package, highlight the required configuration items identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with the Enterprise Change Request and Enterprise Network Standards.

### Verify EtherChannel Operation

Command:

show etherchannel summary

Device: HQ-S1 and HQ-S2

Highlight the following:

- Port-Channel 1

- PAgP Protocol

- Member Interfaces

- EtherChannel Status (SU)

Paste the complete command output in the table provided below.

| Device | Required Output - Paste show etherchannel summary |
| --- | --- |
| HQ-S1 |  |
| HQ-S2 |  |

### Verify Trunk Configuration

Command:

show interfaces trunk

Device: HQ-S1 and HQ-S2

Highlight the following:

- Port-Channel 1

- Trunk Status

- Native VLAN

- Allowed VLANs

Paste the complete command output in the table provided below.

| Device | Required Output - Paste show interfaces trunk |
| --- | --- |
| HQ-S1 |  |
| HQ-S2 |  |

#### Step 4 – Verify Enterprise Spanning Tree Configuration

☐ Verify the enterprise Spanning Tree Protocol (STP) implementation using the required show commands below.

### Background

Enterprise network validation requires reviewing information from multiple show commands to confirm that the Spanning Tree Protocol (STP) configuration has been implemented correctly and is operating according to the Enterprise Network Standards Guide.

Important:

Before submitting your validation package, highlight the required configuration items identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with the Enterprise Change Request and Enterprise Network Standards Guide.

### Verify Enterprise Spanning Tree Operation

Command:

show spanning-tree

Device: HQ-S1 and HQ-S2

Highlight the following:

- Root Bridge

- Root Port

- Designated Ports

- Port Roles

- Port States

Paste the complete command output in the table provided below.

| Device | Required Output - Paste show spanning-tree |
| --- | --- |
| HQ-S1 |  |
| HQ-S2 |  |

#### Step 5 – Verify Enterprise Port Security Configuration

☐ Verify the enterprise Port Security implementation using the required show commands below.

### Background

Enterprise network validation requires reviewing the Port Security configuration to confirm that enterprise access ports have been secured according to the Enterprise Security Configuration Standards. The commands below verify that Port Security is enabled and operating correctly on the designated enterprise access ports.

Important:

Before submitting your validation package, highlight the required configuration items identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with the Enterprise Change Request and Enterprise Network Standards Guide.

### Verify Enterprise Port Security

Command:

show port-security

Device: HQ-S1, HQ-S2, TUC-S3, FLG-S4

Highlight the following:

- Port Security Status

- Enabled Secure Ports

- Maximum Allowed Secure MAC Addresses

- Current Secure MAC Addresses

- Security Violation Count

Paste the complete command output in the table provided below.

| Device | Required Output - Paste show port-security |
| --- | --- |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-S3 |  |
| FLG-S4 |  |

#### Step 6 – Verify Enterprise Secure Device Management

☐ Verify the enterprise Secure Shell (SSH) implementation using the required show commands below.

### Background

Enterprise network validation requires reviewing information from multiple show commands to confirm that secure remote device management has been implemented according to the Enterprise Security Configuration Standards.

Important:

Before submitting your validation package, highlight the required configuration items identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with the Enterprise Change Request and Enterprise Network Standards Guide.

### Verify SSH Configuration

Command:

show ip ssh

Device: HQ-R1, TUC-R2, FLG-R3, HQ-S1, HQ-S2, TUC-S3, FLG-S4

Highlight the following:

- SSH Version

Paste the complete command output in the table provided below.

| Device | Required Output - Paste show ip ssh |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

### Verify Active SSH Sessions

Command:

show ssh

Device: HQ-R1, TUC-R2, FLG-R3, HQ-S1, HQ-S2, TUC-S3, FLG-S4

Highlight the following:

- Active SSH Sessions

- SSH Connection Status

Paste the complete command output in the table provided below.

| Device | Required Output - Paste show ssh |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

### Verify RSA Encryption Keys

Command:

show crypto key mypubkey rsa

Device: HQ-R1, TUC-R2, FLG-R3, HQ-S1, HQ-S2, TUC-S3, FLG-S4

Highlight the following:

- RSA Key

- Key Modulus Size (1024 bits)

Paste the complete command output in the table provided below.

| Device | Required Output - Paste show crypto key mypubkey rsa |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

#### Step 7 – Verify Final Device Configurations

☐ Verify the final configuration of each enterprise device.

For every router and switch, paste the complete output of:

show running-config

This configuration will be used to verify compliance with the Enterprise Network Standards Guide and Enterprise Security Configuration Standards and assist with troubleshooting if necessary.

Important:

Before submitting your show running-config output, highlight the configuration items implemented as part of this Enterprise Change Request that apply to each device. Highlighting these items helps the Director of Network Engineering quickly verify compliance with the Enterprise Change Request and Enterprise Network Standards Guide.

Highlight the configuration items implemented as part of this Enterprise Change Request that apply to each device.

- Channel-group configuration

- Port-Channel interface configuration

- Interface descriptions

- Root Bridge Priority

- PortFast configuration

- BPDU Guard configuration

- Root Guard configuration

- Maximum Secure MAC Addresses

- Sticky MAC configuration

- Port Security violation mode (shutdown)

- Parking Lot VLAN assignments

- Shutdown unused interfaces

- Local username

- Privilege level

- Domain name

- VTY configuration

- login local

- transport input ssh

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

#### Step 8 – Final Validation

☐ Confirm that the Enterprise Change Request has been completed successfully.

Verify that:

- All required configurations have been completed.

- All verification tests have passed.

- All device configurations have been saved.

- The enterprise network is operating according to the Enterprise Network Standards Guide.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 08/20/2026