ECR-008

## Enterprise Gateway Redundancy Deployment

| Field | Scope |
| --- | --- |
| Requested by | Network Engineering Department |
| Assigned to | Junior Network Engineer |
| Priority | High |
| Change type | Normal |
| Implementation Window | Friday 11:00pm to Saturday 4:00am Estimated Implementation Effort: 2–5 hours See established due date |
| Business Impact | High (Temporary interruption expected during implementation) |
| Rollback Plan | Restore HQ-R1 as the sole enterprise gateway if implementation cannot be completed successfully. |

## Business Justification

As Sonoran Outdoor Equipment has continued to expand, Phoenix Headquarters has become increasingly dependent on continuous network availability. Currently, all Phoenix departments rely on a single router (HQ-R1) as their default gateway for inter-VLAN routing. Although the existing design meets current operational requirements, it introduces a single point of failure. If HQ-R1 becomes unavailable due to planned maintenance, hardware failure, or an unexpected outage, users throughout Phoenix Headquarters would lose connectivity to enterprise resources and remote locations.

To improve network resiliency and business continuity, the Network Engineering Department has approved the deployment of a second headquarters router (HQ-R2). The additional router will provide gateway redundancy for all Phoenix VLANs while supporting future enterprise growth.

As part of this Enterprise Change Request (ECR), Hot Standby Router Protocol Version 2 (HSRPv2) will be implemented to provide first-hop gateway redundancy for IPv4 services at Phoenix Headquarters. HSRPv2 allows HQ-R1 and HQ-R2 to operate as redundant default gateways while presenting a virtual IPv4 gateway address to devices on each implemented headquarters VLAN. If the active gateway becomes unavailable, the standby router can assume the gateway role, helping maintain network connectivity and reducing the impact of a single router failure on business operations.

Following implementation, Phoenix Headquarters will use HSRPv2 virtual IPv4 default gateways for all implemented VLANs. The first usable IPv4 address in each VLAN will serve as the virtual gateway address. HQ-R1 will operate as the preferred active gateway through the use of a higher priority and preemption, while HQ-R2 will provide standby gateway redundancy. IPv6 first-hop gateway redundancy would also normally be implemented in the enterprise dual-stack environment; however, IPv6 HSRPv2 implementation will be deferred due to Cisco Packet Tracer limitations and will not be configured as part of this ECR.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

- Deploy a new enterprise router and integrate it into an existing production network.

- Configure and verify a new routed WAN connection between headquarters and a remote enterprise location.

- Extend the enterprise IPv4 and IPv6 addressing plan to support new network infrastructure.

- Update an existing OSPFv2 and OSPFv3 routing domain to support enterprise expansion.

- Configure and verify Router-on-a-Stick inter-VLAN routing on a redundant headquarters router.

- Modify IPv4 gateway addressing to support first-hop redundancy.

- Configure and verify Hot Standby Router Protocol Version 2 (HSRPv2) for IPv4 gateway redundancy.

- Verify enterprise connectivity before, during, and after a simulated gateway failure.

- Validate gateway redundancy using ping, traceroute, HSRPv2, and OSPF verification commands.

- Analyze routing path changes following a simulated gateway failure.

- Restore normal enterprise operations following completion of failover testing.

- Verify enterprise device configurations using Cisco IOS show commands.

- Apply enterprise change management procedures while maintaining network availability and resiliency.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Company Overview

✔ Enterprise Network Standards Guide

✔ Enterprise Network Addressing Plan

✔ Enterprise Topology (.pka)

✔ Interface Addressing Plan

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

- Enterprise Network Topology

- Enterprise Network Standards Guide

- Enterprise IP Addressing Plan

- One (1) additional Cisco 2911 Router (HQ-R2)

- Two (2) Ethernet network cables

- This Enterprise Change Request (ECR-008)

## Planned Change Window

To minimize disruption to business operations, this Enterprise Change Request has been approved for implementation during the enterprise's scheduled maintenance window. During this time, temporary interruptions to network connectivity may occur as gateway addresses are updated and first-hop redundancy protocols are deployed.

All implementation, validation, and documentation activities should be completed before the maintenance window closes and normal business operations resume.

## Implementation Plan

### Phase 1 – Deploy HQ-R2

### Background

To improve enterprise resiliency, the Network Engineering Department has approved the deployment of a second headquarters router (HQ-R2). In addition to providing redundant gateway services for Phoenix Headquarters, HQ-R2 will become an active participant in the enterprise routing infrastructure. A new routed WAN connection will be established between HQ-R2 and Tucson Distribution Center (TUC-R2), allowing HQ-R2 to exchange routing information with the enterprise OSPF routing domain and maintain connectivity to remote enterprise locations.

☐ Deploy HQ-R2.

Important

Dual-Stack Enterprise Network

Sonoran Outdoor Equipment operates a dual-stack IPv4 and IPv6 network. As a new enterprise device, HQ-R2 must be configured to support both IPv4 and IPv6 operation in accordance with the Enterprise Network Standards Guide and the existing enterprise router configuration standards.

Ensure all required global IPv6 router settings are configured before implementing IPv6 addressing, routing, and gateway redundancy.

#### Step 1 – Install and Connect HQ-R2

At a minimum:

- Add one Cisco 2911 router to the enterprise topology.

- Configure the hostname as HQ-R2.

- Connect HQ-R2 GigabitEthernet0/0 to HQ-S2 GigabitEthernet0/1.

- Configure the basic router settings according to the Enterprise Network Standards Guide.

- Configure interface descriptions according to the Enterprise Network Standards Guide.

- Verify Layer 1 and Layer 2 connectivity.

- Save the device configuration.

- Update the enterprise topology diagram to include HQ-R2.

#### Step 2 – Configure the New Enterprise WAN Connection

At a minimum:

- Determine the next available IPv4 WAN subnet by reviewing the Enterprise IP Addressing Plan.

- Determine the next available IPv6 WAN subnet by reviewing the Enterprise IP Addressing Plan.

- Configure IPv4 addressing on the new WAN connection.

- Configure IPv6 global unicast addressing on the new WAN connection.

- Configure the static IPv6 link-local address FE80::4 on HQ-R2 GigabitEthernet0/1.

- Configure TUC-R2 GigabitEthernet0/1 using its established router link-local address, FE80::2.

- Verify IPv4 and IPv6 connectivity across the new WAN link.

- Save the device configuration.

#### Step 3 – Update the Enterprise Routing Domain

### Background

The addition of HQ-R2 introduces a new routed WAN connection into the enterprise network. To ensure that all enterprise routers can exchange routing information and maintain connectivity during normal operations and failover events, the enterprise OSPF routing domain must be updated.

Configure HQ-R2 to participate in the existing OSPFv2 and OSPFv3 Area 0 routing domain using the same enterprise implementation methods established during ECR-003 and ECR-004. Update TUC-R2 as necessary to support the new WAN connection.

Use the established enterprise OSPF process ID 160 for both OSPFv2 and OSPFv3.

Assign HQ-R2 the manually configured router ID 160.0.0.4 for both OSPFv2 and OSPFv3.

Reference ECR-003 and ECR-004 and the Enterprise Network Standards Guide when completing this implementation.

### Configure HQ-R2

At a minimum:

- Configure HQ-R2 to participate in the existing OSPFv2 routing domain using process ID 160.

- Configure the manually assigned OSPFv2 router ID 160.0.0.4.

- Advertise the new IPv4 WAN network in Area 0.

- Configure OSPFv3 using process ID 160 and router ID 160.0.0.4.

- Enable OSPFv3 on the new IPv6 WAN interface in Area 0.

- Verify OSPFv2 and OSPFv3 neighbor relationships.

- Verify IPv4 and IPv6 routes are successfully learned from the enterprise routing domain.

- Save the device configuration.

### Update Configurations on TUC-R2

At a minimum:

- Update the existing OSPFv2 configuration to advertise the new HQ-R2 WAN network in Area 0.

- Enable OSPFv3 on the new WAN interface in Area 0.

- Verify OSPFv2 and OSPFv3 neighbor relationships with HQ-R2.

- Verify IPv4 and IPv6 routes are successfully exchanged across the new WAN connection.

- Save the device configuration.

### Phase 2 – Update HQ-R1 Enterprise Gateway Interface Addressing

### Background

Prior to deploying gateway redundancy at Phoenix Headquarters, the existing IPv4 gateway addressing on HQ-R1 must be updated to accommodate the HSRPv2 virtual IPv4 gateway addresses.

The current HQ-R1 subinterfaces use the first usable IPv4 address in each implemented VLAN as the default gateway. HSRPv2 will use these first usable IPv4 addresses as the virtual IPv4 gateway addresses presented to enterprise hosts. Therefore, the physical IPv4 addresses assigned to HQ-R1 must be changed before HSRPv2 is deployed.

For VLANs 10, 20, 30, 40, and 50, HQ-R1 will be readdressed using the second usable IPv4 address in each assigned network. The first usable IPv4 address will then be available for use as the HSRPv2 virtual IPv4 gateway.

VLAN 99 – Network Management requires a different IPv4 addressing assignment because the second and third usable IPv4 addresses are already assigned to the HQ-S1 and HQ-S2 management interfaces. These existing switch management addresses will remain unchanged. HQ-R1 will therefore use the fourth usable IPv4 address in the VLAN 99 network.

The existing IPv6 global unicast and link-local addresses on HQ-R1 do not need to be changed. In a future IPv6 HSRPv2 deployment, the virtual IPv6 gateway would be automatically generated by HSRPv2, so the first usable global unicast IPv6 address does not need to be reserved as a virtual gateway address. IPv6 HSRPv2 will not be configured as part of this ECR due to Cisco Packet Tracer limitations.

These changes prepare the Phoenix Headquarters gateway infrastructure for IPv4 HSRPv2 while preserving the existing IPv6 and switch management addressing for future enterprise requirements.

Phoenix Headquarters Gateway Addressing & HSRPv2 Reference

Use the following reference when configuring the redundant gateway infrastructure for Phoenix Headquarters.

| Configuration | VLANs 10, 20, 30, 40, 50 | VLAN 99 – Network Management |
| --- | --- | --- |
| HQ-R1 IPv4 Address | Second usable IPv4 address | Fourth usable IPv4 address |
| HQ-R2 IPv4 Address | Third usable IPv4 address | Fifth usable IPv4 address |
| HSRP Virtual IPv4 Gateway | First usable IPv4 address | First usable IPv4 address |
| HQ-R1 IPv6 Link-Local Address | No change – retain existing address | No change – retain existing address |
| HQ-R2 IPv6 Link-Local Address | FE80::3 | FE80::3 |
| IPv4 HSRP Group Number | Match the associated VLAN ID | 99 |

Important:

IPv4 and IPv6 use different gateway addressing methods. HQ-R1's IPv4 addresses must be changed to make the first usable IPv4 addresses available for the HSRPv2 virtual gateways. Do not change the existing IPv6 addresses on HQ-R1. In a future IPv6 HSRPv2 deployment, the virtual IPv6 gateway addresses would be automatically generated by HSRPv2 and would not require changes to the existing IPv6 global unicast addressing.

☐ Update the enterprise IPv4 gateway interface addressing on HQ-R1.

At a minimum:

- Update the HQ-R1 subinterfaces for VLANs 10, 20, 30, 40, and 50 to use the second usable IPv4 address in their respective networks.

- Update the HQ-R1 VLAN 99 subinterface to use the fourth usable IPv4 address in the VLAN 99 network.

- Reserve the first usable IPv4 address in each implemented VLAN for the HSRPv2 virtual IPv4 gateway.

- Do not modify the existing IPv4 management addresses assigned to HQ-S1 and HQ-S2.

- Do not modify the existing IPv6 global unicast or link-local addresses assigned to the HQ-R1 VLAN subinterfaces.

- Verify that all updated subinterfaces are operational.

- Verify IPv4 connectivity after updating the interface addresses.

- Verify that the existing IPv6 interface addressing remains operational and unchanged.

- Troubleshoot and correct any configuration issues.

### Save the device configuration.

### Phase 3 – Configure Inter-VLAN Routing on HQ-R2

#### Step 1 – Create VLAN Subinterfaces on HQ-R2

### Background

HQ-R2 must provide Layer 3 gateway services for the same implemented Phoenix Headquarters VLANs currently supported by HQ-R1. Configure Router-on-a-Stick on HQ-R2 using the existing enterprise VLAN assignments and dual-stack addressing standards.

For VLANs 10, 20, 30, 40, and 50, HQ-R2 will use the third usable IPv4 address in each assigned network. For IPv6, HQ-R1 retains the first usable global unicast address; therefore, HQ-R2 will use the second usable IPv6 global unicast address in each assigned IPv6 network.

VLAN 99 – Network Management requires different addressing assignments because existing addresses are already assigned to the HQ-S1 and HQ-S2 management interfaces. For IPv4, HQ-R2 will use the fifth usable IPv4 address in the VLAN 99 network. For IPv6, HQ-R1 retains the first usable global unicast address, HQ-S1 and HQ-S2 use the second and third usable addresses, and HQ-R2 will therefore use the fifth usable IPv6 global unicast address.

Configure static IPv6 link-local addressing on each HQ-R2 VLAN subinterface. Configure FE80::3 as the static IPv6 link-local address on each implemented HQ-R2 VLAN subinterface. Because each VLAN operates as a separate IPv6 link, the same router-specific link-local address may be used on each HQ-R2 subinterface. Existing IPv6 link-local addresses on HQ-R1 remain unchanged.

IPv6 HSRPv2 will not be deployed as part of this ECR due to Cisco Packet Tracer limitations. In a full dual-stack HSRPv2 deployment, IPv6 gateway redundancy would also be configured on the HQ-R1 and HQ-R2 VLAN subinterfaces. Unlike IPv4 HSRPv2, which requires an explicitly configured virtual IPv4 gateway address, HSRPv2 automatically generates the virtual IPv6 link-local gateway address. Therefore, the existing IPv6 global unicast addresses assigned to HQ-R1 and HQ-R2 would not need to be changed or reserved to accommodate the virtual IPv6 gateway.

☐ Configure dual-stack Router-on-a-Stick on HQ-R2.

At a minimum:

- Create subinterfaces for the implemented Phoenix Headquarters VLANs:

  - VLAN 10 – Executive

  - VLAN 20 – Human Resources

  - VLAN 30 – Accounting & Finance

  - VLAN 40 – Sales & Customer Service

  - VLAN 50 – Information Technology

  - VLAN 99 – Network Management

- Configure the appropriate 802.1Q VLAN encapsulation on each subinterface.

- For the VLAN 99 subinterface on HQ-R2, use the native keyword with the IEEE 802.1Q encapsulation command:

interface gigabitEthernet 0/0.99

encapsulation dot1Q 99 native

- For VLANs 10, 20, 30, 40, and 50, assign the third usable IPv4 address in the corresponding VLAN network to the HQ-R2 subinterface.

- For VLAN 99, assign the fifth usable IPv4 address in the VLAN 99 network to the HQ-R2 subinterface.

- For VLANs 10, 20, 30, 40, and 50, assign the second usable IPv6 global unicast address in the corresponding IPv6 network to the HQ-R2 subinterface.

- For VLAN 99, assign the fifth usable IPv6 global unicast address in the VLAN 99 network to the HQ-R2 subinterface.

- Configure FE80::3 as the static IPv6 link-local address on each implemented HQ-R2 VLAN subinterface.

- Verify that all required subinterfaces are operational.

- Verify IPv4 and IPv6 connectivity.

- Troubleshoot and correct any configuration issues.

- Save the device configuration.

#### Step 2 – Configure the HQ-S2 Trunk to HQ-R2

Background

The connection between HQ-S2 and the newly deployed HQ-R2 router must carry traffic for all implemented Phoenix Headquarters VLANs. Configure the HQ-S2 interface connected to HQ-R2 as an IEEE 802.1Q trunk using the established enterprise trunking standards.

The trunk configuration must match the Router-on-a-Stick configuration implemented on HQ-R2, including the enterprise native VLAN.

☐ Configure the HQ-S2 connection to HQ-R2 as an enterprise trunk.

At a minimum:

- Configure HQ-S2 GigabitEthernet0/1 as a static trunk.

- Configure VLAN 99 – Network Management as the native VLAN.

- Allow the implemented Phoenix Headquarters VLANs required by HQ-R2 across the trunk:

  - VLAN 10 – Executive

  - VLAN 20 – Human Resources

  - VLAN 30 – Accounting & Finance

  - VLAN 40 – Sales & Customer Service

  - VLAN 50 – Information Technology

  - VLAN 99 – Network Management

- Ensure VLAN 999 is not permitted across the trunk in accordance with enterprise standards.

- Configure an appropriate interface description according to the Enterprise Network Standards Guide.

- Ensure the interface is administratively enabled.

- Verify that the interface is operating as a trunk.

- Verify that the correct native and allowed VLANs are configured.

- Save the device configuration.

### Phase 4 – Deploy IPv4 Gateway Redundancy Using HSRPv2

Background

Phoenix Headquarters now has two routers capable of providing Layer 3 gateway services for the implemented headquarters VLANs. To eliminate the IPv4 default gateway as a single point of failure, Hot Standby Router Protocol Version 2 (HSRPv2) will be implemented on HQ-R1 and HQ-R2 to provide redundant IPv4 gateway services.

HSRPv2 allows the two headquarters routers to operate together while presenting a virtual IPv4 gateway address to devices within each VLAN. Enterprise hosts will use the HSRPv2 virtual IPv4 gateway address rather than the physical IPv4 interface address assigned to either router. If the active gateway becomes unavailable, the standby router can assume the gateway role and continue forwarding traffic.

For this implementation, HQ-R1 will serve as the preferred active gateway, while HQ-R2 will provide standby gateway services. Priority and preemption will be used to establish the preferred gateway role and ensure that HQ-R1 resumes the active role after service is restored.

Configure HSRPv2 for IPv4 on all implemented Phoenix Headquarters VLANs according to the enterprise standards provided in this Enterprise Change Request. Upon completion, verify normal gateway operation, redundancy, failover, and recovery.

IPv6 HSRPv2 will not be deployed as part of this ECR due to Cisco Packet Tracer limitations. In a full dual-stack enterprise deployment, HSRPv2 would also be configured to provide IPv6 first-hop gateway redundancy. IPv6 HSRPv2 uses a separate HSRP group from IPv4 and automatically generates the virtual IPv6 link-local gateway address. The existing IPv6 global unicast addresses assigned to HQ-R1 and HQ-R2 would remain unchanged. IPv6 HSRPv2 configuration will be addressed in a future implementation.

Enterprise HSRPv2 Standards

| Setting | Enterprise Standard |
| --- | --- |
| HSRP Version | Version 2 |
| IPv4 HSRP Group Number | Match the associated VLAN ID |
| Virtual IPv4 Gateway Address | First usable IPv4 address in the associated VLAN network |
| HQ-R1 Priority | 150 |
| HQ-R2 Priority | 100 |
| Preemption | Enabled on the priority router (HQ-R1) |

#### Step 1 – Configure HSRPv2 on HQ-R1

☐ Configure HSRPv2 for IPv4 on each implemented Phoenix Headquarters VLAN using the Enterprise HSRPv2 Standards shown above.

Configure HSRPv2 on each HQ-R1 VLAN subinterface using the following command template. Replace the placeholders with the appropriate values for each implemented VLAN.

standby version 2

standby <IPv4-group-#> ip <First Usable IPv4 Address>

standby <IPv4-group-#> priority 150

standby <IPv4-group-#> preempt

### Example Configuration – IPv4 HSRPv2 (VLAN 100)

interface GigabitEthernet0/0.100

standby version 2

standby 100 ip 10.10.10.1

standby 100 priority 150

standby 100 preempt

!

Note:

Example Only: The configuration shown demonstrates the HSRPv2 command syntax. Use the enterprise standards defined above when configuring the implemented Phoenix Headquarters VLANs.

- Repeat this configuration for each implemented Phoenix Headquarters VLANs.

- Save the device configuration.

⚠ Important – IPv6 HSRPv2 Configuration Reference

IPv6 HSRPv2 will not be configured in Cisco Packet Tracer as part of this ECR due to Packet Tracer limitations. However, IPv6 first-hop redundancy would normally be implemented as part of a complete dual-stack enterprise deployment. The following commands are provided as a reference for future IPv6 HSRPv2 deployment.

On the preferred active router, IPv6 HSRPv2 would use the following command syntax:

standby <IPv6-group-#> ipv6 autoconfig

standby <IPv6-group-#> priority 150

standby <IPv6-group-#> preempt

Example Configuration – IPv6 HSRPv2 (VLAN 100)

interface GigabitEthernet0/0.100

standby version 2

standby 200 ipv6 autoconfig

standby 200 priority 150

standby 200 preempt

The ipv6 autoconfig command allows HSRPv2 to automatically generate the virtual IPv6 link-local gateway address. The IPv6 HSRP group number must be different from the IPv4 HSRP group configured on the same interface.

Do not enter the IPv6 HSRPv2 commands in Packet Tracer for this ECR.

#### Step 2 – Configure HSRPv2 on HQ-R2

☐ Configure HSRPv2 for IPv4 on each implemented Phoenix Headquarters VLAN using the Enterprise HSRPv2 Standards shown above.

Configure HSRPv2 on each HQ-R2 VLAN subinterface using the following command template. Replace the placeholders with the appropriate values for each implemented VLAN.

Interface g0/0.100

standby version 2

standby <IPv4-group-#> ip <First Usable IPv4 Address>

standby <IPv4-group-#> priority 100

Example Configuration – IPv4 HSRPv2 (VLAN 100)

interface GigabitEthernet0/0.100

standby version 2

standby 100 ip 10.10.10.1

standby 100 priority 100

Note:

Example Only: The configuration shown demonstrates the IPv4 HSRPv2 command syntax. Use the enterprise standards defined above when configuring the implemented Phoenix Headquarters VLANs.

- Repeat the IPv4 HSRPv2 configuration for each implemented Phoenix Headquarters VLAN.

- Verify that HQ-R2 participates as the standby IPv4 gateway.

- Save the device configuration.

#### Step 3 – Verify IPv4 HSRPv2 Operation

At a minimum:

- Verify HSRPv2 has formed successfully for IPv4 on each implemented Phoenix Headquarters VLAN.

- Verify the IPv4 HSRP group number matches the associated VLAN ID.

- Verify the correct virtual IPv4 gateway address is configured for each VLAN.

- Verify HQ-R1 is operating as the Active router for each HSRP group.

- Verify HQ-R2 is operating as the Standby router for each HSRP group.

- Verify the configured HSRP priorities on HQ-R1 and HQ-R2.

- Verify preemption is enabled on HQ-R1.

- Verify IPv4 connectivity through the redundant gateway configuration.

- Troubleshoot and correct any configuration issues.

- Save the device configurations.

### Phase 5 – Update the Enterprise Routing Domain

Background

With HQ-R2 now connected to both the Phoenix Headquarters LAN and the enterprise WAN, it must participate in the existing dynamic routing environment. The enterprise uses OSPFv2 for IPv4 routing and OSPFv3 for IPv6 routing, and HQ-R2 must exchange routing information with the appropriate neighboring enterprise routers for both protocols.

The Phoenix Headquarters VLAN subinterfaces on HQ-R1 and HQ-R2 also participate in the OSPF routing domain so that their connected IPv4 and IPv6 networks can be advertised throughout the enterprise. However, these subinterfaces connect to end-user and management VLANs rather than router-to-router transit networks. Therefore, they should advertise their connected networks without forming OSPF neighbor relationships across the Phoenix VLANs.

To accomplish this, the Phoenix VLAN subinterfaces for VLANs 10, 20, 30, 40, 50, and 99 must be configured as passive interfaces for both OSPFv2 and OSPFv3 on HQ-R1 and HQ-R2. Configuring an interface as passive prevents OSPF Hello messages and neighbor formation on that interface while allowing the connected network to continue to be advertised through OSPF.

The routed WAN interfaces used for communication between enterprise routers must remain non-passive, because these interfaces are expected to exchange OSPF routing information and form neighbor relationships.

As part of this phase, you will update and verify the OSPFv2 and OSPFv3 configurations on both headquarters routers to ensure that enterprise routes continue to be advertised while OSPF neighbor relationships form only across the appropriate routed WAN links.

Use the established enterprise OSPF standards and the implementation methods previously used during ECR-003 and ECR-004.

At a minimum:

- Update the existing OSPFv2 process 160 on HQ-R2 to include all implemented Phoenix Headquarters IPv4 VLAN networks in Area 0.

- Configure the Phoenix VLAN subinterfaces for VLANs 10, 20, 30, 40, 50, and 99 on both HQ-R1 and HQ-R2 as passive interfaces for OSPFv2 and OSPFv3. If the passive-interface configuration was not previously applied to HQ-R1, correct it at this time.

- Ensure that the routed WAN interfaces used for communication between enterprise routers remain non-passive for OSPFv2 and OSPFv3 so the routers can continue forming neighbor relationships and exchanging routing information.

- Enable OSPFv3 process 160 on all implemented HQ-R2 IPv6 VLAN subinterfaces in Area 0.

- Do not modify the established HQ-R2 OSPF router ID 160.0.0.4.

- Verify that OSPFv2 and OSPFv3 neighbor relationships are established only on the appropriate routed WAN links and are not formed across the Phoenix user or management VLANs.

- Verify that the Phoenix VLAN networks continue to be advertised throughout the enterprise routing domain.

- Verify normal enterprise connectivity after updating the routing domain.

- Troubleshoot and correct any routing issues.

- Save the device configuration.

Important – Passive OSPF Interfaces

A passive OSPF interface does not remove the connected network from OSPF. The network continues to be advertised to other OSPF routers. The passive-interface setting only prevents OSPF Hello packets from being transmitted on that interface and prevents neighbor relationships from forming there.

The Phoenix Headquarters VLAN subinterfaces on both HQ-R1 and HQ-R2 should be configured as passive interfaces for OSPFv2 and OSPFv3 because they connect to user and management VLANs rather than dedicated OSPF transit networks.

If the required passive-interface configuration was not previously applied to HQ-R1 for both OSPFv2 and OSPFv3, correct the configuration at this time.

The routed WAN interfaces used to exchange routing information between enterprise routers must remain non-passive for OSPFv2 and OSPFv3 so that the required OSPF neighbor relationships can form.

### Phase 6 – Apply Established Enterprise Security Standards

HQ-R2 has now been integrated into the enterprise network and configured to provide redundant IPv4 gateway services for Phoenix Headquarters. Before the new router can be considered production-ready, it must comply with the enterprise security controls established during previous Enterprise Change Requests.

Review the security requirements implemented during ECR-005 and ECR-007 and apply all applicable device-management security and management-access controls to HQ-R2. Do not remove or modify security controls already operating on existing enterprise devices unless specifically required by this change request.

Security Policy Guidance

When applying the established enterprise security standards to HQ-R2, replicate only the controls that must remain effective if HQ-R2 becomes the active headquarters gateway.

HQ-R2 must include the established secure remote-management access controls and the Accounting & Finance protection ACLs. These controls are associated with device management and the Accounting & Finance gateway interface and therefore must also be enforced by the redundant headquarters router.

The established external HTTPS access policy does not need to be replicated on HQ-R2 at this time. IPv4 ACL 110 and the IPv6 WEB-IPV6-ACL are applied outbound on HQ-R1 GigabitEthernet0/1 toward the Internet Service Provider. Because all enterprise Internet-bound traffic currently continues to exit through HQ-R1, these ACLs remain effective regardless of which headquarters router is serving as the active first-hop gateway. Internet-edge redundancy and any required changes to the placement of these security controls may be addressed in a future Enterprise Change Request (ECR).

At a minimum:

- Configure HQ-R2 to comply with the established SSH secure remote-management standards.

- Apply the established IPv4 and IPv6 management-access ACLs so that only authorized Information Technology hosts can remotely manage HQ-R2.

- Apply the established IPv4 and IPv6 Accounting & Finance protection ACLs to the appropriate HQ-R2 VLAN subinterface.

- Do not duplicate the external HTTPS access ACLs on HQ-R2 at this time. Under the current network design, these controls remain enforced on HQ-R1 at the enterprise Internet egress point.

- Verify authorized management access succeeds and unauthorized management access is denied.

- Verify the Accounting & Finance protection policy remains effective.

- Verify the security configuration does not interfere with normal routing or gateway redundancy.

- Save the device configuration.

### Verify HQ-R2 Accounting & Finance Security Controls

On HQ-R2, verify that the IPv4 and IPv6 Accounting & Finance protection ACLs are complete and applied to the correct VLAN 30 subinterface.

Run:

show access-lists ACCOUNTING-ACL

show ipv6 access-list ACCOUNTING-IPV6-ACL

show ip interface GigabitEthernet0/0.30

show ipv6 interface GigabitEthernet0/0.30

| Validation Requirement | Verified |
| --- | --- |
| The IPv4 Accounting ACL permits established HTTPS return traffic before the final deny. | ☐ |
| The IPv6 Accounting ACL permits HTTPS return traffic from ISP-WEB1 before the final deny. | ☐ |
| ACCOUNTING-ACL is applied outbound on HQ-R2 G0/0.30. | ☐ |
| ACCOUNTING-IPV6-ACL is applied outbound on HQ-R2 G0/0.30. | ☐ |

Important:

When replicating the Accounting & Finance protection policy on HQ-R2, review the existing ACCOUNTING-ACL and ACCOUNTING-IPV6-ACL configurations on HQ-R1 before applying them. Ensure the ACLs permit all traffic required for normal enterprise operation and deny unauthorized access to the Accounting & Finance network as defined by the established enterprise security policy. Do not assume that an existing ACL configuration should be copied without first validating its entries and placement.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete command output or ping results.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1A – Validate Expected Enterprise Connectivity

Verify that normal enterprise communications are functioning before implementing the gateway failover validation. Perform the following ping connectivity tests.

Expected Successful IPv4 Ping Connectivity

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 | ☐ | ☐ |
| Phoenix PC-AF1 | Flagstaff FLG-PC-SCS1 | ☐ | ☐ |
| Phoenix PC-IT1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-HR1 | Phoenix PC-EX1 | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

Expected Successful IPv6 Ping Connectivity

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 | ☐ | ☐ |
| Phoenix PC-AF1 | Flagstaff FLG-PC-SCS1 | ☐ | ☐ |
| Phoenix PC-IT1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-HR1 | Phoenix PC-EX1 | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

#### Expected Successful HTTPS Access

#### Use the web browser on the specified workstation to verify access to the external enterprise HTTPS service.

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | ISP-WEB1 enter https://198.51.100.10 | ☐ | ☐ |
| Flagstaff FLG-PC-SCS1 | ISP-WEB1 enter https://[2001:DB8:FFFF:1::10] | ☐ | ☐ |

#### Expected Successful Secure Management Access

#### From Phoenix PC-IT1, establish an SSH connection to HQ-R2 to verify that authorized Information Technology hosts can securely manage the newly deployed redundant router.

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | HQ-R2 | ☐ | ☐ |

#### Step 1B - Expected Restricted Enterprise Connectivity

The following connectivity tests are expected to fail as a result of established enterprise security policies. A failed connection during these tests indicates that the applicable access control policy is operating as intended.

#### Expected Blocked Accounting & Finance Access

Verify that an unauthorized enterprise department cannot initiate communications to the protected Accounting & Finance VLAN.

IPv4

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | Phoenix PC-AF1 | ☐ | ☐ |
| Flagstaff FLG-PC-SCS1 | Phoenix PC-AF1 | ☐ | ☐ |

IPv6

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | Phoenix PC-AF1 | ☐ | ☐ |
| Tucson PC-WH1 | Phoenix PC-AF1 | ☐ | ☐ |

#### Expected Blocked External HTTPS Access

From the Tucson Warehouse & Inventory workstation, attempt to access ISP-WEB1 using the web browser. Warehouse & Inventory is not authorized to access the external HTTPS service.

IPv4

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Tucson PC-WH1 | ISP-WEB1 enter https://198.51.100.10 | ☐ | ☐ |

IPv6

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Tucson PC-WH1 | ISP-WEB1 enter https://[2001:DB8:FFFF:1::10] | ☐ | ☐ |

#### Expected Blocked Secure Management Access

From an unauthorized non-IT workstation, attempt to establish an SSH connection to HQ-R2. Enterprise remote-management access is restricted to authorized Information Technology hosts.

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-SCS1 | HQ-R2 | ☐ | ☐ |

Important:

For the tests in this section, a blocked connection is the expected result. If any restricted connection succeeds, troubleshoot and correct the applicable access control configuration before proceeding with the HSRPv2 failover validation.

#### Step 2 – Record Baseline HSRPv2 Status and Routing

Before simulating an HSRPv2 gateway failure, verify the current HSRP operating state and record the normal IPv4 routing path.

On HQ-R1 and HQ-R2, run:

show standby brief

Review the output and verify that:

- HQ-R1 is operating as the Active router for each configured IPv4 HSRP group.

- HQ-R2 is operating as the Standby router for each configured IPv4 HSRP group.

- The expected virtual IPv4 gateway address is displayed for each HSRP group.

Next, run the following IPv4 traceroute tests and copy the results into the space provided. These results establish the normal routing path before simulating an HSRPv2 gateway failure for VLAN 30 – Accounting & Finance.

IPv4 Traceroute Connectivity

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 |  |
| Phoenix PC-AF1 | Flagstaff PC-SCS1 |  |

Important:

Review the traceroute results and identify the gateway/router used during normal operation. HQ-R1 should be operating as the Active HSRPv2 gateway for VLAN 30 before proceeding with the failover test.

Important:

Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify implementation.

#### Step 3 – Simulate an HSRPv2 Gateway Failure

On HQ-R1, simulate the loss of the active gateway for VLAN 30 – Accounting & Finance by administratively shutting down only the GigabitEthernet0/0.30 subinterface.

This validation uses VLAN 30 – Accounting & Finance as the controlled HSRPv2 failover test. Successful failover of this VLAN, combined with verification of the operating state of all configured HSRP groups, provides validation of the redundant gateway implementation without unnecessarily disrupting multiple enterprise VLANs.

configure terminal

interface GigabitEthernet0/0.30

shutdown

end

Allow sufficient time for HSRPv2 to detect the loss of the active VLAN 30 gateway and transition HQ-R2 to the Active role for HSRP group 30.

#### Verify HSRPv2 Failover

On HQ-R2, run:

show standby brief

Review the output and verify that HQ-R2 is now operating as the Active router for HSRP group 30. The remaining HSRP groups should continue operating in their normal Active/Standby state.

### HSRPv2 Failover Verification

Before proceeding, review the show standby brief output and verify each of the following conditions.

| Verification Requirement | Verified |
| --- | --- |
| HQ-R2 is operating as the Active router for HSRP group 30. | ☐ |
| HSRP group 30 displays the correct virtual IPv4 gateway address. | ☐ |
| The remaining HSRP groups continue operating in their normal Active/Standby state. | ☐ |

Do not proceed until all HSRPv2 failover conditions above have been verified.

Important:

Shut down only GigabitEthernet0/0.30. Do not shut down the parent GigabitEthernet0/0 interface or multiple VLAN subinterfaces. This validation is designed to test a controlled HSRPv2 failover for a single VLAN while the remaining headquarters gateway services continue operating normally.

Do not restore the subinterface at this time. Continue to the next validation step to verify HSRPv2 failover, routing, connectivity, and security-policy operation while HQ-R2 is serving as the Active gateway for VLAN 30.

#### Step 4 – Validate Enterprise Redundancy

### Verify Accounting & Finance Security Policy During Failover

While HQ-R2 is serving as the Active HSRPv2 gateway for VLAN 30 – Accounting & Finance, verify that the established Accounting & Finance access-control policy remains enforced.

At a minimum:

- Verify that an authorized Executive Management workstation can communicate with PC-AF1.

- Verify that an unauthorized enterprise workstation cannot initiate communication with PC-AF1.

- Record the results in the validation table.

- If an unauthorized connection succeeds, troubleshoot and correct the Accounting & Finance ACL configuration on HQ-R2 before restoring normal gateway operation.

IPv4 Ping Security Validation

| Source Department | Destination Department | Expected Result | Success | Fail |
| --- | --- | --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 | Permit | ☐ | ☐ |
| Phoenix PC-HR1 | Phoenix PC-AF1 | Deny | ☐ | ☐ |

IPv4 HTTPS Security Validation

| Source Department | Destination | Expected Result | Success | Fail |
| --- | --- | --- | --- | --- |
| Phoenix PC-AF1 | ISP-WEB1 https://198.51.100.10 | Permit | ☐ | ☐ |

Verify Continued Enterprise Connectivity

Verify that enterprise services outside the affected Accounting & Finance gateway continue to operate normally following the simulated gateway failure. These tests confirm that the controlled HSRPv2 failover did not adversely affect other enterprise network services.

IPv4 Ping Connectivity

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-HR1 | Flagstaff FLG-PC-SCS1 | ☐ | ☐ |

Important:

Both connectivity tests should remain successful while HQ-R2 is operating as the Active gateway for VLAN 30. If either test fails, troubleshoot and resolve the issue before continuing.

Confirm Failover to Standby Router

Run the following traceroute tests and compare the results to the baseline collected in Step 2. Verify that the traceroute reflects the updated routing path through HQ-R2 following the simulated failure of HQ-R1 while maintaining successful connectivity to the destination.

IPv4 Traceroute Connectivity

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 |  |
| Phoenix PC-AF1 | Flagstaff FLG-PC-SCS1 |  |

⚠ Important:
Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify the implementation.

Note:

Compare the traceroute results to the baseline collected in Step 2 and document any differences in the routing path observed following the simulated gateway failure.

#### Step 5 – Restore Normal Enterprise Operations

On HQ-R1, restore the VLAN 30 – Accounting & Finance gateway subinterface:

configure terminal

interface GigabitEthernet0/0.30

no shutdown

end

Allow sufficient time for HSRPv2 to recognize that the preferred gateway has been restored.

On HQ-R1 and HQ-R2, run:

show standby brief

Verify that:

- HQ-R1 has resumed the Active role for HSRP group 30 as a result of its higher priority and configured preemption.

- HQ-R2 has returned to the Standby role for HSRP group 30.

- The correct virtual IPv4 gateway address is displayed for HSRP group 30.

- Enterprise connectivity remains operational following recovery.

- No HSRPv2 gateway redundancy issues remain before proceeding to the remaining verification steps.

### HSRPv2 Recovery Verification

Before proceeding, review the show standby brief output and verify each of the following conditions.

| Verification Requirement | Verified |
| --- | --- |
| HQ-R1 has returned to the Active router role for HSRP group 30. | ☐ |
| HQ-R2 has returned to the Standby router role for HSRP group 30. | ☐ |
| HSRP group 30 displays the correct virtual IPv4 gateway address. | ☐ |
| The remaining HSRP groups continue operating in their normal Active/Standby states. | ☐ |

Do not proceed until all HSRPv2 recovery conditions above have been verified.

Important:

Do not modify or shut down any additional HSRP subinterfaces during this validation.

#### Step 6 – Verify First-Hop Redundancy Protocol Operation

Run the following commands on both headquarters routers.

IPv4 HSRPv2 Gateway Redundancy

Show standby

Highlight the following:

- IPv4 HSRP group number

- Virtual IPv4 gateway address

- Router priority

- Active router

- Standby router

- Current HSRP state

- Preemption status

| Device | Required Output - Paste show standby |
| --- | --- |
| HQ-R1 |  |
| HQ-R2 |  |

#### Step 7 – Verify Enterprise OSPF Operation

Run the following commands on HQ-R2 and TUC-R2.

Review the command output and verify that the OSPF routing process is operating according to the Enterprise Change Request requirements.

Copy and paste the complete command output into the table below.

This verifies that HQ-R2 has successfully joined the enterprise OSPF routing domain and established neighbor adjacencies.

### OSPFv2

show ip ospf neighbor

| Device | Required Output - Paste show ip ospf neighbor |
| --- | --- |
| HQ-R2 |  |
| TUC-R2 |  |

### OSPFv3

show ipv6 ospf neighbor

| Device | Required Output - Paste show ipv6 ospf neighbor |
| --- | --- |
| HQ-R2 |  |
| TUC-R2 |  |

### OSPFv2 and OSPFv3 Protocol Configuration Verification

On HQ-R1 and HQ-R2, run:

show ip protocols

show ipv6 protocols

Review the command output and verify that the OSPF routing process is operating according to the Enterprise Change Request requirements.

Copy and paste the complete command output into the table below.

| Device | Required Output - Paste show ip protocols |
| --- | --- |
| HQ-R1 |  |
| HQ-R2 |  |

| Device | Required Output - Paste show ipv6 protocols |
| --- | --- |
| HQ-R1 |  |
| HQ-R2 |  |

Review the command output and verify each of the following:

- The Phoenix Headquarters VLAN subinterfaces for VLANs 10, 20, 30, 40, 50, and 99 are configured as passive interfaces for both OSPFv2.

- The routed WAN interfaces required for OSPF neighbor formation remain non-passive.

- The correct OSPF processes and router IDs are operating on each headquarters router.

- The OSPFv2 and OSPFv3 configurations match the Enterprise Change Request requirements.

Correct any unexpected OSPF configuration or operational results before proceeding.

Important – OSPFv3 Passive-Interface Verification

The show ipv6 protocols command does not identify which OSPFv3 interfaces are configured as passive interfaces in Packet Tracer. The OSPFv3 passive-interface configuration will be validated in a later verification step using the running configuration.

#### Step 8 – Verify Final Device Configurations

Run the following command on:

- HQ-R1

- HQ-R2

- TUC-R2

show running-config

Highlight the following on the appropriate devices:

### HQ-R1

- Updated IPv4 gateway interface addresses

- OSPFv3 Passive-Interface Configuration showing that the Phoenix Headquarters VLAN subinterfaces for VLANs 10, 20, 30, 40, 50, and 99 are configured as passive interfaces under OSPFv3 process 160.

### HQ-R2

- Router hostname

- WAN interface configuration

- Router-on-a-Stick subinterfaces

- IPv4 and IPv6 interface addressing

- Interface descriptions

- Access control lists

- OSPFv3 Passive-Interface Configuration: Highlight the configuration showing that the Phoenix Headquarters VLAN subinterfaces for VLANs 10, 20, 30, 40, 50, and 99 are configured as passive interfaces under OSPFv3 process 160.

### TUC-R2

- New WAN interface configuration

- IPv4 and IPv6 addressing

Important:

Before submitting your show running-config output, highlight the required configuration items identified in this Enterprise Change Request.

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| HQ-R2 |  |
| TUC-R2 |  |

#### Step 9 – Update Enterprise Documentation

### Background

Enterprise documentation must accurately reflect the production network before an Enterprise Change Request can be closed. Following successful implementation and validation of this Enterprise Change Request, update the enterprise documentation to reflect the approved changes. These updates ensure future network engineers have accurate information for ongoing operations, troubleshooting, and future enterprise expansions.

☐ Update the enterprise documentation.

#### Enterprise Network Standards Guide

Open the Sonoran Outdoor Equipment Enterprise Network Standards Guide and complete the ECR-008 – HSRP and Network Redundancy Standards Update section.

Update the appropriate tables and standards to document:

- HQ-R2 hardware and device information.

- New and updated HQ-R2 router interface information.

- The new HQ-R2 to TUC-R2 private WAN connection.

- Enterprise HSRPv2 implementation standards, including the approved virtual gateway and redundancy design.

- Any additional enterprise standards introduced or modified by this Enterprise Change Request.

Important:

Complete the ECR-008 standards update only after the implementation has been configured and successfully validated. Do not overwrite the original baseline standards unless specifically directed by the ECR.

#### Enterprise IP Addressing Plan

Open the Sonoran Outdoor Equipment Enterprise IP Addressing Plan and complete the ECR-008 – HSRP Addressing Update Addendum.

Update the addendum to document:

- HQ-R2 interface addressing.

- The new HQ-R2 to TUC-R2 WAN network and interface addressing.

- Updated Phoenix Headquarters router interface addressing.

- HSRPv2 virtual IPv4 gateway addresses.

- Any additional addressing information affected by this Enterprise Change Request.

Important:

Complete the ECR-008 addressing addendum only after the HSRP implementation has been configured and successfully validated. Do not overwrite the original baseline addressing documented in the Enterprise IP Addressing Plan.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Network Standards Guide –

ECR-008 HSRP and Network Redundancy Standards Update

☐  Updated Enterprise IP Addressing Plan –

ECR-008 HSRP Addressing Update Addendum

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 08/20/2026