ECR-001

## Enterprise Network Initial Deployment

| Requested By | Network Engineering Department |
| --- | --- |
| Assigned To | Junior Network Engineer |
| Priority | High |
| Change Type | Standard |
| Implementation Window | Estimated Implementation Time: 2–4 hours See established due date |
| Business Impact | High – Initial deployment of the enterprise network. |

## Business Justification

Sonoran Outdoor Equipment has completed the initial planning and documentation phases of the enterprise network. The Enterprise Network Standards Guide and Enterprise Network Addressing Plan have been approved by the Network Engineering Department. Your team has now been authorized to begin the initial deployment of the enterprise network at Phoenix Headquarters.

The first phase of the deployment focuses on establishing the headquarters network, implementing VLAN segmentation, enabling inter-VLAN routing, and providing basic connectivity between enterprise locations using static routing. These services provide the foundation for all future enterprise change requests.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

- Build the initial Sonoran Outdoor Equipment enterprise network.

- Configure the baseline settings on enterprise routers and switches.

- Implement the initial Phoenix Headquarters VLAN infrastructure.

- Configure Router-on-a-Stick (ROAS) inter-VLAN routing at Phoenix Headquarters.

- Configure the initial Tucson Distribution Center and Flagstaff Sales Office networks.

- Implement static routing to provide end-to-end enterprise connectivity.

- Verify enterprise connectivity and validate the network implementation using Cisco IOS verification commands.

- Demonstrate professional network implementation and validation practices using enterprise documentation.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Company Overview

✔ Enterprise Network Standards Guide

✔ Enterprise Network Addressing Plan

✔ Enterprise Topology (.pkt)

✔ Interface Addressing Plan

## Required Resources

  Three Cisco 2911 routers

  Three Cisco Catalyst switches

  Console cables

  Ethernet cables

  Serial DCE/DTE cables

  Packet Tracer

  Enterprise topology (.pkt)

  Enterprise Change Request template

## Implementation Plan

### Phase 1 – Review Documentation

Before beginning the implementation, review the enterprise documentation provided for this project. These documents serve as your engineering references and should be used throughout the implementation process.

Refer to the following documents as needed while completing this Enterprise Change Request:

- Enterprise Company Overview

- Enterprise Network Standards Guide

- Enterprise Network Addressing Plan

- Enterprise Interface Addressing Plan

- Enterprise Network Topology

Verify that you understand the implementation requirements before proceeding to Phase 2.

### Phase 2 – Build the Enterprise Physical Network

In this phase, you will construct the initial Sonoran Outdoor Equipment enterprise network. This implementation establishes the physical infrastructure that will support all future Enterprise Change Requests (ECRs). Build the network carefully and verify each connection before proceeding with device configuration.

#### Step 1: Review the Enterprise Documentation

Before connecting any devices, review the following documents:

- Enterprise Network Standards Guide

- Enterprise Network Addressing Plan

- Enterprise Interface Addressing Plan

- Enterprise Topology (.pkt)

Verify that you understand the required device names, interface standards, VLAN assignments, and addressing requirements before beginning the implementation.

#### Step 2: Build the Enterprise Topology

Using the Enterprise Network Standards Guide, build the initial Sonoran Outdoor Equipment enterprise network in Packet Tracer.

The Enterprise Network Standards Guide provides the information needed to construct the network, including:

- Device names

- Router and switch interface standards

- WAN connections

- Switch uplinks

- End-user device connections

As you build the network, update your Enterprise Topology (.pkt) documentation to reflect the physical layout. Your completed topology should accurately represent the enterprise network before device configuration begins.

#### Step 3: Install and Connect the Network Devices

Using the interface standards defined in the Enterprise Network Standards Guide, install and connect the devices required for the initial Sonoran Outdoor Equipment enterprise network.

Although the detailed VLAN and inter-VLAN routing configuration in this Enterprise Change Request focuses on Phoenix Headquarters, you will build the complete enterprise topology, including the Tucson Distribution Center and Flagstaff Sales Office. Implementing all three locations provides the remote networks required to configure, verify, and troubleshoot static routing between enterprise sites. Additional services and technologies will be implemented at each location in future Enterprise Change Requests.

The initial enterprise deployment includes the following business locations:

- Phoenix Headquarters

- Tucson Distribution Center

- Flagstaff Sales Office

At a minimum, complete the following:

- ☐ Install the required routers and switches for each location.

- ☐ Install at least one end device on each implemented user network.

- ☐ Connect all required router-to-router WAN links.

- ☐ Connect all required router-to-switch uplinks.

- ☐ Connect all required switch-to-end device connections.

As additional Enterprise Change Requests are completed throughout the semester, you will continue expanding this enterprise network by adding devices, services, and new business locations.

Verify that all network connections follow the Enterprise Network Standards Guide before proceeding to device configuration.

#### Step 4: Verify Physical Connectivity

Before applying any configurations:

- ☐ Verify that every cable is connected to the correct interface.

- ☐ Verify that all devices match the enterprise topology.

- ☐ Verify that the physical layout matches your Enterprise Topology (.pkt) documentation.

- ☐ Correct any cabling errors before continuing.

#### Step 5: Save Your Progress

Before moving to Phase 3:

- ☐ Save your Packet Tracer (.pkt) topology file.

- ☐ Verify that your Enterprise Topology documentation reflects the physical network that you have built.

### Phase 3 – Configure Enterprise Devices

In this phase, you will configure the foundational settings on all enterprise routers and switches. These configurations establish a consistent baseline across the enterprise network and ensure that all devices comply with the Enterprise Network Standards Guide before implementing additional network services and technologies.

Unless otherwise directed, all device configurations must comply with the Enterprise Network Standards Guide.

#### Step 1: Configure HQ-R1

Configure the Phoenix Headquarters router according to the Enterprise Network Standards Guide.

At a minimum, complete the following:

- Configure the hostname.

- Disable DNS lookup.

- Configure the required passwords using the enterprise password standard.

- Encrypt all plaintext passwords.

- Configure the required banner message.

- Configure interface descriptions per the Enterprise Network Standards Guide and the Lab Process & Expectations Guide.

- Save the running configuration.

#### Step 2: Configure TUC-R2

Configure the Tucson Distribution Center router according to the Enterprise Network Standards Guide.

Complete the same baseline configuration tasks performed on HQ-R1.

#### Step 3: Configure FLG-R3

Configure the Flagstaff Sales Office router according to the Enterprise Network Standards Guide.

Complete the same baseline configuration tasks performed on HQ-R1.

#### Step 4: Configure Enterprise Switches

Configure each enterprise switch according to the Enterprise Network Standards Guide.

The initial enterprise deployment includes:

- ☐ HQ-S1

- ☐ HQ-S2

- ☐ TUC-S2

- ☐ FLG-S3

At a minimum, complete the following tasks on each switch:

- Configure the hostname.

- Disable DNS lookup.

- Configure the required passwords using the enterprise password standard.

- Encrypt all plaintext passwords.

- Configure the required banner message.

- Configure interface descriptions on all active interfaces.

- Configure the switch management interface (SVI).

- Configure the default gateway.

- Save the running configuration.

#### Step 5: Verify Baseline Device Configurations

Before proceeding to the next phase:

- ☐ Verify that all routers and switches have been configured according to the Enterprise Network Standards Guide.

- ☐ Verify that interface descriptions have been configured on all active interfaces.

- ☐ Verify that the running configuration has been saved on each device.

Troubleshoot and correct any configuration issues before proceeding.

### Phase 4 – Configure Enterprise VLANs & Inter-VLAN Routing

In this phase, you will configure the Layer 2 and Layer 3 services required for Phoenix Headquarters. You will create the enterprise VLANs, configure the required trunk links, implement Router-on-a-Stick (ROAS) inter-VLAN routing, and verify communication between the implemented business networks.

Unless otherwise directed, all configurations must comply with the Enterprise Network Standards Guide.

Important:

Inter-VLAN routing was introduced in CNT150 and is a foundational skill for this course. If you need a refresher, review the Router-on-a-Stick module, lab, or your instructor's notes before continuing. Future Enterprise Change Requests will assume proficiency with this configuration.

#### Step 1: Configure Enterprise VLANs

☐ Create the enterprise VLAN infrastructure on the Phoenix Headquarters switches.

At a minimum:

- Create all enterprise VLANs defined in the Enterprise Network Standards Guide.

- Assign the appropriate VLAN names.

- Configure the required access ports for the departments listed below using the switch port assignments defined in the Enterprise Network Standards Guide.

HQ-S1 – Configure Access Ports

- Human Resources

- Sales & Customer Service

- Information Technology

HQ-S2 – Configure Access Ports

- Executive Administration

- Human Resources

- Accounting & Finance

- Sales & Customer Service

- Information Technology

- Configure the Phoenix Headquarters Network Management VLAN according to the Enterprise Network Standards Guide and the Enterprise IP Addressing Plan.

- Configure all unused switch ports according to the Enterprise Network Standards Guide.

Note:

All enterprise VLANs should be created; however, only the departments listed above and the Phoenix Headquarters Network Management VLAN are implemented as part of ECR-001. VLANs identified as Reserved in the Enterprise Network Standards Guide should be created on the switches, but their associated access ports should remain unconfigured until directed by a future Enterprise Change Request (ECR).

#### Step 2: Configure Switch Trunk Links

☐ Configure the required trunk links for the Phoenix Headquarters network.

During the initial enterprise deployment, all physical uplink cables between HQ-S1 and HQ-S2 will be installed according to the Enterprise Network Standards Guide. Although three uplink cables are physically connected, only a single trunk link will be activated during ECR-001. The remaining uplink interfaces will remain administratively shut down and will be configured as an EtherChannel in a future Enterprise Change Request.

This implementation approach reflects common enterprise deployment practices in which the physical infrastructure is installed during the initial network build, while additional services and redundancy are implemented as the network evolves.

At a minimum:

- Configure the router-to-switch trunk connection using the interface assignments defined in the Enterprise Network Standards Guide.

- Configure a single active switch-to-switch trunk link between HQ-S1 and HQ-S2 using F0/1 ↔ F0/1.

- Leave the remaining physical uplink connections (F0/2 ↔ F0/2 and F0/3 ↔ F0/3) administratively shut down.

- Verify the native VLAN.

- Verify the allowed VLANs.

- Configure interface descriptions on all trunk interfaces according to the Enterprise Network Standards Guide and the Lab Process & Expectations Guide.

Important:

Activating multiple parallel switch links without configuring EtherChannel creates Layer 2 loops. During ECR-001, only the F0/1 ↔ F0/1 trunk link should be active. The remaining uplink interfaces will be physically connected but remain administratively shut down until EtherChannel is implemented in a future Enterprise Change Request.

Troubleshoot and correct any trunking issues before proceeding.

#### Step 3: Configure Router-on-a-Stick (ROAS)

☐ Configure inter-VLAN routing on HQ-R1.

Note:

Configure router subinterfaces only for the VLANs implemented during ECR-001. Although all enterprise VLANs have been created on the switches, configure subinterfaces only for the departments currently being deployed. Do not configure subinterfaces for VLANs reserved for future enterprise expansion.

At a minimum:

- Configure the required router subinterfaces.

- Configure IEEE 802.1Q encapsulation for each implemented VLAN.

- Configure the default gateway address for each VLAN.

- Configure interface descriptions on all subinterfaces.

#### Step 4: Configure Department Workstations

☐ Configure the department workstations identified in Step 1 using the switch port assignments defined in the Enterprise Network Standards Guide.

At a minimum:

- Assign the appropriate static IPv4 address.

- Configure the subnet mask.

- Configure the default gateway.

- Verify that each workstation is connected to the correct switch port and assigned to the appropriate VLAN.

Note:

Install end devices only for the departments identified in Step 1. Reserved ports and future expansion ports should remain unconfigured until implemented in a future Enterprise Change Request (ECR).

#### Step 5: Verify Inter-VLAN Routing

☐ Verify successful inter-VLAN communication.

At a minimum:

- Verify that each end device can communicate with its default gateway.

- Verify communication between devices located on different VLANs.

- Verify that router subinterfaces are operational.

- Verify that VLAN assignments and trunk links are functioning correctly.

Troubleshoot and correct any issues before proceeding to the next phase.

### Phase 5 – Configure Remote Site Networks

In this phase, you will configure the initial network services for the Tucson Distribution Center and Flagstaff Sales Office. During the initial enterprise deployment, each remote location operates as a single routed network using VLAN 1. Additional VLANs and enterprise services will be implemented in future Enterprise Change Requests as the organization grows.

Unless otherwise directed, all configurations must comply with the Enterprise Network Standards Guide and the Enterprise Interface Addressing Plan.

#### Step 1: Configure the Tucson Distribution Center

☐ Configure the initial Tucson Distribution Center network.

At a minimum:

- Configure VLAN 1 (Warehouse & Inventory).

- Configure the switch management interface (SVI) using VLAN 1.

- Configure the router LAN interface.

- Configure one end device.

- Assign IPv4 addressing according to the Enterprise Interface Addressing Plan.

#### Step 2: Configure the Flagstaff Sales Office

☐ Configure the initial Flagstaff Sales Office network.

At a minimum:

- Configure VLAN 1 (Sales & Customer Service).

- Configure the switch management interface (SVI) using VLAN 1.

- Configure the router LAN interface.

- Configure one end device.

- Assign IPv4 addressing according to the Enterprise Interface Addressing Plan.

#### Step 3: Verify Remote Site Connectivity

☐ Verify that each remote site has been configured correctly.

At a minimum:

- Verify that the router LAN interfaces are operational.

- Verify that the switch management interfaces (SVIs) are operational.

- Verify that the end devices can communicate with their default gateway.

- Verify that all interfaces have been configured according to the Enterprise Interface Addressing Plan.

Troubleshoot and correct any issues before proceeding to the next phase.

### Phase 6 – Configure Static Routing

In this phase, you will implement static routing to provide full connectivity between the Phoenix Headquarters, Tucson Distribution Center, and Flagstaff Sales Office. Each router will use a different static routing method to demonstrate multiple enterprise routing techniques.

Unless otherwise directed, all configurations must comply with the Enterprise Network Standards Guide and the Enterprise Interface Addressing Plan.

#### Step 1: Configure Static Routes on HQ-R1

☐ Configure fully specified static routes on the Phoenix Headquarters router.

At a minimum:

- Configure fully specified static routes to all remote enterprise networks.

- Verify that each route references both the outgoing interface and the next-hop IP address.

- Ensure that all remote networks are reachable from Phoenix Headquarters.

#### Step 2: Configure Static Routes on TUC-R2

☐ Configure directly connected (exit interface) static routes on the Tucson Distribution Center router.

At a minimum:

- Configure exit interface static routes to all remote enterprise networks.

- Verify that the correct outgoing interfaces are used.

- Ensure that all remote networks are reachable from Tucson.

#### Step 3: Configure Static Routes on FLG-R3

☐ Configure next-hop static routes on the Flagstaff Sales Office router.

At a minimum:

- Configure next-hop static routes to all remote enterprise networks.

- Verify that each route references the correct next-hop IP address.

- Ensure that all remote networks are reachable from Flagstaff.

#### Step 4: Verify Enterprise Routing

☐ Verify successful enterprise routing.

At a minimum:

- Verify that all expected static routes appear in the routing tables.

- Verify connectivity between all enterprise locations.

- Verify that end devices can communicate with devices located on remote enterprise networks.

- Verify that all routing methods are functioning correctly.

Troubleshoot and correct any routing issues before proceeding to the next phase.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete command output or ping results.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Verify Enterprise Connectivity

☐ Verify end-to-end enterprise communication.

For each connectivity test:

- Include the source device in the screenshot.

- Include the successful ping results.

- Ensure the destination address is visible in the screenshot.

⚠ IMPORTANT
Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the successful ping results so the Director of Network Engineering can verify the implementation.

At a minimum, perform and document the following connectivity tests:

Phoenix Headquarters

Verify that inter-VLAN routing is functioning correctly within Phoenix Headquarters by performing the following connectivity tests between the enterprise departments listed below.

Connectivity Test

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Human Resources | Accounting & Finance |  |
| Executive Administration | Information Technology |  |
| Sales & Customer Service | Human Resources |  |

Remote Site Connectivity

Verify that enterprise routing is functioning correctly by performing the following connectivity tests between business locations.

Connectivity Test

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Human Resources (Phoenix Headquarters) | Warehouse & Inventory (Tucson Distribution Center) |  |
| Accounting & Finance (Phoenix Headquarters) | Sales & Customer Service (Flagstaff Sales Office) |  |
| Warehouse & Inventory (Tucson Distribution Center) | Sales & Customer Service (Flagstaff Sales Office) |  |

Connectivity Test

|  |  | Screenshot |
| --- | --- | --- |
| HR → | Accounting |  |
| Executive → | IT |  |
| Sales & Customer Service → | Human Resources |  |

#### Step 2 – Verify Enterprise Routing

☐ Verify enterprise routing on all routers.

Paste the output in space below of the following command:

show ip route

Verify that:

- Connected networks are present.

- Local networks are present.

- Static routes have been installed correctly.

- All expected enterprise networks appear in the routing table.

| Device | Required Output - Paste show ip route |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

#### Step 3 – Verify VLAN Configuration

☐ Verify the VLAN configuration on all enterprise switches.

From each switch, paste the output of:

show vlan brief

Verify that:

- Required VLANs have been created.

- Access ports are assigned to the correct VLANs.

- VLAN assignments match the Enterprise Network Standards Guide.

| Device | Required Output - Paste show vlan brief |
| --- | --- |
| HQ-S1 |  |
| HQ-S2 |  |

#### Step 4 – Verify Device Configurations

☐ Verify the final configuration of each enterprise device.

Paste the complete output of:

show running-config

for every router and switch.

This configuration will be used to verify compliance with the Enterprise Network Standards Guide and assist with troubleshooting if necessary.

Important:
Before submitting your show running-config output, highlight the required configuration items identified in this Enterprise Change Request. For ECR-001, highlight the interface descriptions on all active interfaces.

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

#### Step 5 – Final Validation

☐ Confirm that the Enterprise Change Request has been completed successfully.

Verify that:

- All required configurations have been completed.

- All verification tests have passed.

- All device configurations have been saved.

- The enterprise network is operating according to the Enterprise Network Standards Guide.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Enterprise Topology (.pkt)

☐  Enterprise Implementation (.pkt or .pka, depending on the assignment)Version 1: Updated 06/29/2026