ECR-002

## Enterprise IPv6 Addressing Deployment

| Requested By | Network Engineering Department |
| --- | --- |
| Assigned To | Junior Network Engineer |
| Priority | Medium |
| Change Type | Normal |
| Implementation Window | Estimated Implementation Effort: 1–2 hours See established due date. |
| Business Impact | Low – IPv6 is being introduced alongside the existing IPv4 infrastructure with minimal user disruption expected. |

## Business Justification

Sonoran Outdoor Equipment has approved the next phase of the enterprise network deployment by implementing IPv6 throughout the existing enterprise infrastructure. As the organization continues to grow, IPv6 provides a scalable addressing solution that supports future expansion while maintaining compatibility with the existing IPv4 network.

To minimize business disruption, the enterprise will implement a dual-stack network in which IPv4 and IPv6 operate simultaneously. Existing IPv4 services must remain fully operational throughout the deployment while IPv6 is introduced across the enterprise routers, switch management interfaces, and end devices.

This Enterprise Change Request (ECR) implements IPv6 addressing based on the approved Enterprise IP Addressing Plan and prepares the enterprise network for future routing implementation. All IPv6 addressing, interface assignments, and documentation must conform to the standards defined in the Enterprise Network Standards Guide and the Enterprise IP Addressing Plan.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

- Implement enterprise IPv6 addressing using the approved Enterprise IP Addressing Plan.

- Configure a dual-stack enterprise network that supports both IPv4 and IPv6.

- Configure IPv6 addresses on enterprise router interfaces, switch management interfaces (SVIs), and end devices.

- Enable IPv6 routing on enterprise routers.

- Verify IPv6 addressing and dual-stack connectivity throughout the enterprise network.

- Update enterprise documentation to reflect the completed network changes.

- Apply enterprise implementation standards while following a structured change management process.

## Prerequisites

Before beginning this Enterprise Change Request (ECR), verify that the following requirements have been completed:

✔ Successfully completed ECR-001 – Enterprise IPv4 Deployment

✔ Review the assigned IPv6 training materials

✔ Successfully completed Enterprise IP Addressing Plan

## Required Resources

  Enterprise Network Standards Guide

  Enterprise IP Addressing Plan

  Updated Enterprise Topology (.pkt)

  Lab Process & Expectations Guide

  Enterprise Change Request (ECR-002)

## Implementation Plan

### Phase 1 – Configure Enterprise IPv6 Addressing

#### Step 1: Configure Router IPv6 Interfaces

☐  Configure IPv6 addressing on all active router interfaces according to the Enterprise IP Addressing Plan.

At a minimum:

- Enable IPv6 routing on all enterprise routers.

- Configure the assigned global unicast IPv6 address on each active router interface and subinterface according to the Enterprise IP Addressing Plan.

- Configure a static link-local IPv6 address on each active router interface and subinterface according to the enterprise IPv6 addressing standards found in the Enterprise IP Addressing Plan.

- Verify that all configured router interfaces and subinterfaces are operational.

- Update interface descriptions according to the Enterprise Network Standards Guide and the Lab Process & Expectations Guide.

#### Step 2: Configure Switch IPv6 Management

Important:

Enterprise switches require additional platform-specific configuration to fully support IPv6 routing. Verify that all prerequisite switch configurations required by the enterprise standards have been completed before proceeding with IPv6 deployment.

☐  Configure IPv6 management addressing on the enterprise switches according to the Enterprise IP Addressing Plan.

At a minimum:

  Enable IPv6 dual-stack support on all enterprise switches

Configure the assigned global unicast IPv6 address on each switch management interface (SVI) according to the Enterprise IP Addressing Plan.

  Configure the appropriate IPv6 default gateway on each switch according to the Enterprise IP Addressing Plan.

  Verify that each switch management interface (SVI) is operational.

  Update the switch management interface descriptions according to the Enterprise Network Standards Guide and the Lab Process & Expectations Guide.

  Troubleshoot and correct any IPv6 addressing or connectivity issues before proceeding.

#### Step 3: Update Interface Descriptions

☐  Update enterprise interface descriptions to accurately reflect the IPv6 deployment.

At a minimum:

  Update interface descriptions on all affected router and switch interfaces to include the appropriate IPv6 network information.

  Verify that all interface descriptions accurately reflect the current enterprise configuration.

  Ensure all interface descriptions comply with the Enterprise Network Standards Guide and the Lab Process & Expectations Guide before proceeding.

Important:

Interface descriptions are part of the enterprise documentation and must be updated whenever network changes are implemented. Review the Lab Process & Expectations Guide to ensure all interface descriptions comply with the required enterprise documentation standards.

#### Step 4: Configure End Device IPv6 Addressing

☐  Configure IPv6 addressing on all required enterprise end devices according to the Enterprise IP Addressing Plan.

At a minimum:

- Configure the assigned global unicast IPv6 address on each required end device according to the Enterprise IP Addressing Plan.

- Configure the appropriate IPv6 default gateway on each end device according to the Enterprise IP Addressing Plan.

- Verify that each end device has been assigned the correct IPv6 address and prefix length.

- Verify that each end device is configured with the correct IPv6 default gateway.

- Troubleshoot and correct any IPv6 addressing or configuration issues before proceeding.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete command output or ping results.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Verify Enterprise IPv6 Connectivity

☐ Verify IPv6 connectivity on all directly connected enterprise networks.

For each connectivity test:

- Include the source device in the screenshot.

- Include the successful ping results.

- Ensure the destination address is visible in the screenshot.

⚠ IMPORTANT:

Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the successful ping results so the Director of Network Engineering can verify the implementation.

At a minimum, perform and document the following connectivity tests:

Phoenix Headquarters

Verify that IPv6 communication is functioning correctly within Phoenix Headquarters by performing the following connectivity tests between the enterprise departments listed below.

Connectivity Test

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Human Resources | Accounting & Finance |  |
| Executive Administration | Information Technology |  |
| Sales & Customer Service | Human Resources |  |

Remote Site Connectivity

Verify IPv6 communication within each remote site by performing the following connectivity tests.

Connectivity Test - Tucson Distribution Center

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Warehouse & Inventory (Tucson PC) | Warehouse IPv6 Default Gateway |  |
| Warehouse & Inventory (Tucson PC) | TUC-S3 Management SVI |  |

Connectivity Test - Flagstaff Sales Office

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Sales & CSR (Flagstaff PC) | Sales & CSR IPv6 Default Gateway |  |
| Sales & CSR (Flagstaff PC) | FLG-S4 Management SVI |  |

Connectivity Test – Enterprise WAN Links

Verify IPv6 communication between directly connected enterprise routers across each WAN link.

| Source Router | Destination Router | Screenshot |
| --- | --- | --- |
| HQ-R1 | TUC-R2 |  |
| HQ-R1 | FLG-R3 |  |
| TUC-R2 | FLG-R3 |  |

#### Step 2 – Verify Enterprise IPv6 Addressing

☐ Verify enterprise IPv6 addressing on all routers.

Paste the output in space below of the following command:

show ipv6 route

Verify that:

- All required connected (C) IPv6 networks are present.

- All required local (L) IPv6 addresses are present.

- The IPv6 network prefixes match the Enterprise IP Addressing Plan.

- Each connected IPv6 network is associated with the correct router interface.

- No unexpected IPv6 routes are present.

| Device | Required Output - Paste show ipv6 route |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

#### Step 3 – Verify Switch IPv6 Management Interfaces

☐ Verify the IPv6 management interface configuration on all enterprise switches.

Paste the output of the following commands:

- HQ-S1# show ipv6 interface vlan 99

- HQ-S2# show ipv6 interface vlan 99

- TUC-S3# show ipv6 interface vlan 1

- FLG-S4# show ipv6 interface vlan 1

Verify that:

  The switch management SVI has the correct global unicast IPv6 address.

  The SVI is operational.

  The IPv6 address and prefix match the Enterprise IP Addressing Plan.

| Device | Required Output - Paste show ipv6 interface vlan [ID#] |
| --- | --- |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-S3 |  |
| FLG-S4 |  |

#### Step 4 – Verify Device Configurations

☐ Verify the final configuration of each enterprise device.

For every router and switch paste the complete output of:

show running-config

This configuration will be used to verify compliance with the Enterprise Network Standards Guide and assist with troubleshooting if necessary.

Important:

Before submitting your show running-config output, highlight the configuration items implemented as part of this Enterprise Change Request.

For ECR-002, highlight:

- IPv6 unicast routing (ipv6 unicast-routing)

- All active router interfaces and subinterfaces

  - Global unicast IPv6 addresses

  - Static IPv6 link-local addresses

  - Updated interface descriptions

- All switch management interfaces (SVIs)

  - Global unicast IPv6 addresses

  - Updated interface descriptions

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

#### Step 5 – Final Validation

☐ Confirm that the Enterprise Change Request has been completed successfully.

Verify that:

- All required configurations have been completed.

- All verification tests have passed.

- All device configurations have been saved.

- The enterprise network is operating according to the Enterprise Network Standards Guide and the Enterprise IP Addressing Plan.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Enterprise Topology (.pkt)

☐  Enterprise Implementation (.pkt or .pka, depending on the assignment)

## Version 1: Updated 07/02/2026