ECR-004

## Enterprise OSPFv3 Deployment

| Requested By | Network Engineering Department |
| --- | --- |
| Assigned To | Junior Network Engineer |
| Priority | Medium |
| Change Type | Normal |
| Implementation Window | Estimated Implementation Time: 1–3 hours See established due date. |
| Business Impact | Medium - Existing IPv4 OSPF routing will not be affected. |

## Business Justification

Sonoran Outdoor Equipment has successfully deployed OSPFv2 throughout the enterprise IPv4 network. As part of the organization's long-term transition to a dual-stack enterprise infrastructure, the Network Engineering Department has approved the deployment of OSPFv3 to provide dynamic routing for the enterprise IPv6 network.

Implementing OSPFv3 will prove automatic route exchange, fast convergence, and improved scalability as additional enterprise locations are added. Existing IPv4 OSPF routing must remain fully operational throughout this implementation.

Upon completion of this change, both IPv4 and IPv6 routing will operate dynamically, providing a fully dual-stack enterprise routing environment.

Successful completion of this change request establishes the enterprise routing foundation required for future implementations, first-hop redundancy, and enterprise network services.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

  Deploy OSPFv3 throughout the enterprise network.

  Advertise enterprise LAN, WAN, inter-VLAN, management, and future expansion networks using OSPFv3.

  Verify OSPFv3 neighbor adjacencies and dynamic route exchange.

  Configure and advertise the enterprise IPv6 default route from the Autonomous System Boundary Router (ASBR).

  Validate enterprise connectivity and routing using connectivity testing and Cisco IOS show commands.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Successfully completed ECR-001 – Enterprise IPv4 Deployment

✔ Successfully completed ECR-002 – Enterprise IPv6 Deployment

✔ Successfully completed ECR-003 – Enterprise OSPFv2 Deployment

## Required Resources

  Enterprise Network Standards Guide

  Enterprise IP Addressing Plan

  Enterprise Topology

  Enterprise Change Request (ECR-004)

  Lab Process & Expectations Guide

## Implementation Plan

### Phase 1 – Implement Enterprise OSPFv3

#### Step 1:  Configure IPv6 Future Expansion Loopback Interfaces

☐ Configure IPv6 addressing on the enterprise future expansion loopback interfaces according to the Enterprise IP Addressing Plan.

### Background

The enterprise future expansion loopback interfaces implemented during ECR-003 now require IPv6 addressing to support dual-stack operation. These loopback interfaces represent future enterprise locations and will participate in the enterprise IPv6 OSPF routing domain.

### At a minimum:

- Configure the assigned global unicast IPv6 address on each future expansion loopback interface according to the Enterprise IP Addressing Plan.

- Verify that each loopback interface is operational.

- Verify that each configured IPv6 address and prefix length match the Enterprise IP Addressing Plan.

- Ensure the loopback interfaces are ready to participate in the enterprise IPv6 OSPF routing domain before proceeding.

#### Step 2: Configure OSPFv3 Interfaces

☐ Configure OSPFv3 on all required enterprise IPv6 interfaces according to the Enterprise Network Standards Guide.

### Background

Unlike OSPFv2, which advertises IPv4 networks using the network command in router configuration mode, OSPFv3 is enabled directly on individual IPv6-enabled interfaces. Each participating interface must be assigned to the appropriate OSPFv3 process and area.

To ensure a consistent enterprise deployment, the Director of Network Engineering has standardized the OSPFv3 implementation method used throughout the organization. Configure each participating IPv6 interface using the following interface configuration command:

ipv6 ospf 160 area 0

### At a minimum:

- Configure OSPFv3 on all required enterprise IPv6 interfaces using the following interface configuration command:

ipv6 ospf 160 area 0

- Configure OSPFv3 on all participating router interfaces and subinterfaces that are part of the enterprise IPv6 routing domain.

- Configure OSPFv3 on the future expansion loopback interfaces.

- Do not configure OSPFv3 on interfaces that are not intended to participate in enterprise dynamic routing.

- Verify that every required IPv6 interface has been successfully assigned to OSPF Area 0 before proceeding.

#### Step 3: Configure the Enterprise IPv6 Default Route

☐  Configure the enterprise IPv6 default route on HQ-R1, the enterprise Autonomous System Boundary Router (ASBR).

### Background

### The enterprise Autonomous System Boundary Router (ASBR) provides the connection between the enterprise OSPFv3 routing domain and external IPv6 networks. Prior to this Enterprise Change Request, the Internet Service Provider (ISP) completed the IPv6 configuration of the ISP router. The ISP-facing interface, ISP-R1 GigabitEthernet0/0, has already been configured with IPv6 address 2001:DB8:203:113::1/64.

### As part of this change request, configure the enterprise-facing interface on HQ-R1, establish IPv6 connectivity to the ISP router, and configure the enterprise IPv6 default route. Once verified, advertise the IPv6 default route throughout the enterprise OSPFv3 routing domain.

### At a minimum:

  Configure the ISP facing interface HQ-R1 GigabitEthernet0/1 with IPv6 address 2001:DB8:203:113::2/64.

  Verify IPv6 connectivity between HQ-R1 and ISP-R1 before configuring the default route.

  Configure a fully specified IPv6 default static route on HQ-R1, the enterprise Autonomous System Boundary Router (ASBR).

The route must include both the exit interface and the next-hop IPv6 address.

  Verify that the IPv6 default route has been successfully installed in the routing table.

  Configure the ASBR to advertise the enterprise IPv6 default route throughout the OSPFv3 routing domain.

  Verify that TUC-R2 and FLG-R3 have dynamically learned the enterprise IPv6 default route.

  Verify that all enterprise locations maintain IPv6 connectivity after the routing update.

Important:

Only the Autonomous System Boundary Router (ASBR) should originate the enterprise IPv6 default route into the OSPFv3 routing domain. Internal enterprise routers should learn the default route dynamically through OSPFv3 and should not be configured with their own IPv6 default static routes.

#### Step 4: Verify OSPF Neighbor Adjacencies

☐  Verify that OSPF neighbor adjacencies have been successfully established between all enterprise routers.

### Background

The Network Engineering team must verify that all enterprise routers have successfully established OSPF neighbor relationships. Neighbor adjacencies confirm that routers are exchanging OSPF routing information and are prepared to calculate the enterprise routing topology.

### At a minimum:

- Verify that all expected OSPF neighbor adjacencies have been established.

- Confirm that each enterprise router has formed neighbor relationships with the appropriate adjacent routers.

- Verify that no unexpected OSPF neighbor issues are present before proceeding.

- Document the OSPF neighbor information for use during the Enterprise Verification & Validation process.

Important:

Neighbor adjacencies must be successfully established before relying on OSPF to exchange routing information. Troubleshoot and resolve any OSPF neighbor issues before continuing with the routing migration.

#### Step 5: Verify Dynamic Route Exchange

☐  Verify that OSPF is successfully exchanging enterprise routing information.

### Background

After OSPF neighbor adjacencies have been established, enterprise routers begin exchanging routing information and calculating the best paths throughout the enterprise network.

### At a minimum:

- Verify that OSPF has successfully learned the required enterprise routes.

- Run show ipv6 route and review the routing table.

- Document the routing table. This information will be used to validate routing during the final verification phase.

- Verify that all required enterprise LAN, WAN, and future expansion networks have been learned through OSPF before proceeding.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete command output or ping results.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Verify Enterprise IPv6 Connectivity

☐  Verify IPv6 end-to-end enterprise communication between business locations.

For each connectivity test:

- Include the source device in the screenshot.

- Include the successful ping results.

- Ensure the destination address is visible in the screenshot.

⚠ Important:

Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the successful ping results so the Director of Network Engineering can verify the implementation.

At a minimum, perform and document the following connectivity tests:

Phoenix Headquarters

Verify that Phoenix Headquarters can communicate with the enterprise remote sites and future expansion networks.

IPv6 Connectivity Test

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Human Resources | Warehouse & Inventory (Tucson Distribution Center) |  |
| Executive Administration | Tubac Adventure Lodge (Future Expansion Loopback) |  |
| Accounting & Finance | Sales & Customer Service (Flagstaff Sales Office) |  |
| Information Technology | Greer Mountain Cabins (Future Expansion Loopback) |  |

Tucson Distribution Center

Verify that Tucson Distribution Center can communicate with enterprise business locations and future expansion networks.

IPv6 Connectivity Test

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Warehouse & Inventory | Sales & Customer Service (Flagstaff Sales Office) |  |
| Warehouse & Inventory | Greer Mountain Cabins (Future Expansion Loopback) |  |
| Warehouse & Inventory | Human Resources (Phoenix Headquarters) |  |

Flagstaff Sales Office

Verify that Flagstaff Sales Office can communicate with enterprise business locations and future expansion networks.

IPv6 Connectivity Test

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Sales & Customer Service | Warehouse & Inventory (Tucson Distribution Center) |  |
| Sales & Customer Service | Tubac Adventure Lodge (Future Expansion Loopback) |  |
| Sales & Customer Service | Executive Administration (Phoenix Headquarters) |  |

Enterprise Internet Connectivity

Verify that each enterprise location can reach the Internet Service Provider (ISP) by successfully pinging the IPv6 address assigned to ISP-R1 GigabitEthernet0/0 (2001:DB8:203:113::1/64).

At a minimum, perform and document the following connectivity tests:

IPv6 Connectivity Test

| Source Location | Destination ISP | Screenshot |
| --- | --- | --- |
| Phoenix Headquarters | ISP |  |
| Phoenix Human Resources | ISP |  |
| Phoenix Sales & Customer Service | ISP |  |
| Phoenix Information Tech | ISP |  |
| Phoenix Accounting | ISP |  |
| Tucson Distribution Center | ISP |  |
| Flagstaff Sales Office | ISP |  |

⚠ Important

Each screenshot must include the source device, the successful ping results, and the destination IPv6 address 2001:DB8:203:113::1/64. These tests validate that the enterprise default route has been successfully propagated by the Autonomous System Boundary Router (ASBR) and that all enterprise locations can reach the Internet Service Provider.

#### Step 2 – Verify Enterprise OSPF Configuration

☐  Verify the enterprise OSPF implementation using the required show commands below.

### Background

Enterprise network validation requires reviewing information from multiple show commands to confirm that all configurations have been implemented correctly. The commands below verify both the OSPF configuration and the operational status of the enterprise routing environment.

Important:

Before submitting your validation package, highlight the required configuration items identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with the Enterprise Change Request and Enterprise Network Standards.

### Verify OSPF Process Configuration

### show ipv6 protocols

Highlight the following:

- IPv6 Routing Protocol

- Interfaces Participating in OSPFv3

- Paste the complete output in the table provided below.

| Device | Required Output - Paste show ipv6 protocols |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

### Verify OSPF Neighbor Adjacencies

### show ipv6 ospf neighbor

Highlight the following:

  Neighbor Router IDs

  Neighbor State

  Neighbor Interface

- Paste the complete output in the table provided below.

| Device | Required Output - Paste show ipv6 ospf neighbor |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

### Verify Enterprise Routing

### show ipv6 route

Highlight the following:

OSPF Routes (O)

OSPF Default Route (O*E2 or whatever route type is appropriate after default-information originate)

- Paste the complete output in the table provided below.

| Device | Required Output - Paste show ipv6 route |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

#### Step 3 – Verify Final Device Configurations

☐ Verify the final configuration of each enterprise device.

For every router paste the complete output of:

show running-config

This configuration will be used to verify compliance with the Enterprise Network Standards Guide and assist with troubleshooting if necessary.

Important:

Before submitting your show running-config output, highlight the required configuration items identified in this Enterprise Change Request. For ECR-001, highlight the interface descriptions on all active interfaces.

Highlight the following:

- Enterprise Default Static Route

- default-information originate

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

#### Step 4 – Final Validation

☐  Confirm that the Enterprise Change Request has been completed successfully.

Verify that:

- All required configurations have been completed.

- All verification tests have passed.

- All device configurations have been saved.

- The enterprise network is operating according to the Enterprise Network Standards Guide.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Topology

☐  Enterprise Implementation

## Version 1: Updated 07/03/2026