ECR-006

## Enterprise DHCP & Stateless DHCPv6 Deployment

| Requested By | Network Engineering Department |
| --- | --- |
| Assigned To | Junior Network Engineer |
| Priority | High |
| Change Type | Normal |
| Implementation Window | Estimated Implementation Time: 2–4 hours See established due date. |
| Business Impact | Medium – Automates enterprise address assignment while requiring verification to ensure clients receive the correct network configuration. |

## Business Justification

As Sonoran Outdoor Equipment continues to expand its enterprise network, manually assigning IPv4 and IPv6 addresses to end-user devices has become increasingly time-consuming and difficult to manage. The Network Engineering Department has approved the deployment of centralized Dynamic Host Configuration Protocol (DHCP) services to improve operational efficiency, simplify network administration, and support future enterprise growth.

This Enterprise Change Request integrates enterprise DHCP and DHCPv6 services into the existing network infrastructure. Building upon the enterprise foundation established in previous change requests, the implementation will automate IPv4 and IPv6 address assignment while maintaining consistent addressing standards across all business locations.

The implementation will follow the Enterprise Network Standards Guide to ensure DHCP services are deployed consistently throughout the enterprise and provide a scalable foundation for future network expansion and additional enterprise services.

## Objectives

Upon successful completion of this Enterprise Change Request (ECR), you will be able to:

- Implement centralized enterprise DHCP services to automate IPv4 address assignment throughout the Sonoran Outdoor Equipment network.

- Configure DHCP relay services to support centralized IPv4 address management across multiple enterprise locations.

- Implement centralized Stateless DHCPv6 services to distribute enterprise IPv6 network configuration information.

- Configure enterprise router interfaces to support Stateless Address Autoconfiguration (SLAAC) and Stateless DHCPv6 operation.

- Configure enterprise DHCP and DHCPv6 services according to the Enterprise Network Standards Guide and Enterprise IP Addressing Plan.

- Verify successful IPv4 and IPv6 address assignment for enterprise client devices.

- Validate enterprise DHCP and DHCPv6 services using Cisco IOS show commands and end-device testing.

- Troubleshoot and resolve DHCP and DHCPv6 addressing issues while maintaining enterprise network connectivity.

- Document and submit an enterprise validation package demonstrating successful implementation of centralized enterprise addressing services.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Successfully complete ECR-001 through ECR-005.

✔ The enterprise network is fully operational using both IPv4 and IPv6 addressing.

✔ OSPFv2 and OSPFv3 are fully implemented and enterprise connectivity has been verified.

✔ The Enterprise Network Standards Guide is available for reference.

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

  Enterprise Network Standards Guide

  Enterprise IP Addressing Plan

  Enterprise Topology

  Enterprise Change Request (ECR-006)

  Lab Process & Expectations Guide

## Implementation Plan

### Phase 1 – Configure Enterprise DHCPv4 Services

☐ Configure centralized IPv4 DHCP services for the Sonoran Outdoor Equipment enterprise.

### Background

As the Sonoran Outdoor Equipment enterprise continues to expand, manually assigning IPv4 addresses to client devices has become increasingly difficult to manage and maintain. To improve operational efficiency and simplify enterprise administration, the Network Engineering Department has approved the deployment of centralized Dynamic Host Configuration Protocol (DHCP) services.

The enterprise DHCP server will be centrally hosted at Phoenix Headquarters (HQ-R1). Remote enterprise locations will obtain DHCP services through DHCP relay, allowing all IPv4 address assignments to be managed from a single enterprise location.

The enterprise has deployed a centralized Domain Name System (DNS) server at Phoenix Headquarters. The DNS server has already been configured and is operational using the IPv4 address 10.10.10.9/25. As part of this Enterprise Change Request, configure all enterprise DHCPv4 pools to distribute the enterprise DNS server address to client devices.

### At a minimum:

- Configure centralized IPv4 DHCP services on HQ-R1.

- Create an IPv4 DHCP address pool for each enterprise VLAN according to the Enterprise Network Standards Guide.

- Configure all DHCP pools using the approved enterprise DHCP naming convention.

- Configure excluded IPv4 address ranges according to the Enterprise Network Standards Guide.

- Configure the required default gateway, DNS server (10.10.10.9), and any additional enterprise DHCP options for each DHCP pool.

- Configure DHCP relay services on TUC-R2 and FLG-R3 to forward DHCP requests to the centralized DHCP server.

- Verify that all DHCP pools have been configured correctly before proceeding.

### Phase 2 – Configure Enterprise DHCPv6 Services

☐ Configure centralized Stateless DHCPv6 services for the Sonoran Outdoor Equipment enterprise.

### Background

The Sonoran Outdoor Equipment enterprise has adopted a dual-stack network architecture supporting both IPv4 and IPv6. To maintain consistency across the enterprise, IPv6 address assignment will use Stateless DHCPv6 in combination with Stateless Address Autoconfiguration (SLAAC).

Under this deployment model, client devices will automatically generate their IPv6 addresses while obtaining additional network configuration information from the centralized DHCPv6 server located at Phoenix Headquarters.

The enterprise Domain Name System (DNS) server supports both IPv4 and IPv6. As part of this Enterprise Change Request, configure the centralized Stateless DHCPv6 service to distribute the enterprise IPv6 DNS server address to enterprise client devices.

### At a minimum:

- Configure centralized Stateless DHCPv6 services on HQ-R1.

- Create a DHCPv6 address pool for each enterprise VLAN according to the approved enterprise naming convention.

- Configure each DHCPv6 pool to provide the required enterprise network information.

- Configure the enterprise IPv6 DNS server address (2001:DB8:C001:50::9/64) in each DHCPv6 pool according to the Enterprise IP Addressing Plan.

- Configure the appropriate router interfaces to support Stateless DHCPv6 operation.

- Configure DHCPv6 relay services on TUC-R2 and FLG-R3 to forward DHCPv6 requests to the centralized DHCPv6 server.

- Verify that all DHCPv6 pools have been configured successfully before proceeding.

### Phase 3 – Deploy Enterprise Client Addressing Services

☐ Deploy automated IPv4 and IPv6 addressing services throughout the enterprise network.

### Background

With centralized DHCP services operational, enterprise client devices will transition from manually assigned addressing to automated address assignment. This deployment simplifies network administration while ensuring consistent enterprise addressing standards are applied across all business locations.

### At a minimum:

- Configure enterprise client devices to obtain IPv4 addressing automatically.

- Configure enterprise client devices to obtain IPv6 addressing automatically using Stateless DHCPv6 and SLAAC.

- Verify that enterprise clients receive the correct IPv4 addressing information.

- Verify that enterprise clients receive the correct IPv6 addressing information.

- Verify that enterprise clients receive the appropriate default gateway information.

- Verify that enterprise clients maintain full IPv4 and IPv6 connectivity throughout the enterprise network.

- Troubleshoot and correct any DHCP or DHCPv6 issues before proceeding.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete command output or ping results.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Verify Enterprise Address Assignment

☐ Verify that enterprise client devices have successfully obtained IPv4 and IPv6 addressing through the centralized DHCP services.

### Background

Following the deployment of centralized DHCP and Stateless DHCPv6 services, all enterprise client devices should automatically obtain their IPv4 and IPv6 network configuration. Before validating enterprise connectivity, verify that each enterprise client has received the correct addressing information according to the Enterprise IP Addressing Plan and Enterprise Network Standards Guide.

Important:

Record the IPv4 and IPv6 addresses assigned to each enterprise client. This information will be used throughout the remaining validation activities to verify enterprise connectivity and troubleshoot addressing issues if necessary.

DHCP Addressing Table

| Location | Department | IPv4 Address | IPv4 Default Gateway | IPv6 Address | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |
| HQ-S1 | Human Resources |  |  |  |  |
| HQ-S1 | Sales & Customer Service |  |  |  |  |
| HQ-S1 | Information Technology |  |  |  |  |
| HQ-S2 | Accounting & Finance |  |  |  |  |
| HQ-S2 | Executive Administration |  |  |  |  |
| HQ-S2 | Information Technology |  |  |  |  |
| TUC-S3 | Warehouse & Inventory |  |  |  |  |
| FLG-S4 | Sales & Customer Service |  |  |  |  |

#### Step 2 – Verify Enterprise Connectivity

☐ Verify that enterprise communications between departments and business locations remain fully operational following the deployment of centralized DHCP and Stateless DHCPv6 services.

### Background

Following the implementation of centralized DHCP services, enterprise client devices have transitioned from statically assigned IP addresses to dynamically assigned IPv4 and IPv6 addressing. Before closing this Enterprise Change Request, verify that all business locations continue to communicate successfully and that enterprise operations have not been disrupted by the deployment.

Important:

Each required connectivity test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination IP address or device is visible.

- Include the complete ping results.

- Perform and document both IPv4 and IPv6 connectivity tests for each required validation.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

At a minimum, perform and document the following enterprise connectivity tests.

### Phoenix Headquarters

Verify that Phoenix Headquarters can communicate with enterprise remote locations using both IPv4 and IPv6.

| Source Department | Destination Department | IPv4 Screenshot |
| --- | --- | --- |
| Human Resources | Warehouse & Inventory (Tucson Distribution Center) |  |
| Executive Administration | Sales & Customer Service (Flagstaff Sales Office) |  |

| Source Department | Destination Department | IPv6 Screenshot |
| --- | --- | --- |
| Accounting & Finance | Warehouse & Inventory (Tucson Distribution Center) |  |
| Information Technology | Sales & Customer Service (Flagstaff Sales Office) |  |

### Tucson Distribution Center

Verify that Tucson Distribution Center can communicate with enterprise business locations using both IPv4 and IPv6.

| Source Department | Destination Department | IPv4 Screenshot |
| --- | --- | --- |
| Warehouse & Inventory | Executive Administration |  |
| Warehouse & Inventory | Sales & Customer Service (Flagstaff Sales Office) |  |

| Source Department | Destination Department | IPv6 Screenshot |
| --- | --- | --- |
| Warehouse & Inventory | Accounting & Finance |  |
| Warehouse & Inventory | Sales & Customer Service (Flagstaff Sales Office) |  |

### Flagstaff Sales Office

Verify that Flagstaff Sales Office can communicate with enterprise business locations using both IPv4 and IPv6.

| Source Department | Destination Department | IPv4 Screenshot |
| --- | --- | --- |
| Sales & Customer Service | Information Technology |  |
| Sales & Customer Service | Warehouse & Inventory (Tucson Distribution Center) |  |

| Source Department | Destination Department | IPv6 Screenshot |
| --- | --- | --- |
| Sales & Customer Service | Accounting & Finance |  |
| Sales & Customer Service | Warehouse & Inventory (Tucson Distribution Center) |  |

Top of Form

⚠ Important:

The Sonoran Outdoor Equipment enterprise operates as a dual-stack IPv4/IPv6 network. Unless otherwise specified, all required connectivity validation must include successful IPv4 and IPv6 connectivity tests. Every screenshot must include the source device, the successful ping results, and the destination IP address so the Director of Network Engineering can verify that enterprise communications remain fully operational.

#### Step 3 – Verify Enterprise DHCPv4 Services

☐ Verify the enterprise DHCPv4 implementation using the required show commands below.

### Background

Enterprise DHCP validation requires reviewing operational information from the centralized DHCP server to confirm that IPv4 address assignment is functioning correctly throughout the enterprise. The commands below verify that DHCP pools have been configured successfully and that enterprise client devices have received IPv4 address leases.

Important:

Before submitting your validation package, highlight the required information identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with this Enterprise Change Request and the Enterprise Network Standards Guide.

### Verify DHCPv4 Address Bindings

show ip dhcp binding

Highlight the following:

- Client IPv4 Address

- Client Identifier (MAC Address)

- Lease Type

Paste the complete output in the table provided below.

| Device | Required Output - Paste show ip dhcp binding |
| --- | --- |
| HQ-R1 |  |

### Verify Enterprise DHCPv4 Pools

show ip dhcp pool

Highlight the following:

- DHCP Pool Name

- Network

- Leased Addresses

- Available Addresses

Paste the complete output in the table provided below.

| Device | Required Output - Paste show ip dhcp pool |
| --- | --- |
| HQ-R1 |  |

#### Step 4 – Verify Enterprise DHCPv6 Services

☐ Verify the enterprise Stateless DHCPv6 implementation using the required show commands below.

### Background

Enterprise DHCPv6 validation requires reviewing operational information from the centralized DHCPv6 server to confirm that Stateless DHCPv6 services are functioning correctly throughout the enterprise. The commands below verify that DHCPv6 pools have been configured successfully and that enterprise client devices have received the required IPv6 network configuration information.

Important:

Before submitting your validation package, highlight the required information identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with this Enterprise Change Request and the Enterprise Network Standards Guide.

### Verify DHCPv6 Address Bindings

show ipv6 dhcp binding

Highlight the following:

- Client IPv6 Address

- Client DUID

- IAID

Paste the complete output in the table provided below.

| Device | Required Output - Paste show ipv6 dhcp binding |
| --- | --- |
| HQ-R1 |  |

### Verify Enterprise DHCPv6 Pools

show ipv6 dhcp pool

Highlight the following:

- DHCPv6 Pool Name

- DNS Server

- Active Clients

Paste the complete output in the table provided below.

| Device | Required Output - Paste show ipv6 dhcp pool |
| --- | --- |
| HQ-R1 |  |

#### Step 5 – Verify Final Device Configurations

☐ Verify the final configuration of each enterprise device.

### Background

The final device configurations provide the official implementation record for this Enterprise Change Request. The show running-config output will be used to verify compliance with the Enterprise Network Standards Guide, validate the successful implementation of enterprise DHCP and Stateless DHCPv6 services, and assist with troubleshooting if necessary.

Important:

Before submitting your show running-config output, highlight the required configuration items identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with this Enterprise Change Request and the Enterprise Network Standards Guide.

### Highlight the following:

HQ-R1

- IPv4 DHCP excluded-address configurations

- IPv4 DHCP pools

- Enterprise DNS server configuration within each DHCP pool

- Stateless DHCPv6 pools

- Enterprise IPv6 DNS server configuration within each DHCPv6 pool

TUC-R2 and FLG-R3

- IPv4 DHCP relay (helper-address) configuration

- DHCPv6 relay configuration

- Router interface configuration supporting Stateless DHCPv6 operation

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

#### Step 6 – Final Validation

☐ Confirm that the Enterprise Change Request has been completed successfully.

Verify that:

- All required configurations have been completed.

- All verification tests have passed.

- All device configurations have been saved.

- The enterprise network is operating according to the Enterprise Network Standards Guide.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 07/08/2026