ECR-008

## Enterprise Gateway Redundancy Deployment

| Field | Scope |
| --- | --- |
| Requested by | Network Engineering Department |
| Assigned to | Junior Network Engineer |
| Priority | High |
| Change type | Normal |
| Implementation Window | Friday 11:00pm to Saturday 4:00am Estimated Implementation Effort: 2–5 hours See established due date |
| Business Impact | High (Temporary interruption expected during implementation) |
| Rollback Plan | Restore HQ-R1 as the sole enterprise gateway if implementation cannot be completed successfully. |

## Business Justification

As Sonoran Outdoor Equipment has continued to expand, Phoenix Headquarters has become increasingly dependent on continuous network availability. Currently, all Phoenix departments rely on a single router (HQ-R1) as their default gateway for inter-VLAN routing. Although the existing design meets current operational requirements, it introduces a single point of failure. If HQ-R1 becomes unavailable due to planned maintenance, hardware failure, or an unexpected outage, users throughout Phoenix Headquarters would lose connectivity to enterprise resources and remote locations.

To improve network resiliency and business continuity, the Network Engineering Department has approved the deployment of a second headquarters router (HQ-R2). The additional router will provide gateway redundancy for all Phoenix VLANs while supporting future enterprise growth.

As part of this Enterprise Change Request (ECR), Gateway Load Balancing Protocol (GLBP) will be implemented for IPv4 gateway services, while Hot Standby Router Protocol Version 2 (HSRPv2) will provide IPv6 gateway redundancy. Together, these first-hop redundancy protocols will deliver highly available default gateway services for Phoenix Headquarters. GLBP provides both automatic failover and load balancing by distributing IPv4 client traffic across multiple gateway routers, while HSRPv2 provides reliable gateway redundancy for IPv6 traffic. This design improves network availability, optimizes the use of available routing resources, and reduces the impact of a single device failure on business operations.

Following implementation, Phoenix Headquarters will use virtual default gateways for all production VLANs. Client devices, network infrastructure, and DHCP services will be updated to use the GLBP virtual gateway addresses, ensuring transparent gateway redundancy without requiring changes to end-user devices during future gateway failures.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

  Deploy a new enterprise router and integrate it into an existing production network.

  Configure and verify a new routed WAN connection between headquarters and a remote enterprise location.

  Extend the enterprise IPv4 and IPv6 addressing plan to support new network infrastructure.

  Update an existing OSPFv2 and OSPFv3 routing domain to support enterprise expansion.

  Configure and verify Router-on-a-Stick inter-VLAN routing on a redundant headquarters router.

  Modify IPv4 gateway addressing to support first-hop redundancy.

  Configure and verify Gateway Load Balancing Protocol (GLBP) for IPv4 gateway redundancy and load balancing.

  Configure and verify Hot Standby Router Protocol Version 2 (HSRPv2) for IPv6 gateway redundancy.

  Verify enterprise connectivity before, during, and after a simulated gateway failure.

  Validate gateway redundancy using ping, traceroute, GLBP, HSRPv2, and OSPF verification commands.

  Analyze routing path changes following a simulated gateway failure.

  Restore normal enterprise operations following completion of failover testing.

  Verify enterprise device configurations using Cisco IOS show commands.

  Apply enterprise change management procedures while maintaining network availability and resiliency.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Company Overview

✔ Enterprise Network Standards Guide

✔ Enterprise Network Addressing Plan

✔ Enterprise Topology (.pkt)

✔ Interface Addressing Plan

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

  Enterprise Network Topology

  Enterprise Network Standards Guide

  Enterprise IP Addressing Plan

  One (1) additional Cisco 2911 Router (HQ-R2)

  Two (2) Ethernet network cables

  This Enterprise Change Request (ECR-008)

## Planned Change Window

To minimize disruption to business operations, this Enterprise Change Request has been approved for implementation during the enterprise's scheduled maintenance window. During this time, temporary interruptions to network connectivity may occur as gateway addresses are updated and first-hop redundancy protocols are deployed.

All implementation, validation, and documentation activities should be completed before the maintenance window closes and normal business operations resume.

## Implementation Plan

### Phase 1 – Deploy HQ-R2

### Background

To improve enterprise resiliency, the Network Engineering Department has approved the deployment of a second headquarters router (HQ-R2). In addition to providing redundant gateway services for Phoenix Headquarters, HQ-R2 will become an active participant in the enterprise routing infrastructure. A new routed WAN connection will be established between HQ-R2 and Tucson Distribution Center (TUC-R2), allowing HQ-R2 to exchange routing information with the enterprise OSPF routing domain and maintain connectivity to remote enterprise locations.

☐ Deploy HQ-R2.

#### Step 1 – Install and Connect HQ-R2

At a minimum:

- Add one Cisco 2911 router to the enterprise topology.

- Configure the hostname as HQ-R2.

- Connect HQ-R2 GigabitEthernet0/0 to HQ-S2 GigabitEthernet0/1.

- Configure the basic router settings according to the Enterprise Network Standards Guide.

- Configure interface descriptions according to the Enterprise Network Standards Guide.

- Verify Layer 1 and Layer 2 connectivity.

- Save the device configuration.

- Update the enterprise topology diagram to include HQ-R2.

#### Step 2 – Configure the New Enterprise WAN Connection

At a minimum:

- Determine the next available IPv4 WAN subnet by reviewing the Enterprise IP Addressing Plan.

- Determine the next available IPv6 WAN subnet by reviewing the Enterprise IP Addressing Plan.

- Configure IPv4 addressing on the new WAN connection.

- Configure IPv6 global unicast addressing on the new WAN connection.

- Configure the static IPv6 link-local address FE80::4 on HQ-R2 GigabitEthernet0/1.

- Configure TUC-R2 GigabitEthernet0/1 using its established router link-local address, FE80::2.

- Verify IPv4 and IPv6 connectivity across the new WAN link.

- Save the device configuration.

#### Step 3 – Update the Enterprise Routing Domain

### Background

The addition of HQ-R2 introduces a new routed WAN connection into the enterprise network. To ensure that all enterprise routers can exchange routing information and maintain connectivity during normal operations and failover events, the enterprise OSPF routing domain must be updated. This requires configuring the new WAN connection on HQ-R2 and modifying the existing OSPF configuration on TUC-R2 so that both routers establish OSPFv2 and OSPFv3 neighbor relationships and advertise the new transit network. This change expands the existing enterprise OSPF routing domain without disrupting routing services for the remainder of the enterprise network.

### Configure HQ-R2

At a minimum:

- Configure OSPFv2 for the new WAN connection in Area 0.

- Configure OSPFv3 for the new WAN connection in Area 0.

- Advertise the new IPv4 WAN network.

- Enable OSPFv3 on the new WAN interface.

- Verify OSPFv2 neighbor relationships.

- Verify OSPFv3 neighbor relationships.

- Verify IPv4 and IPv6 routes are successfully learned from the enterprise routing domain.

### Update Configurations on TUC-R2

At a minimum:

- Update the existing OSPFv2 configuration to advertise the new HQ-R2 WAN network in Area 0.

- Enable OSPFv3 on the new WAN interface in Area 0.

- Verify OSPFv2 neighbor relationships with HQ-R2.

- Verify OSPFv3 neighbor relationships with HQ-R2.

- Verify IPv4 and IPv6 routes are successfully exchanged across the new WAN connection.

- Save the device configuration.

### Phase 2 – Update Enterprise Gateway Interface Addressing

### Background

Prior to deploying Gateway Load Balancing Protocol (GLBP), the IPv4 gateway addressing scheme at Phoenix Headquarters must be updated to support gateway redundancy. The current router interfaces use the first usable IPv4 address in each subnet as the default gateway. To prepare for GLBP deployment, the first usable IPv4 address will be reserved for the virtual gateway. HQ-R1 will be readdressed using the second usable IPv4 address for each production VLAN. These changes establish the enterprise addressing standard required for GLBP while maintaining the existing subnet design.

☐ Update the enterprise IPv4 gateway interface addressing.

At a minimum:

- Update each HQ-R1 production VLAN subinterface to use the second usable IPv4 address.

- Verify that all updated subinterfaces are operational.

- Verify IPv4 connectivity after updating the interface addresses.

- Troubleshoot and correct any configuration issues.

- Save the device configuration.

### Phase 3 – Configure Inter-VLAN Routing on HQ-R2

### Background

Before Gateway Load Balancing Protocol (GLBP) can provide gateway redundancy, HQ-R2 must be configured to perform the same inter-VLAN routing functions as HQ-R1 for the enterprise IPv4 network. Each Phoenix Headquarters production VLAN requires a corresponding router subinterface with a unique IPv4 address. Once configured, HQ-R2 will be prepared to participate in GLBP and provide redundant default gateway services for the enterprise IPv4 network.

☐ Configure inter-VLAN routing on HQ-R2 for the enterprise IPv4 network.

At a minimum:

  Configure Router-on-a-Stick for all Phoenix Headquarters production VLANs.

  Create one router subinterface for each production VLAN.

  Assign the third usable IPv4 address to each production VLAN subinterface.

  Configure IEEE 802.1Q encapsulation for each subinterface.

  Configure interface descriptions according to the Enterprise Network Standards Guide.

  Verify that all configured subinterfaces are operational.

  Verify IPv4 connectivity between HQ-R2 and the enterprise network.

  Troubleshoot and correct any configuration issues.

  Save the device configuration.

### Phase 4 – Deploy Gateway Load Balancing Protocol (GLBP)

Background

Gateway Load Balancing Protocol (GLBP) provides highly available default gateway services by allowing multiple routers to share forwarding responsibilities while presenting a single virtual gateway to end devices. As part of this Enterprise Change Request, GLBP will be deployed on HQ-R1 and HQ-R2 for all Phoenix Headquarters production VLANs. HQ-R1 will function as the preferred gateway through the use of priority and preemption, while HQ-R2 will provide gateway redundancy and participate in load balancing. Upon completion, enterprise hosts will use the GLBP virtual gateway as their default IPv4 gateway.

Enterprise GLBP Standards

| Setting | Enterprise Standard |
| --- | --- |
| GLBP Group Number | Match the associated VLAN ID |
| Virtual Gateway Address | First usable IPv4 address |
| HQ-R1 Priority | 150 |
| HQ-R2 Priority | 100 |
| Preemption | Enabled on priority router |

☐ Deploy Gateway Load Balancing Protocol (GLBP).

#### Step 1 – Configure GLBP on HQ-R1

☐ Configure GLBP on each Phoenix Headquarters production VLAN using the Enterprise GLBP Standards shown above.

Use the following command template, replacing the placeholders with the appropriate values for each production VLAN.

glbp <group-#> ip <First Usable IPv4 Address>
glbp <group-#> priority 150
glbp <group-#> preempt

### Example Configuration (VLAN 100)

interface GigabitEthernet0/0.100
 encapsulation dot1Q 10
 ip address 10.10.10.2 255.255.255.0

 glbp 100 ip 10.10.10.1
 glbp 100 priority 200
 glbp 100 preempt

Note:

Example Only: The configuration shown below demonstrates the GLBP command syntax. Use the enterprise standards defined above when configuring the production VLANs.

Repeat this configuration for each Phoenix Headquarters production VLAN.

Save the device configuration.

#### Step 2 – Configure GLBP on HQ-R2

☐ Configure GLBP on each Phoenix Headquarters production VLAN using the Enterprise GLBP Standards shown above.

Use the following command template, replacing the placeholders with the appropriate values for each production VLAN.

glbp <group#> ip <First Usable IPv4 Address>
glbp <group#> priority 100

Repeat this configuration for each Phoenix Headquarters production VLAN.

Save the device configuration.

#### Step 3 – Verify GLBP Operation

At a minimum:

- Verify GLBP has formed successfully on each production VLAN.

- Verify the GLBP group numbers match the VLAN IDs.

- Verify the virtual gateway addresses.

- Verify HQ-R1 is the Active Virtual Gateway (AVG).

- Verify the configured priorities.

- Verify preemption is enabled on the priority router.

- Troubleshoot and correct any configuration issues.

- Save the device configuration.

### Phase 5 – Configure IPv6 Gateway Redundancy Using HSRPv2

### Background

To provide gateway redundancy for the enterprise IPv6 network, Hot Standby Router Protocol Version 2 (HSRPv2) will be implemented on both headquarters routers. Unlike the IPv4 implementation, which uses GLBP to provide redundancy and load balancing, the IPv6 implementation will use HSRPv2 to provide a highly available default gateway for each Phoenix Headquarters production VLAN.

| Setting | Enterprise Standard |
| --- | --- |
| HSRP Version | Version 2 |
| HSRP Group Number | Match the associated VLAN ID |
| Virtual IPv6 Gateway Address | FE80::1 |
| HQ-R1 IPv6 Link-Local Address | FE80::2 |
| HQ-R2 IPv6 Link-Local Address | FE80::3 |
| HQ-R1 Priority | 150 |
| HQ-R2 Priority | 100 |
| Preemption | Enabled on priority router |

Use the following command template, replacing the placeholders with the appropriate values for each production VLAN.

standby version 2

standby <group#> ipv6 FE80::1
standby <group#> priority <priority>
standby <group#> preempt

### Example Configuration (VLAN 100)

interface GigabitEthernet0/0.100
 standby version 2
 standby 100 ipv6 FE80::1
 standby 100 priority 200
 standby 100 preempt

Note:

Example Only: The configuration shown below demonstrates the GLBP command syntax. Use the enterprise standards defined above when configuring the production VLANs.

☐ Configure IPv6 gateway redundancy using HSRPv2.

At a minimum:

- Configure HSRPv2 on each Phoenix Headquarters production VLAN.

- Configure the HSRP group number to match the associated VLAN ID.

- Configure the virtual IPv6 gateway address for each production VLAN.

- Configure HQ-R1 as the preferred active gateway using the enterprise priority.

- Configure preemption on the priority headquarters router.

- Configure HQ-R2 as the standby router.

- Verify HSRPv2 operation for each production VLAN.

- Troubleshoot and correct any configuration issues.

- Save the device configuration.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete command output or ping results.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Validate Normal Enterprise Connectivity

Verify that normal enterprise communications are functioning before implementing the gateway failover validation. Perform the following ping connectivity tests.

IPv4 Ping Connectivity

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Executive Administration | Tucson Warehouse & Inventory | ☐ | ☐ |
| Accounting & Finance | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Phoenix Information Technology | Tucson Warehouse & Inventory | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

IPv6 Ping Connectivity

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Executive Administration | Tucson Warehouse & Inventory | ☐ | ☐ |
| Accounting & Finance | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Phoenix Information Technology | Tucson Warehouse & Inventory | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

#### Step 2 – Record Baseline Routing

Run the following traceroute tests and copy the results into the space provided. These results establish the baseline routing path before simulating a gateway failure.

IPv4 Traceroute Connectivity

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service |  |
| Phoenix Information Technology | Tucson Warehouse & Inventory |  |

⚠ IMPORTANT:
Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify the implementation.

IPv6 Traceroute Connectivity

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service |  |
| Phoenix Information Technology | Tucson Warehouse & Inventory |  |

⚠ IMPORTANT:
Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering

#### Step 3 – Simulate a Gateway Failure

On HQ-R1, administratively shut down interface GigabitEthernet0/0 to simulate the loss of the primary headquarters gateway. Allow sufficient time (up to 3 minutes) for GLBP, HSRPv2, and OSPF to converge before continuing with the validation procedures.

#### Step 4 – Validate Enterprise Redundancy

Repeat the connectivity tests to verify that enterprise communications continue to operate following the simulated gateway failure.

IPv4 Ping Connectivity

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Executive Administration | Tucson Warehouse & Inventory | ☐ | ☐ |
| Accounting & Finance | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Phoenix Information Technology | Tucson Warehouse & Inventory | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

IPv6 Ping Connectivity

| Source Department | Destination Department | Success | Fail |
| --- | --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Executive Administration | Tucson Warehouse & Inventory | ☐ | ☐ |
| Accounting & Finance | Flagstaff Sales & Customer Service | ☐ | ☐ |
| Phoenix Information Technology | Tucson Warehouse & Inventory | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

Confirm Failover to Standby Router

Run the following traceroute tests and compare the results to the baseline collected in Step 2. Verify that the traceroute reflects the updated routing path through HQ-R2 following the simulated failure of HQ-R1 while maintaining successful connectivity to the destination.

IPv4 Traceroute Connectivity

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service |  |
| Phoenix Information Technology | Tucson Warehouse & Inventory |  |

⚠ IMPORTANT:
Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify the implementation.

IPv6 Traceroute Connectivity

| Source Department | Destination Department | Screenshot |
| --- | --- | --- |
| Phoenix Information Technology | Flagstaff Sales & Customer Service |  |
| Phoenix Information Technology | Tucson Warehouse & Inventory |  |

⚠ Important:
Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering

Note:

Compare the traceroute results to the baseline collected in Step 2 and document any differences in the routing path observed following the simulated gateway failure.

#### Step 5 – Restore Normal Enterprise Operations

Re-enable interface GigabitEthernet0/0 on HQ-R1.

Verify that:

- Enterprise connectivity remains operational.

- HQ-R1 resumes its role as the preferred IPv4 gateway.

- HQ-R1 resumes its role as the active IPv6 gateway.

- No routing or gateway redundancy issues remain before proceeding to the protocol verification steps.

#### Step 6 – Verify First-Hop Redundancy Protocol Operation

Run the following commands on the appropriate headquarters routers.

### IPv4 Gateway Redundancy (GLBP)

show glbp

Highlight the following:

- GLBP Group Number

- Virtual IPv4 Gateway Address

- Router Priority

- Active Virtual Gateway (AVG)

- Active Virtual Forwarder (AVF)

- Current Router State

| Device | Required Output - Paste show glbp |
| --- | --- |
| HQ-R1 |  |
| HQ-R2 |  |

### IPv6 Gateway Redundancy (HSRPv2)

show standby

Highlight the following:

- HSRP Group Number

- Virtual IPv6 Gateway

- Active Router

- Standby Router

- Router Priority

- Preemption (if displayed)

- Router State

| Device | Required Output - Paste show standby |
| --- | --- |
| HQ-R1 |  |
| HQ-R2 |  |

#### Step 7 – Verify Enterprise OSPF Operation

Run the following commands on HQ-R2.

Highlight the following:

- Neighbor Router ID

- Neighbor State

- Neighbor IP Address

- Interface

This verifies that HQ-R2 has successfully joined the enterprise OSPF routing domain and established neighbor adjacencies.

### OSPFv2

show ip ospf neighbor

| Device | Required Output - Paste show ip ospf neighbor |
| --- | --- |
| HQ-R2 |  |
| TUC-R2 |  |

### OSPFv3

show ipv6 ospf neighbor

| Device | Required Output - Paste show ipv6 ospf neighbor |
| --- | --- |
| HQ-R2 |  |
| TUC-R2 |  |

#### Step 8 – Verify Final Device Configurations

Run the following command on:

- HQ-R1

- HQ-R2

- TUC-R2

show running-config

Highlight the following on the appropriate devices:

### HQ-R1

- Updated IPv4 gateway interface addresses

- Router-on-a-Stick subinterfaces

- GLBP configuration

- HSRPv2 configuration

- OSPFv2 configuration

- OSPFv3 configuration

### HQ-R2

- Router hostname

- WAN interface configuration

- Router-on-a-Stick subinterfaces

- IPv4 and IPv6 interface addressing

- GLBP configuration

- HSRPv2 configuration

- OSPFv2 configuration

- OSPFv3 configuration

### TUC-R2

- New WAN interface configuration

- IPv4 and IPv6 addressing

- OSPFv2 configuration

- OSPFv3 configuration

Important:

Before submitting your show running-config output, highlight the required configuration items identified in this Enterprise Change Request. For ECR, highlight the interface descriptions on all active interfaces.

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| HQ-R2 |  |
| TUC-R2 |  |

#### Step 9 – Update Enterprise Documentation

### Background

Enterprise documentation must accurately reflect the production network before an Enterprise Change Request can be closed. Following successful implementation and validation of this Enterprise Change Request, update the enterprise documentation to reflect the approved changes. These updates ensure future network engineers have accurate information for ongoing operations, troubleshooting, and future enterprise expansions.

☐ Update the enterprise documentation.

### At a minimum:

#### Enterprise Network Standards Guide

- Add the HQ-R2 hardware information.

- Update the router interface standards.

- Document the new HQ-R2 to TUC-R2 WAN connection.

- Add the Enterprise GLBP Standards.

- Add the Enterprise HSRPv2 Standards.

- Update any additional standards affected by this Enterprise Change Request.

#### Enterprise IP Addressing Plan

- Document the HQ-R2 interface addressing.

- Document the new HQ-R2 to TUC-R2 WAN network.

- Update the Phoenix Headquarters gateway addressing.

- Document the GLBP virtual IPv4 gateway addresses.

- Document the HSRPv2 virtual IPv6 gateway addresses.

- Update any additional addressing information affected by this Enterprise Change Request.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Network Standards Guide

☐  Updated Enterprise IP Addressing Plan

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 07/10/2026