ECR-010

## Enterprise Internet Redundancy

| Field | Scope |
| --- | --- |
| Requested by | Network Engineering Department |
| Assigned to | Junior Network Engineer |
| Priority | High |
| Change type | Normal |
| Implementation Window | Estimated Implementation Effort: 2–4 hours See established due date |
| Business Impact | Medium |

## Business Justification

Following the successful deployment of enterprise Internet access, Sonoran Outdoor Equipment has become increasingly dependent on cloud-based applications, vendor portals, software updates, online ordering systems, and other Internet-based business services. Although the current enterprise network provides reliable Internet connectivity through a single Internet Service Provider (ISP), the organization remains vulnerable to a complete Internet outage if the ISP connection or headquarters Internet-edge router becomes unavailable.

To improve business continuity and increase enterprise resilience, the Network Engineering Department has approved the implementation of a secondary Internet Service Provider (ISP) connection at Phoenix Headquarters. The secondary ISP will provide an alternate path to the Internet should the primary ISP connection become unavailable. During normal operations, the enterprise will continue to use the primary ISP connection. If the primary Internet path fails, the enterprise will automatically transition to the secondary ISP connection, minimizing service disruption while maintaining Internet access for authorized users.

As part of this Enterprise Change Request (ECR), a second ISP router will be connected to HQ-R2 using a dedicated public IPv4 network. A floating static default route will be implemented to provide automatic failover while ensuring the primary Internet connection remains the preferred path during normal operations. Port Address Translation (PAT) will also be configured on HQ-R2 to support Internet access through the secondary ISP when failover occurs.

Following implementation, Sonoran Outdoor Equipment will benefit from a dual-homed Internet connection that improves network availability, reduces the impact of ISP outages, and supports continued access to critical Internet-based business services. This design follows enterprise networking best practices for organizations that depend on reliable and highly available Internet connectivity.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

  Deploy a secondary Internet Service Provider (ISP) connection to improve enterprise Internet redundancy.

  Configure and verify a floating static default route to provide automatic Internet failover.

  Configure OSPFv2 to advertise a default route only when a valid default route exists.

  Configure and verify Port Address Translation (PAT) using interface overload on the secondary Internet-edge router.

  Configure NAT inside and NAT outside interfaces according to enterprise design requirements.

  Create and apply a standard IPv4 ACL to identify enterprise networks eligible for Port Address Translation.

  Verify that enterprise security policies remain in effect during normal operations and after Internet failover.

  Validate enterprise Internet connectivity before and after a simulated ISP failure.

  Analyze and compare traceroute results to verify that Internet traffic automatically transitions to the secondary ISP connection.

  Verify active PAT translations and interpret NAT translation tables and statistics.

  Use Cisco IOS verification commands to validate routing, PAT operation, and enterprise Internet-edge configurations.

  Troubleshoot and resolve connectivity or routing issues that prevent successful Internet failover.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Company Overview

✔ Enterprise Network Standards Guide

✔ Enterprise Network Addressing Plan

✔ Enterprise Topology

✔ Interface Addressing Plan

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

  Enterprise Network Topology

  Enterprise Network Standards Guide

  Enterprise IP Addressing Plan

  This Enterprise Change Request (ECR-010)

## Implementation Plan

### Phase 1 – Deploy the Secondary ISP Connection

### Background

Sonoran Outdoor Equipment currently relies on a single Internet Service Provider connected through HQ-R1. Although the existing connection supports enterprise Internet access, the loss of ISP-1 or the HQ-R1 Internet-facing link would interrupt access to cloud applications, vendor services, software updates, and other Internet-based business resources.

To improve Internet availability, the Network Engineering Department has approved a secondary connection through ISP-2. ISP-2 has been preconfigured and tested by the service provider. As part of this Enterprise Change Request, HQ-R2 will be connected to ISP-2 and prepared to function as the enterprise backup Internet-edge router.

☐ Deploy the secondary ISP connection.

#### Step 1 – Connect HQ-R2 to ISP-2

At a minimum:

Locate the preconfigured ISP-2 router included in the enterprise topology.

Connect HQ-R2 GigabitEthernet0/2 to ISP-2 GigabitEthernet0/0.

Configure the HQ-R2 interface description according to the Enterprise Network Standards Guide.

Verify Layer 1 and Layer 2 connectivity.

Save the device configuration.

#### Step 2 – Configure the Secondary Public WAN Connection

The secondary ISP connection uses the public IPv4 network 203.0.113.4/30.

| Device | Interface | IPv4 Address | Subnet Mask |
| --- | --- | --- | --- |
| ISP-2 | G0/0 | 203.0.113.5 | 255.255.255.252 |
| HQ-R2 | G0/2 | 203.0.113.6 | 255.255.255.252 |

At a minimum:

- Verify that ISP-2 GigabitEthernet0/0 is configured with 203.0.113.5/30.

- Configure HQ-R2 GigabitEthernet0/2 with 203.0.113.6/30.

- Enable the HQ-R2 interface.

- Verify IPv4 connectivity between HQ-R2 and ISP-2.

- Troubleshoot and correct any connectivity issues before proceeding.

- Save the device configuration.

### Phase 2 – Configure Backup Internet Routing

### Background

During normal enterprise operations, HQ-R2 learns the primary default route through the OSPFv2 routing domain from HQ-R1. The primary Internet connection should remain preferred while it is available.

A floating static default route will be configured on HQ-R2 with an administrative distance greater than the OSPF administrative distance. While the primary OSPF default route is available, the floating static route will remain inactive. If the primary default route is withdrawn, the floating static route will be installed and HQ-R2 will advertise the backup default route throughout the enterprise OSPFv2 routing domain.

ISP-1 and ISP-2 must not participate in the enterprise OSPF routing domain.

☐ Configure the enterprise backup Internet route.

#### Step 1 – Configure the Floating Static Default Route

At a minimum:

- Configure an IPv4 floating static default route on HQ-R2.

- Use 203.0.113.5 as the ISP-2 next-hop address.

- Configure the route with an administrative distance of 120.

- Verify that the existing OSPF-learned default route remains preferred during normal operations.

- Verify that the floating static route does not replace the primary default route while the primary route is available.

- Save the device configuration.

#### Step 2 – Configure Backup Default Route Advertisement

At a minimum:

- Configure HQ-R2 to advertise a default route through the enterprise OSPFv2 routing domain when a valid default route is installed.

- Do not use an option that forces HQ-R2 to advertise a default route when no valid default route exists.

- Verify that HQ-R1 remains the source of the preferred enterprise default route during normal operations.

- Verify that HQ-R2 is prepared to advertise the backup default route if the primary route is withdrawn.

- Save the device configuration.

### Phase 3 – Configure Port Address Translation on HQ-R2

### Background

When enterprise traffic is redirected to ISP-2, HQ-R2 must translate private enterprise IPv4 addresses before forwarding the traffic to the public ISP network. PAT will be configured on HQ-R2 using the public IPv4 address assigned to GigabitEthernet0/2.

The PAT configuration on HQ-R2 must identify all approved enterprise networks while preserving the security policies established in previous Enterprise Change Requests. The NAT classification ACL identifies traffic eligible for translation; it does not replace or override existing security ACLs.

☐ Configure Port Address Translation on HQ-R2.

#### Step 1 – Configure the NAT Inside Interfaces

At a minimum:

- Identify each HQ-R2 Layer 3 interface or subinterface that receives traffic from the private enterprise network.

- Configure each applicable interface or subinterface as a NAT inside interface.

- Confirm that physical interfaces are not used in place of their routed subinterfaces.

- Verify that all affected interfaces remain operational.

#### Step 2 – Configure the NAT Outside Interface

At a minimum:

- Configure the ISP-2-facing interface on HQ-R2 as the NAT outside interface.

- Verify that the interface remains operational.

- Verify connectivity between HQ-R2 and ISP-2.

#### Step 3 – Identify Traffic Eligible for Translation

At a minimum:

- Configure the named standard IPv4 ACL NAT-INSIDE on HQ-R2.

- Include all approved Phoenix Headquarters production networks.

- Include the Tucson Distribution Center network.

- Include the Flagstaff Sales Office network.

- Ensure the ACL identifies the same approved enterprise networks used by the existing PAT implementation on HQ-R1.

- Verify the ACL entries before proceeding.

#### Step 4 – Configure PAT Interface Overload

At a minimum:

- Configure PAT on HQ-R2 using the public IPv4 address assigned to GigabitEthernet0/2.

- Use the named standard IPv4 ACL NAT-INSIDE to identify eligible enterprise traffic.

- Configure PAT using interface overload.

- Verify that the configuration references the correct ACL, outside interface, and overload option.

- Save the device configuration.

### Phase 4 – Confirm Secondary Internet-Edge Readiness

### Background

Before conducting the formal failover validation, confirm that the secondary ISP connection, backup route, and PAT configuration are complete. Initial verification reduces the risk of disrupting enterprise operations during the simulated failure of the primary Internet connection.

☐ Confirm that the secondary Internet edge is prepared for failover.

At a minimum:

- Verify that the HQ-R2-to-ISP-2 connection is operational.

- Verify that HQ-R2 can communicate with ISP-2.

- Verify that HQ-R2 continues to use the OSPF-learned primary default route during normal operations.

- Verify that the floating static default route is configured but not installed while the primary route remains available.

- Verify the NAT inside and NAT outside interface designations on HQ-R2.

- Verify the NAT-INSIDE ACL on HQ-R2.

- Verify the PAT interface overload configuration.

- Confirm that existing enterprise Internet access through HQ-R1 remains operational.

- Troubleshoot and correct any configuration issues before beginning the Enterprise Verification & Validation procedures.

- Save the device configuration.

The formal validation can then prove the full sequence: normal Internet access through ISP-1, simulated primary-link failure, installation and advertisement of HQ-R2’s floating route, PAT translations through ISP-2, and restoration of the preferred ISP-1 path.

### Phase 5 – Prepare the Enterprise Internet Edge

### Background

HQ-R1 currently serves as the enterprise Autonomous System Boundary Router (ASBR) and provides the only connection between the Sonoran Outdoor Equipment private network and the Internet Service Provider. The enterprise default route has already been configured and distributed through the OSPF routing domain. Before implementing Port Address Translation (PAT), verify that the existing Internet-edge routing configuration remains operational and that all enterprise locations can forward external traffic toward HQ-R1.

☐ Prepare the enterprise Internet edge for PAT deployment.

At a minimum:

Verify that the connection between HQ-R1 GigabitEthernet0/1 and the ISP router is operational.

Verify that HQ-R1 can communicate with the ISP router.

Verify that the enterprise IPv4 default route is installed on HQ-R1.

Verify that the default route continues to be advertised throughout the enterprise OSPFv2 routing domain.

Verify that Phoenix Headquarters, Tucson Distribution Center, and Flagstaff Sales Office have routes directing unknown IPv4 traffic toward HQ-R1.

Troubleshoot and correct any routing or ISP connectivity issues before proceeding.

Save the device configuration.

### Phase 6 – Identify Enterprise NAT Interfaces and Traffic

### Background

Port Address Translation requires the enterprise edge router to distinguish between traffic originating from the private enterprise network and traffic traveling toward the public ISP network. Because HQ-R1 receives internal traffic through multiple routed interfaces and subinterfaces, each applicable enterprise-facing Layer 3 interface must be identified as an inside NAT interface. The ISP-facing interface will be identified as the outside NAT interface.

A standard IPv4 Access Control List will identify the approved private enterprise networks eligible for translation. This ACL is used only to classify traffic for PAT and does not function as an enterprise traffic-filtering security policy.

☐ Identify the enterprise NAT interfaces and approved inside traffic.

#### Step 1 – Configure the NAT Inside Interfaces

☐ Configure the appropriate HQ-R1 interfaces as NAT inside interfaces:

At a minimum:

Apply the NAT inside designation to each applicable Layer 3 interface or subinterface.

Confirm that the physical GigabitEthernet0/0 interface is not used in place of its routed subinterfaces.

Verify that all affected interfaces remain operational.

Save the device configuration.

#### Step 2 – Configure the NAT Outside Interface

☐ Configure the appropriate HQ-R1 interface as the enterprise NAT outside interface.

At a minimum:

Apply the NAT outside designation to the ISP-facing interface.

Verify that the interface remains operational.

Verify connectivity between HQ-R1 and the ISP router.

#### Step 3 – Identify Enterprise Traffic Eligible for PAT

☐ Configure the named standard IPv4 ACL NAT-INSIDE to identify the approved private IPv4 enterprise networks that require Internet access.

At a minimum:

Include all currently implemented Phoenix Headquarters production networks.

Include the Tucson Distribution Center network.

Include the Flagstaff Sales Office network.

Ensure that only approved internal enterprise IPv4 networks are included.

Verify the ACL entries before proceeding.

Save the device configuration.

### Phase 7 – Deploy Port Address Translation

### Background

Port Address Translation allows multiple private enterprise devices to share the public IPv4 address assigned to HQ-R1’s ISP-facing interface. When internal traffic exits the enterprise, HQ-R1 translates the private source address and assigns a unique Layer 4 port number to each session. This conserves public IPv4 address space while allowing simultaneous Internet access for users throughout the enterprise.

☐ Deploy Port Address Translation on HQ-R1.

### At a minimum:

Configure PAT on HQ-R1 using the public IPv4 address assigned to GigabitEthernet0/1.

Use the named standard IPv4 ACL NAT-INSIDE to identify traffic eligible for translation.

Configure PAT using interface overload.

Verify that the PAT configuration references the correct:

Inside traffic ACL

ISP-facing interface

Overload option

Generate Internet-bound traffic from multiple enterprise departments to create active PAT translations.

Verify that translations are created for Phoenix Headquarters, Tucson Distribution Center, and Flagstaff Sales Office clients.

Troubleshoot and correct any PAT configuration issues.

Save the device configuration.

### Phase 8 – Confirm Enterprise Internet Access

### Background

Following the PAT deployment, verify that enterprise clients can access approved external resources while maintaining internal enterprise connectivity. Initial testing during the implementation confirms that the configuration is functioning before the formal validation package is completed.

☐ Confirm enterprise Internet access through PAT.

### At a minimum:

Verify IPv4 connectivity from Phoenix Headquarters to the ISP network.

Verify IPv4 connectivity from Flagstaff Sales Office to the ISP network.

Verify that Internet access from the Tucson Distribution Center remains restricted according to the Enterprise Security Policy.

Verify that multiple enterprise clients can establish simultaneous translated sessions.

Verify that internal communication between enterprise locations remains operational.

Confirm that all connectivity and security requirements are functioning as expected before beginning the Enterprise Verification & Validation procedures.

Save the device configuration.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete output for all required verification commands.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Validate Enterprise Internet Connectivity

### Background

Before simulating an ISP failure, establish a baseline of normal enterprise Internet connectivity. These tests confirm that the enterprise is currently using the primary Internet connection while all previously implemented enterprise services continue to operate normally.

☐ Validate normal enterprise Internet operations.

At a minimum:

- Perform the required IPv4 connectivity tests using the validation table below.

- Record the results of each test using the Success or Fail checkboxes.

- Verify that Phoenix Headquarters and Flagstaff Sales Office have Internet connectivity through the primary ISP.

- Verify that Internet access from the Tucson Warehouse & Distribution Center continues to be restricted according to the Enterprise Security Policy.

- Troubleshoot and resolve any unexpected results before continuing.

IPv4 Ping Connectivity

| Source Department | Destination | Success | Fail |
| --- | --- | --- | --- |
| Phoenix Information Technology | ISP | ☐ | ☐ |
| Phoenix Human Resources | ISP | ☐ | ☐ |
| Phoenix Sales | ISP | ☐ | ☐ |
| Flagstaff Sales | ISP | ☐ | ☐ |
| Tucson Warehouse | ISP | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

#### Step 2 – Record Baseline Traceroute Results

#### Background

Traceroute identifies the path that packets take through the enterprise network to reach their destination. Before simulating the loss of the primary ISP connection, record the baseline traceroute results. These results will be used later to verify that Internet traffic automatically transitions to the secondary ISP connection during failover.

☐ Record the baseline traceroute results.

At a minimum:

- Run the required traceroute tests listed below.

- Analyze the path taken by each packet as it travels from the source department to the destination.

- Verify that Internet traffic is currently exiting the enterprise through the primary Internet connection.

- Copy and paste the complete traceroute results into the tables provided.

- Compare these baseline results with the traceroute results collected after the simulated ISP failure in Step 4 to verify successful Internet failover.

IPv4 Traceroute Connectivity

| Source Department | Destination | Screenshot |
| --- | --- | --- |
| Phoenix Information Tech | ISP |  |
| Flagstaff Sales | ISP |  |
| Tucson Warehouse | ISP |  |

Important:

Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify the implementation.

#### Step 3 – Simulate Primary ISP Failure

### Background

Enterprise Internet redundancy must continue to provide Internet access when the primary ISP connection becomes unavailable. Simulate the loss of the primary Internet connection to verify that HQ-R2 automatically assumes responsibility for Internet connectivity using the secondary ISP.

☐ Simulate the loss of the primary Internet connection.

### At a minimum:

- Disable the primary ISP connection by shutting down HQ-R1 GigabitEthernet0/1.

- Allow sufficient time for the enterprise routing tables to converge.

- Verify that HQ-R2 installs the floating static default route.

- Verify that HQ-R2 begins forwarding Internet-bound traffic through ISP-2.

- Do not proceed until failover has completed successfully.

#### Step 4 – Validate Automatic Internet Failover

### Background

Following the simulated failure, verify that enterprise users continue to access Internet resources through the secondary ISP connection.

☐ Validate automatic Internet failover.

### At a minimum:

Repeat each IPv4 connectivity test performed in Step 1.

Record the results using the Success or Fail checkboxes.

Verify that Phoenix Headquarters and Flagstaff Sales Office continue to have Internet connectivity.

Verify that the Tucson Warehouse & Distribution Center remains restricted by the Enterprise Security Policy.

Confirm that enterprise Internet access was restored without manual routing changes.

IPv4 Ping Connectivity

| Source Department | Destination | Success | Fail |
| --- | --- | --- | --- |
| Phoenix Information Technology | ISP | ☐ | ☐ |
| Phoenix Human Resources | ISP | ☐ | ☐ |
| Phoenix Sales | ISP | ☐ | ☐ |
| Flagstaff Sales | ISP | ☐ | ☐ |
| Tucson Warehouse | ISP | ☐ | ☐ |

Important:

If any test fails, troubleshoot and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

#### Step 5 – Analyze the Enterprise Routing Path After Failover

### Background

Following the successful failover to the secondary ISP connection, analyze the updated routing path to verify that Internet traffic is no longer using the primary ISP. Comparing the traceroute results to the baseline collected before the simulated failure confirms that enterprise traffic automatically transitioned to the secondary Internet connection without requiring manual intervention.

☐ Analyze the enterprise routing path after failover.

### At a minimum:

Run the same traceroute tests completed in Step 2.

Copy and paste the complete traceroute results into the tables provided.

Compare the new traceroute results with the baseline traceroute collected in Step 2.

Verify that Internet traffic is now exiting the enterprise through HQ-R2 and ISP-2.

Verify that the routing path differs from the baseline only as expected following the simulated ISP failure.

Troubleshoot and resolve any unexpected routing behavior before proceeding.

IPv4 Traceroute Connectivity

| Source Department | Destination | Screenshot |
| --- | --- | --- |
| Phoenix Information Tech | ISP |  |
| Flagstaff Sales | ISP |  |
| Tucson Warehouse | ISP |  |

Important:

Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify the implementation.

#### Step 6 - Verify Active Port Address Translation (PAT) Sessions

#### Background

Port Address Translation (PAT) dynamically creates translation entries only while network traffic is actively flowing between the enterprise and external networks. Once communication stops, idle translation entries are automatically removed after a period of inactivity. As a result, the NAT translation table may be empty if the verification commands are executed too long after the connectivity tests have been completed.

To observe active PAT translations, perform the following verification immediately after completing the traceroute tests while Internet traffic is still flowing through the secondary ISP connection.

☐ Verify active PAT translations on HQ-R2.

Run the following command on HQ-R2:

show ip nat translations

Highlight the following:

Inside Local Address

Inside Global Address

Outside Global Address

Protocol

Active port translations

Important:

Run this command immediately after completing the Internet connectivity and traceroute tests. If the translation table is empty, generate additional Internet traffic by repeating a ping or traceroute test, and then run the command again before the PAT entries time out.

| Device | Required Output - Paste show ip nat translations |
| --- | --- |
| HQ-R2 |  |

#### Step 7 – Restore Primary Internet Service

### Background

After successful failover testing, restore the primary ISP connection and verify that normal enterprise operations resume.

☐ Restore the primary Internet connection.

### At a minimum:

Re-enable HQ-R1 GigabitEthernet0/1.

Allow sufficient time for routing convergence.

Verify that the primary default route once again becomes the preferred enterprise Internet path.

Confirm that enterprise Internet traffic is once again using ISP-1.

Perform several IPv4 connectivity tests to verify normal operations.

#### Step 8 – Verify Enterprise Routing to ISP

### Background

Following successful failover and restoration, verify that enterprise routing services are functioning correctly on both Internet-edge routers.

☐ Verify enterprise routing on HQ-R1.

### HQ-R1

Run the following command.

show ip route

Highlight the following:

Gateway of Last Resort

Default route

Connected public network

| Device | Required Output - Paste show ip route |
| --- | --- |
| HQ-R1 |  |

#### Step 9 – Verify Enterprise Routing and Port Address Translation

### Background

Following successful failover and restoration, verify that enterprise routing and NAT/PAT services are functioning correctly on both Internet-edge routers.

☐ Verify enterprise routing and PAT operation.

### HQ-R2

Run the following command.

show ip route

Highlight the following:

Floating static default route

OSPF default route

Gateway of Last Resort

Connected ISP-2 network

| Device | Required Output - Paste show ip route |
| --- | --- |
| HQ-R2 |  |

Run the following command.

show ip nat statistics

Highlight the following:

Total active translations

Hits

Misses

Inside interfaces

Outside interfaces

| Device | Required Output - Paste show ip nat statistics |
| --- | --- |
| HQ-R2 |  |

#### Step 10 – Verify Final Device Configurations

### Background

Review the final enterprise Internet-edge configuration to verify that the secondary ISP deployment has been successfully implemented according to enterprise standards.

☐ Verify the final device configurations on HQ-R2.

### HQ-R2

Run the following command:

show running-config

Highlight the following:

Secondary ISP interface configuration

Floating static default route

PAT configuration

NAT inside interfaces

NAT outside interface

OSPF default route advertisement

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R2 |  |

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 07/15/2026