ECR-012

## Enterprise Network Monitoring & Management Services

| Field | Scope |
| --- | --- |
| Requested by | Network Engineering Department |
| Assigned to | Junior Network Engineer |
| Priority | Low |
| Change type | Normal |
| Implementation Window | Estimated Implementation Effort: 1–3 hours See established due date |
| Business Impact | Low |

## Business Justification

As Sonoran Outdoor Equipment has expanded into a multi-site enterprise with redundant gateways, multiple WAN connections, centralized Internet access, and a resilient backup WAN architecture, the Network Operations Center (NOC) requires additional management services to efficiently monitor, troubleshoot, and maintain the enterprise network. Although the enterprise infrastructure is now highly available, effective day-to-day operations depend on accurate network documentation, consistent system time, centralized event logging, device monitoring, and reliable configuration management.

To improve enterprise operations, the Network Engineering Department has approved the deployment of several core network management services across the enterprise. Network Time Protocol (NTP) will be implemented to synchronize the system clocks on all network devices, ensuring that log messages, security events, and configuration changes share a consistent and accurate timestamp. Centralized Syslog services will be configured to collect and record system events, providing network engineers with a historical record of operational and security-related messages for troubleshooting and auditing purposes.

Simple Network Management Protocol (SNMP) will be introduced to support enterprise monitoring by allowing authorized network management systems to collect operational information from network devices. Although Sonoran Outdoor Equipment does not currently utilize a dedicated network management server, the enterprise network will be configured to support centralized monitoring as the organization continues to grow.

Cisco Discovery Protocol (CDP) and Link Layer Discovery Protocol (LLDP) will be used to verify physical network connectivity and assist with network documentation. During implementation, CDP will be temporarily enabled to validate neighboring Cisco devices and document interface connections. Because CDP advertises infrastructure information that could be useful to an attacker, it will be disabled after the required documentation has been completed. LLDP will remain available as the enterprise-standard discovery protocol to support ongoing network management and interoperability with multi-vendor equipment.

As part of this Enterprise Change Request (ECR), the Junior Network Engineer will configure and verify enterprise network management services, perform device discovery and documentation, implement centralized time synchronization and event logging, configure basic SNMP monitoring, and verify that all device configurations are properly backed up. Upon completion, Sonoran Outdoor Equipment will have a standardized network management framework that improves operational visibility, simplifies troubleshooting, supports disaster recovery and provides a scalable foundation for future enterprise growth.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

Configure and verify Network Time Protocol (NTP) clients to synchronize enterprise network devices with an authoritative time source.

Deploy and verify centralized Syslog services to support enterprise event logging and troubleshooting.

Configure and verify Simple Network Management Protocol (SNMPv2c) for centralized network monitoring.

Secure SNMP access using standard access control lists (ACLs).

Configure and verify Link Layer Discovery Protocol (LLDP) for ongoing enterprise device discovery.

Temporarily enable Cisco Discovery Protocol (CDP) to document the physical enterprise topology and disable it after documentation has been completed.

Connect and configure a network management server according to established Enterprise Network Standards.

Verify enterprise network management services using Cisco IOS show commands and network management tools.

Perform enterprise device discovery and document neighboring network devices and interface connections.

Validate enterprise network management services through systematic implementation verification and final configuration audits.

Maintain accurate enterprise network documentation throughout the implementation process.

Apply enterprise change management procedures while maintaining network availability and operational consistency.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Company Overview

✔ Enterprise Network Standards Guide

✔ Enterprise Network Addressing Plan

✔ Enterprise Topology

✔ Interface Addressing Plan

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

Enterprise Network Topology

Enterprise Network Standards Guide

Enterprise IP Addressing Plan

This Enterprise Change Request (ECR-012)

## Implementation Plan

### Phase 1 – Deploy Enterprise Time Synchronization

### Background

Accurate time synchronization is essential for enterprise network operations. System logs, security events, configuration changes, and troubleshooting activities all rely on consistent timestamps across network devices. Without synchronized clocks, it becomes difficult to determine the order in which events occurred.

Sonoran Outdoor Equipment receives an authoritative Network Time Protocol (NTP) service from ISP-1. Enterprise network engineers typically deploy time synchronization before other management services so that logs, monitoring events, and configuration changes share a common timestamp from the beginning of the implementation. As part of this Enterprise Change Request, all enterprise routers and switches will be configured to synchronize their system clocks with the ISP-1 time source. This ensures that every network device maintains a consistent and accurate time throughout the enterprise.

The ntp server command designates the device from which the router or switch will synchronize its system clock. Once configured, the device automatically operates as an NTP client.

#### Step 1 – Configure Enterprise NTP Clients

☐ Configure the enterprise routers and switches to synchronize with the authoritative NTP server.

Use the following command template to configure each enterprise router and switch to use ISP-1 as its Network Time Protocol (NTP) server.

ntp server 203.0.113.5

At a minimum:

Configure HQ-R1 to use the ISP-1 NTP server.

Configure HQ-R2 to use the ISP-1 NTP server.

Configure TUC-R2 to use the ISP-1 NTP server.

Configure FLG-R3 to use the ISP-1 NTP server.

Configure all enterprise switches to use the ISP-1 NTP server.

Save the device configurations.

#### Step 2 – Verify Enterprise Time Synchronization

### Background

After configuring the NTP clients, verify that each device is successfully synchronizing with the authoritative time source. NTP synchronization may require a short period of time before the devices establish communication with the server and update their system clocks.

☐ Verify enterprise time synchronization.

Use the following Cisco IOS commands to verify NTP operation.

show ntp associations

show clock

Complete the verification table below after reviewing the command output from each enterprise router and switch. Verify that each enterprise router is synchronized with the ISP-1 NTP server.

Phoenix Headquarters

| Verification | HQ-R1 | HQ-S1 | HQ-S2 | HQ-R2 |
| --- | --- | --- | --- | --- |
| Synchronized with ISP-1 NTP Server | ☐ | ☐ | ☐ | ☐ |
| System Clock Correct | ☐ | ☐ | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ | ☐ | ☐ |

Tucson Warehouse & Distribution

| Verification | TUC-R2 | TUC-S3 |
| --- | --- | --- |
| Synchronized with ISP-1 NTP Server | ☐ | ☐ |
| System Clock Correct | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ |

Flagstaff Customer Service & Sales

| Verification | FLG-R3 | FLG-S4 |
| --- | --- | --- |
| Synchronized with ISP-1 NTP Server | ☐ | ☐ |
| System Clock Correct | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ |

If any device does not successfully synchronize with the ISP-1 NTP server, troubleshoot and resolve the issue before proceeding to the next phase.

After all devices have been successfully synchronized, save the device configurations.

### Phase 2 – Deploy the Enterprise Network Management Server

#### Background

The Network Operations Center (NOC) has deployed a new Headquarters Network Mangement Server (HQ-NMS) to provide centralized network management services for the enterprise. The server has already been configured with the approved IPv4 and IPv6 addressing information. As the Junior Network Engineer, your responsibility is to physically connect the server to the enterprise network and configure the appropriate switch access port according to the Enterprise Network Standards.

#### Step 1 – Configure the Enterprise Access Port

☐ Configure the switch interface for the Network Management Server.

At a minimum:

Connect HQ-NMS to HQ-S2 FastEthernet0/20 using the appropriate Ethernet cable.

Verify Layer 1 connectivity after the cable has been connected.

Confirm that the server is connected to the correct switch and interface according to enterprise standards.

#### Step 2 – Configure the Enterprise Access Port

The Network Management Server is a permanent enterprise infrastructure device located within the Phoenix Information Technology Department.

☐ Configure the switch interface for the Network Management Server.

At a minimum:

Configure FastEthernet0/20 on HQ-S2 as an access port.

Assign the interface to the Information Technology VLAN.

Configure the interface description according to the Enterprise Network Standards.

Enable the interface if necessary.

Verify that the interface is operational.

Save the device configuration.

### Phase 3 – Configure Enterprise Syslog Services

### Background

Centralized Syslog allows network devices to forward system messages to a central logging server. Instead of reviewing log messages individually on each router and switch, network administrators can monitor and archive events from across the enterprise in one location. This improves troubleshooting, auditing, and security monitoring.

☐ Configure centralized Syslog logging.

#### Step 1 – Configure Centralized Syslog

The HQ-NMS server has already been deployed and configured to receive Syslog messages. Configure each enterprise network device to forward its system log messages to the centralized Syslog server.

Use the following command template:

logging host 10.10.10.5

logging trap informational

At a minimum:

Configure HQ-R1 to forward Syslog messages to HQ-NMS.

Configure HQ-R2 to forward Syslog messages to HQ-NMS.

Configure TUC-R2 to forward Syslog messages to HQ-NMS.

Configure FLG-R3 to forward Syslog messages to HQ-NMS.

Configure all enterprise switches to forward Syslog messages to HQ-NMS.

Save the device configurations.

#### Step 2 – Verify Enterprise Syslog Services

### Background

After configuring centralized Syslog, verify that the enterprise routers are successfully forwarding system log messages to HQ-NMS. Network engineers commonly generate a few non-disruptive events to confirm that log messages are being received by the centralized Syslog server before placing the network back into normal operation.

☐ Verify centralized Syslog operation.

##### Part A – Verify the Syslog Configuration

Use the following Cisco IOS command on HQ-R1 and TUC-R2.

show logging

Verify the following:

The remote Syslog server address is configured correctly.

Remote logging is enabled.

The logging severity level is configured as expected.

No Syslog configuration errors are reported.

| Verification | HQ-R1 | HQ-R2 | TUC-R2 | FLG-R3 |
| --- | --- | --- | --- | --- |
| Remote Syslog Server Configured | ☐ | ☐ | ☐ | ☐ |
| Remote Logging Enabled | ☐ | ☐ | ☐ | ☐ |
| Logging Severity Level Correct | ☐ | ☐ | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ | ☐ | ☐ |

##### Part B – Generate Enterprise Log Events

Generating a small number of controlled events is a common validation technique used to confirm that centralized logging is functioning before returning the network to production.

Generate several non-disruptive network events to verify that the centralized Syslog server is receiving log messages from multiple enterprise locations.

HQ-R1

Generate one or more of the following events:

Enter and exit global configuration mode.

Save the running configuration.

Shut down and restore an interface.

TUC-R2

Generate one or more of the following events:

Enter and exit global configuration mode.

Save the running configuration.

Shut down and restore the loopback interface.

FLG-R3

Generate one or more of the following events:

Enter and exit global configuration mode.

Save the running configuration.

Shut down and restore the loopback interface.

##### Part C – Verify Centralized Logging

Open the HQ-NMS server and verify that Syslog messages from the enterprise routers have been successfully received.

Confirm that the following information is visible:

The device that generated each event.

The event timestamp.

The Syslog message severity.

A description of the event.

Verify that log messages from HQ-R1, HQ-R2, TUC-R2, and FLG-R3 are present before proceeding to the next phase.

| Verification | HQ-R1 | HQ-R2 | TUC-R2 | FLG-R3 |
| --- | --- | --- | --- | --- |
| Log Messages Received | ☐ | ☐ | ☐ | ☐ |
| Device Name Identified | ☐ | ☐ | ☐ | ☐ |
| Timestamp Present | ☐ | ☐ | ☐ | ☐ |
| Severity Displayed | ☐ | ☐ | ☐ | ☐ |
| Event Description Displayed | ☐ | ☐ | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ | ☐ | ☐ |

### Phase 4 – Deploy Enterprise Network Monitoring (SNMP)

### Background

As enterprise networks grow, network administrators require a centralized method for monitoring the operational status of network devices. Simple Network Management Protocol (SNMP) allows authorized management systems to collect information from routers and switches without requiring an administrator to log into each device individually.

Network devices that provide operational information are called SNMP agents. An authorized management workstation, referred to as an SNMP manager, periodically polls these agents to collect information about device health, interface status, processor utilization, memory usage, and other operational statistics.

As part of this Enterprise Change Request, the Network Operations Center (NOC) has deployed a dedicated network management workstation, HQ-NOC1, within the Phoenix Information Technology Department. The workstation will be used to monitor selected enterprise routers and swtiches using SNMP Version 2c.

SNMPv2c uses community strings for authentication but does not encrypt management traffic. Many modern enterprises deploy SNMPv3 for production environments because it provides authentication and encryption.

To improve security, Sonoran Outdoor Equipment restricts SNMP access to authorized management devices located within the Phoenix Information Technology subnet. A named standard ACL is used to identify the approved management network and is applied to the SNMP community string. This prevents unauthorized devices from polling enterprise network equipment.

#### Step 1 – Deploy the Network Operations Center Workstation

☐ Install the HQ-NOC1 management workstation.

The HQ-NOC1 workstation has been preconfigured with the approved enterprise IPv4 and IPv6 addressing information. As the Junior Network Engineer, your responsibility is to connect the workstation to the enterprise network and configure the appropriate switch access port according to the Enterprise Network Standards.

At a minimum:

Connect HQ-NOC1 to HQ-S2 FastEthernet0/21 using the appropriate Ethernet cable.

Configure FastEthernet0/21 as an access port.

Assign the interface to the Phoenix Information Technology VLAN.

Configure the interface description according to the Enterprise Network Standards.

Verify Layer 1 and Layer 2 connectivity.

Confirm that the workstation receives an IPv4 address from DHCP.

Verify IPv4 and IPv6 connectivity to the enterprise network.

Save the device configuration.

#### Step 2 – Configure Enterprise SNMP Agents

☐ Configure SNMP Version 2c on the Phoenix enterprise infrastructure devices.

Use the provided Cisco IOS command template to configure the listed enterprise routers and switches as SNMP agents.

HQ-R1

HQ-R2

HQ-S1

HQ-S2

Configure the commands:

ip access-list standard SNMP_ACL

permit 10.10.10.0 0.0.0.127

exit

!

snmp-server community SOEpublic RO SNMP_ACL

snmp-server contact "Network Operations Center"

snmp-server location "Phoenix Headquarters"

### At a minimum:

Create the named standard ACL SNMP_ACL.

Permit the Phoenix Information Technology subnet (10.10.10.0/25).

Configure the SOEpublic read-only community string.

Apply the named ACL to the community string.

Configure the SNMP contact information.

Configure the SNMP location information.

Save the device configurations.

#### Step 3 – Verify Enterprise SNMP Operation

### Background

After configuring the SNMP agents, verify that the Network Operations Center workstation can successfully communicate with the enterprise routers and switches. Rather than connecting directly to each router and switch, network administrators commonly use an SNMP management application to retrieve operational information from multiple devices through a centralized interface.

Before testing the management workstation, use Cisco IOS verification commands to confirm that the SNMP configuration has been successfully applied to each device.

☐ Verify SNMP configuration.

##### Part A – Verify the Cisco IOS Configuration

Use the following Cisco IOS command on each configured SNMP device.

show snmp

Verify the following:

- SNMP is enabled.

- The configured contact information is displayed.

- The configured location information is displayed.

- No SNMP configuration errors are reported.

Phoenix Infrastructure Devices

| Verification | HQ-R1 | HQ-S1 | HQ-S2 | HQ-R2 |
| --- | --- | --- | --- | --- |
| SNMP Enabled | ☐ | ☐ | ☐ | ☐ |
| Contact Information Displayed | ☐ | ☐ | ☐ | ☐ |
| Location Information Displayed | ☐ | ☐ | ☐ | ☐ |
| No Configuration Errors Reported | ☐ | ☐ | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ | ☐ | ☐ |

If any device does not display the expected SNMP configuration, troubleshoot and resolve the issue before proceeding to Part B.

After all devices have been successfully verified, save the device configurations.

##### Part B – Verify SNMP Communication

Using HQ-NOC1, open the MIB Browser and verify successful communication with each of the following devices:

- HQ-R1

- HQ-R2

- HQ-S1

- HQ-S2

### Verify the following:

- The device responds to SNMP queries.

- The correct community string is accepted.

- Device information can be successfully retrieved.

- All four Phoenix infrastructure devices respond successfully to the management workstation.

| Verification | HQ-R1 | HQ-S1 | HQ-S2 | HQ-R2 |
| --- | --- | --- | --- | --- |
| Device Responds to SNMP Queries | ☐ | ☐ | ☐ | ☐ |
| Community String Accepted | ☐ | ☐ | ☐ | ☐ |
| Device Information Retrieved Succesfully | ☐ | ☐ | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ | ☐ | ☐ |

If any device does not respond to SNMP queries or the expected information cannot be retrieved, troubleshoot and resolve the issue before proceeding to the next phase.

After all Phoenix infrastructure devices have been successfully verified, save the device configurations.

### Phase 5 – Deploy Enterprise Link Layer Discovery Protocol (LLDP)

### Background

As enterprise networks continue to grow, network administrators require a standardized method for discovering neighboring network devices regardless of vendor. Link Layer Discovery Protocol (LLDP) is an IEEE open-standard Layer 2 discovery protocol that allows directly connected network devices to exchange information about their identity, capabilities, management addresses, and interface connections.

Unlike Cisco Discovery Protocol (CDP), which is Cisco proprietary, LLDP supports interoperability between equipment from multiple network vendors. Sonoran Outdoor Equipment has adopted LLDP as the enterprise-standard discovery protocol to support future network expansion and vendor interoperability.

As the Junior Network Engineer, you will configure and verify LLDP on the Phoenix enterprise infrastructure. Once deployed, LLDP will remain enabled as part of the enterprise network management services.

#### Step 1 – Configure Enterprise LLDP

☐ Configure Link Layer Discovery Protocol (LLDP) on the Phoenix enterprise infrastructure.

Use the following Cisco IOS command template to enable LLDP.

lldp run

Configure LLDP on:

HQ-R1

HQ-R2

HQ-S1

HQ-S2

TUC-R2

TUC-S3

FLG-R3

FLG-S4

At a minimum:

Enable LLDP globally on each Phoenix infrastructure device.

Verify that LLDP is enabled before proceeding.

Save the device configurations.

#### Step 2 – Verify Enterprise LLDP Operation

### Background

After enabling LLDP, verify that neighboring devices are exchanging discovery information. LLDP automatically advertises device identity, interface information, and management information between directly connected network devices. Compare the discovered neighbor information with the Enterprise Topology to verify that the physical network matches the approved design.

☐ Verify enterprise LLDP operation.

Use the following Cisco IOS commands on HQ-R1 and HQ-S1.

show lldp neighbors

show lldp neighbors detail

### Verify the following:

LLDP is operational.

Neighboring devices are discovered successfully.

Local and remote interfaces are displayed correctly.

Neighbor device information is displayed.

Management address information is available.

No LLDP configuration errors are present.

Verify successful LLDP operation before proceeding to the Cisco Discovery Protocol (CDP) deployment phase.

### Phoenix Headquarters

| Verification | HQ-R1 | HQ-S1 | HQ-S2 | HQ-R2 |
| --- | --- | --- | --- | --- |
| Neighbor Devices Discovered | ☐ | ☐ | ☐ | ☐ |
| Local and Remote Interfaces Displayed Correctly | ☐ | ☐ | ☐ | ☐ |
| Management Address Information Available | ☐ | ☐ | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ | ☐ | ☐ |

### Tucson Warehouse & Distribution

| Verification | TUC-R2 | TUC-S3 |
| --- | --- | --- |
| Neighbor Devices Discovered | ☐ | ☐ |
| Local and Remote Interfaces Displayed Correctly | ☐ | ☐ |
| Management Address Information Available | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ |

### Flagstaff Sales & Customer Service Headquarters

| Verification | FLG-R3 | FLG-S4 |
| --- | --- | --- |
| Neighbor Devices Discovered | ☐ | ☐ |
| Local and Remote Interfaces Displayed Correctly | ☐ | ☐ |
| Management Address Information Available | ☐ | ☐ |
| Needs Troubleshooting | ☐ | ☐ |

If any device does not successfully discover neighboring devices or display the expected LLDP information, troubleshoot and resolve the issue before proceeding to the Cisco Discovery Protocol (CDP) deployment phase.

After all enterprise routers and switches have been successfully verified, save the device configurations.

### Phase 6 – Document the Enterprise Using Cisco Discovery Protocol (CDP)

### Background

Although Sonoran Outdoor Equipment has standardized on Link Layer Discovery Protocol (LLDP) for ongoing network discovery, Cisco Discovery Protocol (CDP) remains a valuable troubleshooting and documentation tool within Cisco-only environments. During network deployments, engineers often use CDP to quickly identify neighboring Cisco devices, verify physical connections, and document interface assignments.

Because CDP advertises information about the enterprise infrastructure, including device identities, interface connections, software versions, and management addresses, it may also provide useful information to an attacker. For this reason, Sonoran Outdoor Equipment only enables CDP when needed for deployment, troubleshooting, or documentation activities. After the required documentation has been completed, CDP will be disabled in accordance with enterprise security standards.

As the Junior Network Engineer, you have been asked to temporarily enable CDP, document the physical enterprise topology, and submit the completed neighbor documentation to the Network Operations Center (NOC).

#### Step 1 – Temporarily Enable Cisco Discovery Protocol (CDP)

☐ Temporarily enable Cisco Discovery Protocol (CDP) on the Phoenix enterprise infrastructure.

Use the following Cisco IOS command template to enable CDP.

cdp run

Configure CDP on:

HQ-R1

HQ-R2

HQ-S1

HQ-S2

At a minimum:

Enable CDP globally on each Phoenix infrastructure device.

Verify that CDP is operational before proceeding.

Save the device configurations.

#### Step 2 – Document the Enterprise Topology

### Background

Cisco Discovery Protocol automatically discovers directly connected Cisco devices and reports valuable operational information that assists network engineers during deployment and troubleshooting. Before returning the enterprise to normal operation, the Network Operations Center requires an updated record of the physical network topology.

☐ Use Cisco Discovery Protocol to document the enterprise network.

Use the following Cisco IOS commands on HQ-R1 and HQ-S1.

show cdp neighbors

show cdp neighbors detail

Complete the Enterprise Neighbor Documentation Table using the information discovered through CDP.

Document the following information for each neighboring device:

Local device

Local interface

Neighbor device

Neighbor interface

Management IP address

Device platform

Review the completed documentation for accuracy before submitting it to the Network Operations Center.

### Enterprise Neighbor Documentation Table

Instructions: Use the output from show cdp neighbors and show cdp neighbors detail to document the neighboring Cisco devices discovered during the deployment. Complete one row for each directly connected neighbor.

| Local Device | Local Interface | Neighbor Device | Neighbor Interface | Manage-ment IP Address | Device Platform |
| --- | --- | --- | --- | --- | --- |

Review the completed Enterprise Neighbor Documentation Table for accuracy before proceeding to the next step.

Network Engineer Certification: I certify that the information recorded above accurately reflects the neighboring Cisco devices discovered during the implementation of this Enterprise Change Request.

Network Engineer Initials: _____________

#### Step 3 – Disable Cisco Discovery Protocol (CDP)

### Background

The required enterprise documentation has now been completed. Although CDP is an effective troubleshooting tool, it also advertises information about the enterprise infrastructure that could be useful to unauthorized users. As part of Sonoran Outdoor Equipment's security standards, CDP is disabled after deployment and documentation activities have been completed.

☐ Disable Cisco Discovery Protocol (CDP).

Use the following Cisco IOS command template.

no cdp run

### At a minimum:

Disable CDP on each Phoenix infrastructure device.

Verify that CDP has been disabled.

Save the device configurations.

## Enterprise Verification & Validation

After successfully completing the implementation, perform a final configuration audit of the enterprise network. Move methodically through each enterprise router and switch, reviewing the active configuration to verify that all required network management services have been implemented according to the Enterprise Network Standards. This systematic approach reflects the workflow commonly used by network engineers when validating a completed change before closing an Enterprise Change Request.

For each device listed below, run the required Cisco IOS command and paste the output into the corresponding table. Highlight the requested configuration items to demonstrate that the required management services have been successfully deployed and documented.

#### Step 1 – Verify Final Device Configurations

### Background

The running configuration provides a complete record of the active device configuration and serves as the final validation that the required network management services have been successfully implemented. Before closing this Enterprise Change Request, review the running configuration on each enterprise router and switch to confirm that all required configuration elements are present and comply with the Enterprise Network Standards.

☐ Verify the final device configurations for the enterprise network.

Run the following command on each device:

show running-config

Highlight only the configuration items that apply to the device being verified.

NTP server configuration

Syslog host configuration

Logging trap level

Named standard ACL SNMP_ACL

SNMP community string

SNMP contact information

SNMP location information

LLDP configuration (lldp run)

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| HQ-R2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Network Standards Guide

☐  Updated Enterprise IP Addressing Plan

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 07/19/2026