Enterprise IP Addressing Plan

## Introduction

Before any network devices can be configured, the enterprise network addressing plan must be completed. A well-designed addressing plan provides the foundation for routing, security, network management, scalability, and future growth.

As a Junior Network Engineer for Sonoran Outdoor Equipment, your responsibility is to develop an efficient IPv4 and IPv6 addressing plan that supports the organization's current business operations while reserving space for future expansion.

Using the enterprise requirements provided in this document, develop an efficient Variable Length Subnet Mask (VLSM) addressing plan for IPv4 and an organized hierarchical addressing plan for IPv6. Your completed addressing plan will serve as the foundation for all Enterprise Change Requests (ECRs) and will be used throughout the semester to build, expand, and maintain the enterprise network.

After completing your addressing plan in this document, enter your answers into the corresponding Canvas quiz. A score of 100% on the Canvas quiz is required before continuing to the next phase of the enterprise implementation process.

Important:

The enterprise networks in this document are organized by business location, not by subnet size. Before assigning IPv4 addresses, analyze the host requirements and determine the appropriate VLSM allocation order. For IPv6, follow the enterprise subnetting standards defined in this document. The completed addressing plan should efficiently support the current enterprise while reserving sufficient address space for the future expansion sites identified in this document. Careful planning of both IPv4 and IPv6 addressing is an essential responsibility of a network engineer.

## Enterprise Address Space

Sonoran Outdoor Equipment has been assigned the following enterprise address space:

### IPv4 Private Address Space

- 10.10.0.0/16

Develop an efficient VLSM addressing plan using only addresses from this private IPv4 address block.

### IPv6 Global Unicast Address Space

- 2001:DB8:C001::/48

Develop an enterprise IPv6 addressing plan using the subnetting standards defined in this document.

### IPv6 Enterprise Subnetting Standards

Use the following standards when developing the enterprise IPv6 addressing plan:

- The enterprise has been assigned the global unicast prefix 2001:DB8:C001::/48.

- Assign one unique /64 IPv6 subnet to every enterprise VLAN, routed LAN, loopback network, and WAN network.

Enterprise LAN and loopback networks

- Use the third hexadecimal digit (nibble) of the fourth hextet for enterprise LAN and loopback networks.

- Assign LAN and loopback subnet IDs in the same order used for the IPv4 VLSM design, from largest host requirement to smallest host requirement.

- Phoenix Headquarters networks begin with 0010 and continue sequentially by nibble.

- Tucson Distribution Center and Tubac future expansion networks begin with 00A0 and continue sequentially by nibble.

- Flagstaff Sales Office and Greer future expansion networks begin with 00E0 and continue sequentially by nibble.

Enterprise WAN networks

- Use the fourth hexadecimal digit (nibble) of the fourth hextet for point-to-point WAN networks.

- WAN subnet IDs begin with 0001 and continue sequentially.

- Example WAN subnet IDs include 0001, 0002, and 0003.

The public IPv4 address space used for Internet connectivity will be introduced later during the Network Address Translation (NAT) Enterprise Change Request.

### Part 1 – Enterprise Network Addressing Plan

## Enterprise Network Addressing Requirements

The following tables identify the current network requirements for Sonoran Outdoor Equipment. Review the business requirements and develop an efficient enterprise IP addressing plan that supports the organization's current operations while allowing for future growth.

For each enterprise network, determine:

- The appropriate IPv4 network address and prefix length using Variable Length Subnet Masking (VLSM).

- The appropriate IPv6 network prefix using the enterprise IPv6 addressing standards defined in this document.

The completed addressing plan will serve as the foundation for the Enterprise Interface Addressing Plan and all future Enterprise Change Requests (ECRs).

### Phoenix Headquarters

| Department / Network | VLAN | Hosts |
| --- | --- | --- |
| Executive | 10 | 25 |
| Human Resources | 20 | 50 |
| Accounting & Finance | 30 | 75 |
| Sales & Customer Service | 40 | 350 |
| Information Technology | 50 | 100 |
| IoT & Security Systems | 70 | 500 |
| Voice | 80 | 300 |
| Guest Wireless | 90 | 150 |
| Network Management | 99 | 25 |

### Tucson Distribution Center

| Department / Network | VLAN | Hosts |
| --- | --- | --- |
| Sales & Customer Service | 40 | 225 |
| Information Technology | 50 | 50 |
| Warehouse & Inventory | 60 | 200 |

### Flagstaff Sales Office

| Department / Network | VLAN | Hosts |
| --- | --- | --- |
| Sales & Customer Service | 40 | 125 |

### Reserved for Future Expansion

| Site | Hosts |
| --- | --- |
| Tubac Adventure Lodge | 90 |
| Greer Mountain Cabins | 120 |

### Enterprise WAN Links

| Connection | Hosts |
| --- | --- |
| HQ-R1 ↔ TUC-R2 | 2 |
| HQ-R1 ↔ FLG-R3 | 2 |
| TUC-R2 ↔ FLG-R3 | 2 |

## Enterprise Network Addressing Plan

Complete the table below by developing an efficient enterprise IP addressing plan based on the requirements provided in this document. For each enterprise network, determine the appropriate IPv4 network address and prefix length using VLSM, and assign the corresponding IPv6 network prefix according to the enterprise IPv6 addressing standards.

Future Expansion Clarification:

Tubac Adventure Lodge and Greer Mountain Cabins are future expansion sites. During the initial enterprise deployment, these future networks will be represented by loopback interfaces on existing enterprise routers. Tubac Adventure Lodge will be represented on TUC-R2, and Greer Mountain Cabins will be represented on FLG-R3.

| Future Site | Associated Router and Interface |
| --- | --- |
| Tubac Adventure Lodge | TUC-R2 Loopback 10 |
| Greer Mountain Cabins | FLG-R3 Loopback 10 |

Note:

Complete your calculations in this document before entering your answers into Canvas. You may organize your work however you choose, but your final addressing plan should reflect an efficient IPv4 VLSM design and a well-organized IPv6 addressing plan.

The completed Enterprise Network Addressing Plan will serve as the foundation for the Enterprise Interface Addressing Plan and all future Enterprise Change Requests (ECRs).

### Enterprise Network Addressing Plan

| Site | Department / Network | VLAN ID | Hosts | IPv4 Network / Prefix | IPv6 Network / Prefix |
| --- | --- | --- | --- | --- | --- |

## Version 1: Updated 06/28/2026

### Part 2 – Enterprise Interface Addressing Plan

The Enterprise Network Addressing Plan completed in Part 1 defines the IPv4 and IPv6 networks assigned to each enterprise department and service.

In this part of the document, you will assign IPv4 and IPv6 addresses to enterprise router interfaces, switch management interfaces (SVIs), WAN interfaces, loopback interfaces, and end devices. These interface assignments will be referenced throughout the semester when implementing Enterprise Change Requests (ECRs).

## Enterprise Interface Addressing Standards

Use the following enterprise standards when assigning IPv4 and IPv6 addresses to router interfaces, switch management interfaces, WAN links, loopback interfaces, and end devices.

### Router Interface Addressing Standards

- Assign the first usable IPv4 and IPv6 address in every VLAN and routed LAN network to the router interface that serves as the default gateway for that network.

- Assign point-to-point WAN interface addresses sequentially.

  - The Phoenix Headquarters router (HQ-R1) uses the first usable address on all WAN links connected to Headquarters.

  - The router at the remote site uses the second usable address on WAN links connected to Headquarters.

  - On the Tucson-to-Flagstaff WAN link, TUC-R2 uses the first usable address, and FLG-R3 uses the second usable address.

- Assign the first usable address in each reserved future expansion network to the router loopback interface representing that future site.

### Enterprise Interface Addressing Standards

### IPv6 Link-Local Addressing Standards

Assign static IPv6 link-local addresses to all active router interfaces and subinterfaces using the following enterprise standards:

- Configure a manually assigned IPv6 link-local address on every active router interface and subinterface.

- Derive the link-local address from the associated VLAN ID whenever applicable.

- Use the format FE80::VLAN-ID.

- Examples:

  - VLAN 10 → FE80::10

  - VLAN 20 → FE80::20

  - VLAN 40 → FE80::40

  - VLAN 50 → FE80::50

  - VLAN 70 → FE80::70

  - VLAN 80 → FE80::80

  - VLAN 90 → FE80::90

  - VLAN 99 → FE80::99

- For point-to-point WAN interfaces, assign static IPv6 link-local addresses based on the router:

- HQ-R1: FE80::1

- TUC-R2: FE80::2

- FLG-R3: FE80::3

Link-local addresses must be unique on the local link. The same router link-local address may be reused on different WAN links when the links are separate point-to-point networks.

### Switch Management Interface Addressing Standards

- Assign the Phoenix Headquarters VLAN 99 management addresses as follows:

  - HQ-S1: Second usable address

  - HQ-S2: Third usable address

- Assign remote site switch management addresses as follows:

  - IPv4: Use the last usable IPv4 address in the assigned VLAN 1 network.

  - IPv6: Use the 10th IPv6 address in the assigned VLAN 1 network.

### End Device Addressing Standards

- Assign statically configured end devices using consecutive addresses beginning with the second usable address in the assigned network.

  - First end device: Second usable address

  - Second end device: Third usable address

  - Third end device: Fourth usable address

  - Continue assigning addresses sequentially as additional end devices are deployed.

Important:

All initial end-device addressing will be configured statically. Dynamic address assignment is implemented through Enterprise DHCP and Stateless DHCPv6 services according to the Enterprise Network Standards Guide in a future Enterprise Change Request (ECR).

### Enterprise DNS Standards

The Sonoran Outdoor Equipment enterprise has deployed a centralized Domain Name System (DNS) server at Phoenix Headquarters to provide name resolution services for both IPv4 and IPv6 networks. The DNS server has been preconfigured and is considered an existing enterprise infrastructure service.

All enterprise DHCP and DHCPv6 implementations shall distribute the enterprise DNS server addresses to client devices according to the Enterprise IP Addressing Plan.

| Server | Location | IPv4 Address | IPv6 Address |
| --- | --- | --- | --- |
| HQ-DNS1 | Phoenix Headquarters | 10.10.10.9 | 2001:DB8:C001:50::9/64 |

### Default Gateway Standards

- For router interfaces, enter N/A in the Default Gateway column.

- For switch management interfaces (SVIs) and end devices, enter the IPv4 and IPv6 address of the router interface that serves as the default gateway for that network.

## Enterprise Interface Addressing Plan

Complete the tables below by assigning IPv4 and IPv6 addresses to the enterprise router interfaces, switch management interfaces, WAN interfaces, loopback interfaces, and end devices according to the Enterprise Interface Addressing Standards defined in this document.

Note:

Complete your interface addressing assignments in this document before entering your answers into Canvas. You may organize your work however you choose, but your completed interface addressing plan should follow the Enterprise Interface Addressing Standards and accurately reflect the IPv4 and IPv6 network assignments developed in Part 1 – Enterprise Network Addressing Plan.

The completed Enterprise Interface Addressing Plan will be referenced throughout the semester when configuring devices as part of each Enterprise Change Request (ECR).

### Completed Interface Addressing Example

The following example demonstrates the expected format for documenting interface addresses. The following example demonstrates only how to complete the table. The example does not use the Sonoran Outdoor Equipment enterprise network. Use the assigned IPv4 and IPv6 addressing information from this document when completing your own tables.

Example:

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |
| G0/1.110 | Clinic | 192.168.0.1/26 | N/A | 2001:BED:FAD:20::1/64 | N/A |
| CC-PC01 | Clinic PC | 192.168.0.2/26 | 192.168.0.1/26 | 2001:BED:FAD:20::2/64 | 2001:BED:FAD:20::1/64 |

Note:

The router interface receives the first usable address and serves as the default gateway for the network. End devices are assigned consecutive usable addresses beginning with the second usable address.

### Enterprise Interface Addressing Plan

#### Phoenix Headquarters

#### HQ-R1 – Phoenix Headquarters Router

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |
| G0/0 | Physical trunk interface to HQ-S1 | N/A | N/A | N/A | N/A |

#### HQ-S1 – Phoenix Headquarters Switch 1

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### HQ-S2 – Phoenix Headquarters Switch 2

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### Phoenix Headquarters End Devices

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### Tucson Distribution Center

#### TUC-R2

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### TUC-S3

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### Tucson End Devices

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### Flagstaff Sales Office

#### FLG-R3

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### FLG-S4

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |

#### Flagstaff End Devices

| Interface | Purpose | IPv4 Address / Prefix | IPv4 Default Gateway | IPv6 Address / Prefix | IPv6 Default Gateway |
| --- | --- | --- | --- | --- | --- |