ECR-011

## Enterprise WAN Redundancy with GRE Tunnel

| Field | Scope |
| --- | --- |
| Requested by | Network Engineering Department |
| Assigned to | Junior Network Engineer |
| Priority | Low |
| Change type | Normal |
| Implementation Window | Estimated Implementation Effort: 1–3 hours See established due date |
| Business Impact | Low |

## Business Justification

As Sonoran Outdoor Equipment continues to expand, uninterrupted communication between Phoenix Headquarters and the Tucson Distribution Center has become increasingly critical to daily business operations. The Tucson Distribution Center relies on continuous access to enterprise resources, including inventory management systems, shipping applications, employee records, centralized file storage, and other business services hosted throughout the enterprise network. A failure of the existing WAN connection would isolate the distribution center and significantly disrupt warehouse operations, order fulfillment, and internal communications.

To improve enterprise resiliency, the Network Engineering Department has approved the deployment of a secondary WAN path using Generic Routing Encapsulation (GRE). A new public ISP connection will be installed at the Tucson Distribution Center to provide an alternate transport path across the Internet. Unlike the primary enterprise WAN, this ISP connection is not intended to provide direct Internet access for Tucson users. Instead, it will serve exclusively as the public network used to transport GRE tunnel traffic between Tucson and Phoenix Headquarters.

During normal operations, all enterprise traffic will continue to use the existing private WAN connection between Phoenix Headquarters and the Tucson Distribution Center. If the primary WAN connection becomes unavailable, enterprise routing will automatically transition to the GRE tunnel, allowing the Tucson Distribution Center to maintain connectivity with critical enterprise resources. Internet access for authorized users will remain centralized through Phoenix Headquarters, where existing enterprise security policies, Port Address Translation (PAT), and Internet access controls continue to be enforced.

As part of this Enterprise Change Request (ECR), a public WAN connection will be established between the Tucson Distribution Center and ISP-R2. A GRE tunnel will then be configured between Phoenix Headquarters and Tucson using the public ISP infrastructure as the transport network. OSPFv2 routing information will be exchanged across the GRE tunnel to provide an automatic backup path while maintaining the existing private WAN connection as the preferred route during normal operations.

Following implementation, Sonoran Outdoor Equipment will benefit from a resilient dual-path WAN architecture that improves business continuity, reduces the impact of WAN outages, and ensures the Tucson Distribution Center can continue accessing essential enterprise resources without requiring a separate branch Internet deployment.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

Explain the business purpose of deploying a GRE tunnel as a backup WAN solution.

Configure public IPv4 transport connectivity between an enterprise router and an ISP.

Configure directly connected host static routes to reach GRE tunnel endpoints.

Plan and implement dual-stack IPv4 and IPv6 GRE overlay addressing.

Configure a dual-stack GRE tunnel using Cisco IOS.

Integrate the GRE tunnel into the enterprise IPv4 routing domain using OSPFv2.

Analyze OSPF interface costs to determine path preference between the primary private WAN and GRE backup path.

Verify GRE tunnel operation using Cisco IOS show commands.

Validate enterprise connectivity before, during, and after a simulated WAN failure.

Analyze traceroute output to identify routing path changes during GRE failover.

Verify automatic OSPF convergence and restoration of the preferred enterprise WAN path.

Update enterprise network documentation to reflect the new WAN design and GRE implementation.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Company Overview

✔ Enterprise Network Standards Guide

✔ Enterprise Network Addressing Plan

✔ Enterprise Topology

✔ Interface Addressing Plan

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

Enterprise Network Topology

Enterprise Network Standards Guide

Enterprise IP Addressing Plan

This Enterprise Change Request (ECR-011)

## Implementation Plan

### Phase 1 – Deploy the Tucson ISP Transport Connection

### Background

The GRE backup tunnel requires public IPv4 connectivity between the tunnel endpoints at Phoenix Headquarters and the Tucson Distribution Center. ISP-R2 will provide the public transport network used to carry the encapsulated GRE traffic. This connection is not intended to provide direct Internet access to Tucson users and will not replace the centralized Internet access and security services provided through Phoenix Headquarters.

☐ Deploy the Tucson ISP transport connection.

#### Step 1 – Connect TUC-R2 to ISP-R2

☐ Connect TUC-R2 GigabitEthernet0/2 to ISP-R2 GigabitEthernet0/2.

#### Step 2 – Configure the Public IPv4 Transport Network

The Tucson ISP transport connection uses the public IPv4 network 203.0.113.12/30.

| Device | Interface | IPv4 Address | Subnet Mask |
| --- | --- | --- | --- |
| ISP-R2 | G0/2 | 203.0.113.13 | 255.255.255.252 |
| TUC-R2 | G0/2 | 203.0.113.14 | 255.255.255.252 |

At a minimum:

Verify that ISP-R2 GigabitEthernet0/2 is configured with 203.0.113.13/30.

Configure TUC-R2 GigabitEthernet0/2 with 203.0.113.14/30.

Enable the TUC-R2 GigabitEthernet0/2 interface.

Configure the TUC-R2 interface description according to the Enterprise Network Standards Guide.

Verify IPv4 connectivity between TUC-R2 and ISP-R2 using the directly connected public IPv4 addresses.

Confirm that the Tucson ISP connection is not configured as a NAT outside interface.

Confirm that Tucson users are not provided direct Internet access through this connection. This connection provides the public IPv4 underlay required for the GRE backup path.

Save the device configuration.

### Phase 2 – Establish GRE Underlay Reachability

#### Background

Before a GRE tunnel can be established, each tunnel endpoint must be able to reach the remote public IPv4 address through the ISP transport network. One efficient method of accomplishing this is by configuring a host static route, which creates a route to a single destination rather than an entire network. This approach ensures that only the remote GRE endpoint is reachable through the public ISP connection while maintaining the existing enterprise routing design. Using host routes instead of broader network routes minimizes the amount of public routing introduced into the enterprise and limits the ISP transport connection to only the required GRE endpoints.

☐ Configure static routes to the GRE tunnel endpoints.

#### Step 1 – Configure Static Routes to the GRE Tunnel Endpoints

Use the following command template, replacing the placeholders with the appropriate addressing and next hop IPv4 address.

ip route <Remote Public IPv4 Address> 255.255.255.255 <next IP address>

Example

HQ-R1:

ip route 203.0.113.14 255.255.255.255 203.0.113.1

TUC-R2:

ip route 203.0.113.2 255.255.255.255 203.0.113.13

At a minimum:

- Configure a host static route on HQ-R1 to reach the public IPv4 address assigned to TUC-R2.
Configure a host static route on TUC-R2 to reach the public IPv4 address assigned to HQ-R1.

- Use the appropriate next-hop IPv4 address. Verify that each router can successfully reach the remote GRE tunnel endpoint.

- Troubleshoot and resolve any connectivity issues before continuing.

- Save the device configurations.

#### Step 2 – Verify GRE Underlay Connectivity

### Background

Before configuring the GRE tunnel, verify that the public tunnel endpoints can successfully communicate across the ISP transport network. GRE cannot establish a tunnel if the tunnel source and destination are unable to reach one another. Confirming underlay connectivity before configuring Tunnel0 simplifies troubleshooting and helps isolate any addressing or routing issues.

☐ Verify GRE underlay connectivity.

| Source Device | Destination IPv4 Address | Success | Fail |
| --- | --- | --- | --- |
| HQ-R1 | 203.0.113.14 | ☐ | ☐ |
| TUC-R2 | 203.0.113.2 | ☐ | ☐ |

Both connectivity tests must be successful before proceeding.

If either test fails, troubleshoot and correct the underlay connectivity issue before configuring the GRE tunnel. At a minimum, verify the public IPv4 interface addressing, interface status, host static routes, next-hop reachability, and ISP transport routing.

Important:

Do not proceed to GRE tunnel configuration until both tunnel endpoints have successful bidirectional IPv4 connectivity. GRE depends on the underlying IPv4 transport network. If the tunnel endpoints cannot communicate before the tunnel is configured, the GRE tunnel will not operate correctly.

### Phase 3 – Plan the GRE Overlay Addressing

### Background

Although Tunnel0 is a virtual interface, it functions as a routed point-to-point connection between HQ-R1 and TUC-R2. Just like two physical router interfaces connected by a private WAN link, the two Tunnel0 interfaces must share their own private network.

The GRE tunnel addressing is separate from the public IPv4 addresses used to transport the tunnel across the ISP infrastructure. The public addresses form the underlay, while the private IPv4 and IPv6 addresses assigned to Tunnel0 form the overlay used by the enterprise network.

In this phase, review the current Enterprise IP Addressing Plan and determine the next available private IPv4 and IPv6 networks for the GRE tunnel. Assign an address from each network to the Tunnel0 virtual interfaces on HQ-R1 and TUC-R2.

☐ Plan the dual-stack GRE overlay addressing.

### At a minimum:

- Review the updated Enterprise IP Addressing Plan and identify the next available private IPv4 /30 network for the GRE Tunnel0 point-to-point connection.

- Use the next available enterprise private WAN network, 10.10.12.80/30, for the GRE IPv4 overlay.

- Assign the first usable IPv4 address, 10.10.12.81/30, to HQ-R1 Tunnel0.

- Assign the second usable IPv4 address, 10.10.12.82/30, to TUC-R2 Tunnel0.

- Use the next available enterprise IPv6 /64 subnet, 2001:DB8:C001:0005::/64, for the GRE IPv6 overlay.

- Assign 2001:DB8:C001:0005::1/64 and FE80::1 to HQ-R1 Tunnel0.

- Assign 2001:DB8:C001:0005::2/64 and FE80::2 to TUC-R2 Tunnel0.

- Record the planned addressing in the Enterprise IP Addressing Plan before configuring Tunnel0.

### Phase 4 – Configure the Dual-Stack GRE Tunnel

### Background

Generic Routing Encapsulation creates a logical point-to-point tunnel between Phoenix Headquarters and the Tucson Distribution Center. The tunnel uses the public IPv4 ISP connections as its underlay while carrying private enterprise IPv4, IPv6, and routing protocol traffic through the overlay.

Tunnel0 functions as a logical point-to-point interface between Phoenix Headquarters and the Tucson Distribution Center. Although no physical cable exists between the two routers, OSPF treats the GRE tunnel as another routed interface over which routing information can be exchanged.

GRE does not provide encryption. The tunnel improves WAN availability, but it should not be interpreted as a secure encrypted VPN.

Because GRE configuration syntax is not provided in the NetAcad module, use the approved command templates below.

☐ Configure the dual-stack GRE backup tunnel.

#### Step 1 – Configure Tunnel0 on HQ-R1

Use the following command template to assign the IPv4 and IPv6 addresses planned in Phase 3 to the Tunnel0 interface and configure the GRE tunnel. Replace each placeholder with the appropriate values from your Enterprise IP Addressing Plan.

HQ-R1

interface Tunnel0

ip address 10.10.12.81 255.255.255.252

ipv6 address 2001:DB8:C001:0005::1/64

ipv6 address FE80::1 link-local

description HQ-R1 T0 GRE Backup Tunnel to TUC-R2 | IPv4 10.10.12.80/30 | IPv6 2001:DB8:C001:0005::/64

tunnel source GigabitEthernet0/1

tunnel destination 203.0.113.14

tunnel mode gre ip

no shutdown

At a minimum:

- Configure the approved IPv4 and IPv6 Tunnel0 addressing.

- Use GigabitEthernet0/1 as the tunnel source.

- Use 203.0.113.14 as the tunnel destination.

- Configure the approved interface description.

- Verify that Tunnel0 becomes operational.

- Save the device configuration.

#### Step 2 – Configure Tunnel0 on TUC-R2

Use the following command template to assign the IPv4 and IPv6 addresses planned in Phase 3 to the Tunnel0 interface and configure the GRE tunnel. Replace each placeholder with the appropriate values from your Enterprise IP Addressing Plan.

TUC-R2

interface Tunnel0

ip address 10.10.12.82 255.255.255.252

ipv6 address 2001:DB8:C001:0005::2/64

ipv6 address FE80::2 link-local

description TUC-R2 T0 GRE Backup Tunnel to HQ-R1 | IPv4 10.10.12.80/30 | IPv6 2001:DB8:C001:0005::/64

tunnel source GigabitEthernet0/2

tunnel destination 203.0.113.2

At a minimum:

- Configure the approved IPv4 and IPv6 Tunnel0 addressing.

- Use GigabitEthernet0/2 as the tunnel source.

- Use HQ-R1’s ISP-R1-facing public IPv4 address as the tunnel destination.

- Configure the approved interface description.

- Verify that Tunnel0 becomes operational.

- Save the device configuration.

#### Step 3 – Verify Tunnel Connectivity

Verify the GRE tunnel configuration and operational status on HQ-R1 and TUC-R2. Complete each verification item before proceeding.

Important:

Packet Tracer supports the dual-stack Tunnel0 configuration and correctly displays the assigned IPv6 addresses and connected IPv6 route. However, in this simulated environment, IPv6 traffic may not successfully traverse the IPv4 GRE tunnel. You are still required to configure and verify the IPv6 Tunnel0 addressing as directed. IPv4 GRE connectivity must operate successfully before proceeding.

| Verification | Device(s) / Test | Expected Result | Result |
| --- | --- | --- | --- |
| IPv4 GRE overlay connectivity | HQ-R1 → 10.10.12.82 | Ping successful | ☐ Pass ☐ Fail |
| IPv4 GRE overlay connectivity | TUC-R2 → 10.10.12.81 | Ping successful | ☐ Pass ☐ Fail |
| Tunnel0 operational status | HQ-R1 | Tunnel0 is up/up | ☐ Verified |
| Tunnel0 operational status | TUC-R2 | Tunnel0 is up/up | ☐ Verified |
| IPv6 global unicast addressing | HQ-R1 and TUC-R2 | Correctly assigned to Tunnel0 | ☐ Verified |
| IPv6 link-local addressing | HQ-R1 and TUC-R2 | Correctly assigned to Tunnel0 | ☐ Verified |
| IPv6 connected route | HQ-R1 and TUC-R2 | 2001:DB8:C001:0005::/64 is directly connected via Tunnel0 | ☐ Verified |

Important Packet Tracer IPv6 GRE Limitation:

Successful IPv6 ping connectivity across Tunnel0 is not required for this activity. Packet Tracer allows the IPv6 Tunnel0 addressing and connected IPv6 route to be configured and verified; however, IPv6 traffic may not successfully traverse the IPv4 GRE tunnel in the simulated environment.

Do not proceed until both IPv4 GRE connectivity tests are successful and all Tunnel0 configuration and status items have been verified. Troubleshoot and correct any IPv4 GRE tunnel issues before continuing.

### Phase 5 – Integrate the GRE Tunnel into the Enterprise Routing Domain

### Background

The GRE tunnel must exchange enterprise IPv4 routes while remaining a backup path. OSPFv2 will operate across Tunnel0, allowing the GRE tunnel to participate in the existing enterprise routing domain. The existing private WAN connection must remain the preferred Phoenix-to-Tucson path during normal operations.

OSPF selects the path with the lowest cumulative cost. Before conducting failover testing, you will compare the OSPF cost of the existing private WAN connection with the GRE tunnel. The GRE tunnel should have a higher OSPF cost, allowing the existing private WAN to remain the preferred path while Tunnel0 remains available as a backup.

☐ Integrate the GRE tunnel into the enterprise IPv4 routing domain and verify the intended OSPF path preference.

#### Step 1 – Configure and Verify OSPFv2 on Tunnel0

At a minimum:

Configure OSPFv2 Area 0 on Tunnel0 for both tunnel endpoints on HQ-R1 and TUC-R2.

Advertise the GRE IPv4 private overlay network, 10.10.12.80/30, on both HQ-R1 and TUC-R2.

Ensure Tunnel0 is not configured as a passive interface.

Use the appropriate OSPF verification commands to confirm the OSPFv2 neighbor relationship across Tunnel0 and verify the intended path preference.

| Verification | Expected Result | Result |
| --- | --- | --- |
| HQ-R1 forms an OSPFv2 neighbor relationship with TUC-R2 across Tunnel0 | Full adjacency | ☐ Verified |
| TUC-R2 forms an OSPFv2 neighbor relationship with HQ-R1 across Tunnel0 | Full adjacency | ☒ Verified |

#### Verify OSPF Path Preference

Use show ip ospf interface to identify the OSPF cost of the existing private WAN connection and the GRE backup tunnel. Record the observed values below.

| Verification | Expected Result | Observed OSPF Cost |
| --- | --- | --- |
| Primary Private WAN | Serial0/0/0 | Enter cost value:______ |
| GRE Backup Path | Tunnel0 | Enter cost value:______ |

Compare the two OSPF costs. OSPF prefers the path with the lower cumulative cost. Confirm that Tunnel0 has a higher OSPF cost than the existing private WAN connection, allowing the private WAN to remain the preferred path during normal operations while the GRE tunnel remains available as a backup path.

#### ☐ I verified that the existing private WAN has the lower OSPF cost and remains the preferred path during normal operations.

### Phase 6 – Confirm GRE Backup-WAN Readiness

### Background

Before beginning the formal Enterprise Verification & Validation procedures, confirm that all GRE backup-WAN implementation tasks have been completed and that no unresolved configuration issues remain.

The detailed operational behavior of the enterprise network, preferred routing path, and GRE failover capability will be formally tested and documented during the Enterprise Verification & Validation procedures.

☐ Confirm GRE backup-WAN readiness.

At a minimum:

Confirm that all required GRE underlay, Tunnel0, and OSPFv2 configuration tasks have been completed.

Confirm that all verification checkpoints from the previous implementation phases were successfully completed.

Troubleshoot and resolve any outstanding configuration issues before beginning Enterprise Verification & Validation.

Save the device configurations.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

Include the source device in every screenshot.

Ensure the destination address or device is visible.

Include the complete output for all required verification commands.

If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Validate Normal Enterprise Network Operations

### Background

Before testing the GRE backup tunnel, verify that the enterprise network is operating normally using the existing private WAN connection between Phoenix Headquarters and the Tucson Distribution Center. These baseline tests confirm that all enterprise resources remain accessible prior to simulating a WAN failure.

☐ Validate normal enterprise network operations.

At a minimum:

Perform the required IPv4 & IPv6 connectivity tests using the validation table below.

Record the results of each test using the Success or Fail checkboxes.

Verify that all enterprise departments can successfully communicate.

Troubleshoot and resolve any connectivity issues before proceeding to the next validation step.

IPv4 Ping Connectivity

| Source Department | Destination | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-HR1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-SCS1 | Flagstaff PC-SCS1 | ☐ | ☐ |
| Flagstaff PC-SCS1 | Phoenix PC-AF1 | ☐ | ☐ |
| Tucson PC-WH1 | ISP-WEB1 198.51.100.10 | ☐ | ☐ |

IPv6 Ping Connectivity

| Source Department | Destination | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-HR1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-SCS1 | Flagstaff PC-SCS1 | ☐ | ☐ |
| Flagstaff PC-SCS1 | Phoenix PC-AF1 | ☐ | ☐ |
| Tucson PC-WH1 | ISP-WEB1 2001:DB8:FFFF:1::10 | ☐ | ☐ |

Important:

If any test produces an unexpected result, troubleshoot, and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

#### Step 2 – Analyze the Baseline Enterprise Routing Path

#### Background

Traceroute identifies the path packets take through the enterprise network to reach their destination. Before simulating a WAN failure, analyze and document the current routing path. During normal operations, traffic between Phoenix Headquarters and the Tucson Distribution Center should travel across the existing private WAN connection because it has a lower OSPF cost than the GRE backup tunnel.

These baseline traceroute results will be compared with the results collected after the simulated WAN failure to verify that enterprise traffic automatically transitions to the GRE tunnel.

☐ Analyze the baseline enterprise routing path.

At a minimum:

Run the required traceroute tests listed below.

Analyze the routing path used to reach each destination.

Verify that traffic is using the existing private WAN connection.

Copy and paste the complete traceroute results into the tables provided.

Compare these baseline results with the traceroute results collected after the simulated WAN failure.

IPv4 Baseline Traceroute Connectivity

| Source Department | Destination | Screenshot |
| --- | --- | --- |
| Phoenix PC-IT1 | Tucson PC-WH1 |  |
| Phoenix PC-HR1 | Tucson PC-WH1 |  |

IPv6 Baseline Traceroute Connectivity

| Source Department | Destination | Screenshot |
| --- | --- | --- |
| Phoenix PC-IT1 | Tucson PC-WH1 |  |
| Phoenix PC-HR1 | Tucson PC-WH1 |  |

Important:

Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify the implementation.

#### Step 3 – Simulate Primary WAN Failure

### Background

The GRE tunnel has been deployed as a backup WAN connection between Phoenix Headquarters and the Tucson Distribution Center. Because the enterprise network contains multiple private WAN paths, the failure of a single WAN connection does not necessarily cause traffic to transition to the GRE tunnel. OSPF evaluates all available routes and continues to use a lower-cost private enterprise path whenever one remains available.

To validate the GRE backup design, this test will simulate the loss of Tucson's private enterprise WAN connectivity while leaving the public ISP connection and GRE tunnel operational. Once the private WAN paths are unavailable, OSPF should recalculate the routing topology and select Tunnel0 as the remaining path to the Tucson enterprise networks.

Important:

Packet Tracer IPv6 GRE Limitation: The GRE tunnel has been configured with both IPv4 and IPv6 overlay addressing. However, Packet Tracer does not reliably forward IPv6 traffic across the IPv4 GRE tunnel. Therefore, the following failover validation procedures verify IPv4 connectivity and OSPFv2 routing only. IPv6 ping, traceroute, and OSPFv3 failover validation across Tunnel0 are not required.

☐ Simulate the loss of Tucson private WAN connection.

### At a minimum:

Simulate the loss of Tucson's private enterprise WAN connectivity by disabling the following interfaces:

- HQ-R1 – Serial0/0/0: Primary private WAN connection between HQ-R1 and TUC-R2.

- TUC-R2 – Serial0/0/1: Private WAN connection toward Flagstaff.

- TUC-R2 – GigabitEthernet0/1: Private WAN connection toward HQ-R2.

Do not disable TUC-R2 GigabitEthernet0/2. This interface provides the public ISP underlay required for the GRE tunnel.

Verify that Tunnel0 remains up/up on HQ-R1 and TUC-R2.

Allow approximately 30–60 seconds for OSPFv2 to detect the failures and complete route convergence.

Verify that the OSPFv2 neighbor relationship across Tunnel0 remains FULL.

Verify that OSPF has selected Tunnel0 as the path to the Tucson enterprise networks.

Troubleshoot and resolve any unexpected routing or tunnel issues before proceeding.

- Routing Behavior:

- The GRE tunnel is not intended to replace another available lower-cost private enterprise route. If only one Tucson WAN connection fails, OSPF may select another available private WAN path. Tunnel0 becomes the forwarding path when the lower-cost private WAN paths to Tucson are unavailable.

#### Step 4 – Validate Automatic GRE Tunnel Failover

### Background

Following the simulated WAN failure, verify that enterprise communication continues uninterrupted using the GRE backup tunnel. These tests confirm that the GRE tunnel and dynamic routing configuration provide automatic failover while maintaining connectivity between enterprise locations.

☐ Validate automatic GRE tunnel failover.

### At a minimum:

Repeat each IPv4 connectivity test completed in Step 1.

Record the results using the Success or Fail checkboxes.

Verify that the required IPv4 enterprise communication continues successfully following the simulated WAN failure.

Confirm that enterprise traffic continues to reach the Tucson Distribution Center through the GRE tunnel.

Troubleshoot and resolve any unexpected connectivity issues before proceeding.

IPv4 Ping Connectivity

| Source Department | Destination | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-HR1 | Tucson PC-WH1 | ☐ | ☐ |
| Phoenix PC-SCS1 | Flagstaff PC-SCS1 | ☐ | ☐ |
| Flagstaff PC-SCS1 | Phoenix PC-AF1 | ☐ | ☐ |
| Tucson PC-WH1 | ISP-WEB1 198.51.100.10 | ☐ | ☐ |

Important:

If any test produces an unexpected result, troubleshoot, and resolve the issue before continuing with the Enterprise Verification & Validation procedures.

#### Step 5 – Analyze the Enterprise Routing Path After Failover

### Background

After OSPF has completed route convergence, analyze the updated routing path to verify that enterprise traffic has successfully transitioned from the primary private WAN connection to the GRE backup tunnel. Comparing the traceroute results to the baseline collected in Step 2 provides visual confirmation that the GRE tunnel is now carrying production traffic.

☐ Analyze the enterprise routing path after failover.

### At a minimum:

Repeat the same IPv4 traceroute tests completed in Step 2.

Copy and paste the complete traceroute results into the tables provided.

Compare the new traceroute results with the baseline traceroute collected in Step 2.

Verify that traffic between Phoenix Headquarters and the Tucson Distribution Center is now traveling across the GRE Tunnel0 backup path rather than the primary private WAN connection.

Confirm that the routing path changed only as expected following the simulated WAN failure.

Troubleshoot and resolve any unexpected routing behavior before proceeding.

IPv4 Failover Traceroute Connectivity

| Source Department | Destination | Screenshot |
| --- | --- | --- |
| Phoenix PC-IT1 | Tucson PC-WH1 |  |
| Phoenix PC-HR1 | Tucson PC-WH1 |  |

Important:

Your validation package must clearly document the source of each connectivity test. Every screenshot must include both the source device and the traceroute results so the Director of Network Engineering can verify the implementation.

#### Step 6 - Restore Normal Enterprise Operations

#### Background

After successfully validating the GRE backup tunnel, restore the private enterprise WAN connections serving the Tucson Distribution Center. During normal enterprise operations, OSPF should prefer the available lower-cost private WAN paths rather than the higher-cost GRE backup tunnel.

Once the private WAN connections have been restored, OSPF should automatically recalculate the routing topology and return enterprise traffic to the preferred private WAN path while leaving the GRE tunnel operational and available as a backup.

☐ Restore normal enterprise operations.

At a minimum:

- Restore Tucson's private enterprise WAN connectivity by re-enabling the following interfaces:

  - HQ-R1 – Serial0/0/0: Primary private WAN connection between HQ-R1 and TUC-R2.

  - TUC-R2 – Serial0/0/1: Private WAN connection toward Flagstaff.

  - TUC-R2 – GigabitEthernet0/1: Private WAN connection toward HQ-R2.

Allow approximately 30 to 60 seconds for OSPFv2 to detect the restored WAN connections and complete route convergence.

Verify that the existing private WAN connection once again becomes the preferred path between Phoenix Headquarters and the Tucson Distribution Center.

Verify that the GRE Tunnel0 remains operational and available as the backup WAN connection.

Repeat several of the IPv4 and IPv6 baseline connectivity tests from Step 1 to verify that normal enterprise connectivity has been fully restored.

Run an IPv4 traceroute from Phoenix Information Technology (PC-IT1) to Tucson Warehouse & Distribution (PC-WH1) and verify that traffic is once again traveling across the primary private WAN connection rather than the GRE backup tunnel.

Troubleshoot and resolve any unexpected routing or connectivity issues before proceeding to the remaining verification steps.

#### Step 7 – Verify Enterprise Routing and OSPF Operation

### Background

Following successful failover testing and restoration of the primary WAN connection, verify that the enterprise routing and GRE tunnel operation have been implemented correctly. The following Cisco IOS commands provide evidence that the GRE tunnel is operational and that OSPF has selected the appropriate routing path.

Important:

A GRE tunnel interface may remain operational even if dynamic routing across the tunnel has failed. Always verify both the tunnel status and the OSPF neighbor relationship before concluding that the tunnel is functioning correctly.

☐ Verify enterprise routing and GRE tunnel operation.

### HQ-R1 and TUC-R2

Run the following command.

show ip route

Highlight the following:

Gateway of Last Resort

The preferred OSPF route used to reach the Tucson enterprise networks and verify that traffic is using the primary private WAN connection.

| Device | Required Output - Paste show ip route |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |

Run the following command.

show ipv6 route

Highlight the following:

The preferred OSPF route used to reach the Tucson enterprise networks and verify that traffic is using the primary private WAN connection.

| Device | Required Output - Paste show ipv6 route |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |

Run the following command.

show ip ospf neighbor

Locate the OSPF neighbor relationship formed with TUC-R2 across Tunnel0. Highlight the following:

Neighbor ID

Neighbor state

Interface Tunnel0

| Device | Required Output - Paste show ip ospf neighbor |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |

Run the following command.

show ip ospf interface

Locate the following OSPF interface information.

Highlight the following:

- OSPF cost for the primary private WAN interface Serial0/0/0

- OSPF cost for Tunnel0

- Area 0 participation for both interfaces

- Tunnel0 network type

| Device | Required Output - Paste show ip ospf interface |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |

| Device | Primary WAN Interface | Primary WAN OSPF Cost | GRE Interface | Observed OSPF Cost |
| --- | --- | --- | --- | --- |
| HQ-R1 | Serial0/0/0 | Enter cost value:______ | Tunnel0 | Enter cost value:______ |
| TUC-R2 | Serial0/0/0 | Enter cost value:______ | Tunnel0 | Enter cost value:______ |

#### ☐ I verified that the existing private WAN has the lower OSPF cost and remains the preferred path during normal operations.

#### Step 8 – Verify GRE Tunnel Operation

### Background

The GRE tunnel provides the backup IPv4 WAN path between Phoenix Headquarters and the Tucson Distribution Center. Before completing the Enterprise Change Request, verify that Tunnel0 is operational and that its IPv4 and IPv6 overlay addressing is configured according to enterprise standards.

☐ Verify GRE tunnel operation.

### HQ-R1 and TUC-R2

Run the following command.

show interface tunnel0

Highlight the following:

- Tunnel interface status (up/up)

- IPv4 address assigned to Tunnel0

- Tunnel source interface

- Tunnel destination IPv4 address

- Tunnel protocol/transport (GRE/IP)

| Device | Required Output - Paste show interface tunnel0 |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |

Run the following command.

show ipv6 interface tunnel0

Highlight the following:

- Tunnel interface status (up/up)

- IPv6 link-local address

- IPv6 global unicast address

- IPv6 subnet assigned to Tunnel0

| Device | Required Output - Paste show ipv6 interface tunnel0 |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |

#### Step 9 – Verify Final Device Configurations

### Background

The running configuration provides a complete record of the active device configuration and is commonly reviewed by network engineers following a planned change. Before closing this Enterprise Change Request, verify that all required GRE backup-WAN configuration elements are present on both routers.

☐ Verify the final device configurations.

### HQ-R1 and TUC-R2

Run the following command:

show running-config

Highlight the following:

- Complete Tunnel0 interface configuration

- Static host route to the remote GRE tunnel endpoint

- OSPFv2 configuration that includes the GRE overlay network 10.10.12.80/30

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |

#### Step 10 – Update Enterprise Documentation

### Background

Accurate network documentation is essential for maintaining and supporting an enterprise network. Following a successful implementation, update the enterprise documentation to reflect the new GRE backup WAN design. Future network engineers will rely on this documentation when troubleshooting, expanding, or maintaining the enterprise network.

☐ Update the enterprise documentation.

### Task 1 – Update the Enterprise IP Addressing Plan

Update the Enterprise IP Addressing Plan to include the following:

The new public IPv4 transport network between TUC-R2 and ISP-R2.

The public IPv4 interface addresses assigned to TUC-R2 and ISP-R2.

The new private IPv4 GRE Tunnel0 overlay network.

The IPv4 Tunnel0 interface addresses for HQ-R1 and TUC-R2.

The IPv6 GRE Tunnel0 overlay subnet.

The IPv6 Tunnel0 global unicast addresses for HQ-R1 and TUC-R2.

The Tunnel0 IPv6 link-local addresses.

### Task 2 – Update the Enterprise Network Standards Guide

Update the Enterprise Network Standards Guide to include the following:

The new TUC-R2 to ISP-R2 public WAN connection.

The new Tunnel0 virtual interfaces on HQ-R1 and TUC-R2.

The approved Tunnel0 interface description standard.

The GRE tunnel source and destination configuration standards.

Any new router interface information introduced by this deployment.

Any additional enterprise implementation standards established during this ECR.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Network Standards Guide

☐  Updated Enterprise IP Addressing Plan

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 07/15/2026