ECR-009

## Enterprise Internet Access Deployment

| Field | Scope |
| --- | --- |
| Requested by | Network Engineering Department |
| Assigned to | Junior Network Engineer |
| Priority | High |
| Change type | Normal |
| Implementation Window | Friday 11:00pm to Saturday 2:00am (Approved maintenance window) |
| Implementation Time | Estimated Implementation Effort: 2–4 hours See established due date |
| Business Impact | Medium (Temporary interruption expected during implementation) |

## Business Justification

As Sonoran Outdoor Equipment has continued to expand, employees increasingly rely on Internet access to perform daily business operations. Staff regularly access vendor websites, cloud-based applications, software updates, shipping providers, technical documentation, and other external business resources. Although the enterprise network has been successfully deployed using private IPv4 addressing, these addresses cannot be routed across the public Internet.

To provide secure enterprise Internet connectivity while conserving the limited supply of public IPv4 addresses, the Network Engineering Department has approved the implementation of Port Address Translation (PAT) on the headquarters edge router. PAT enables multiple enterprise devices to share a single public IPv4 address by uniquely identifying each communication session using Layer 4 port numbers.

As part of this Enterprise Change Request (ECR), the headquarters edge router will be configured to translate internal private IPv4 addresses to a single public IPv4 address assigned by the Internet Service Provider (ISP). This implementation will provide Internet access for all enterprise users while reducing public IPv4 address consumption and preventing internal enterprise addressing from being exposed to external networks.

Following implementation, enterprise users will be able to securely access Internet-based resources while maintaining full connectivity to internal enterprise services. This design supports current business operations, improves scalability, and follows industry best practices for enterprise edge deployments.

## Objectives

Upon successful completion of this Enterprise Change Request, you will be able to:

- Explain the purpose and business benefits of Port Address Translation (PAT) in an enterprise network.

- Configure the appropriate NAT inside and NAT outside interfaces on an enterprise edge router.

- Configure a standard IPv4 access control list (ACL) to identify enterprise networks eligible for Port Address Translation (PAT).

- Configure PAT using interface overload to allow multiple private IPv4 devices to share a single public IPv4 address.

- Verify that enterprise Internet connectivity is available to authorized departments while existing enterprise security policies remain enforced.

- Verify Port Address Translation operation using Cisco IOS verification commands.

- Interpret the NAT translation table and PAT statistics to confirm successful address translation.

- Verify the final PAT configuration using Cisco IOS show commands.

- Apply enterprise change management procedures while maintaining enterprise connectivity and security.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Company Overview

✔ Enterprise Network Standards Guide

✔ Enterprise Network Addressing Plan

✔ Enterprise Topology

✔ Interface Addressing Plan

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

- Enterprise Network Topology

- Enterprise Network Standards Guide

- Enterprise IP Addressing Plan

- This Enterprise Change Request (ECR-009)

## Implementation Plan

### Phase 1 – Prepare the Enterprise Internet Edge

### Background

HQ-R1 currently serves as the enterprise Autonomous System Boundary Router (ASBR) and provides the only connection between the Sonoran Outdoor Equipment private network and the Internet Service Provider. The enterprise default route has already been configured and distributed through the OSPF routing domain. Before implementing Port Address Translation (PAT), verify that the existing Internet-edge routing configuration remains operational and that all enterprise locations can forward external traffic toward HQ-R1.

### Enterprise Internet Edge Validation

Before implementing Port Address Translation (PAT), validate the existing IPv4 routing environment. Complete each validation item below and confirm that the expected routing and connectivity are operational before proceeding.

HQ-R1

| Validation Requirement | Verified |
| --- | --- |
| Verify that GigabitEthernet0/1 toward ISP-R1 is operational. | ☐ |
| Verify that the IPv4 default route toward ISP-R1 is installed. | ☐ |
| Verify that the IPv4 default route is being advertised into the OSPFv2 routing domain. | ☐ |
| Verify IPv4 connectivity to ISP-R1. (ping) | ☐ |

HQ-R2

| Validation Requirement | Verified |
| --- | --- |
| Verify that an OSPF-learned IPv4 default route is installed and directs Internet-bound traffic toward HQ-R1. | ☐ |
| Verify IPv4 connectivity to ISP-R1. (ping) | ☐ |

TUC-R2

| Validation Requirement | Verified |
| --- | --- |
| Verify that an OSPF-learned IPv4 default route is installed and directs Internet-bound traffic toward HQ-R1. | ☐ |
| Verify IPv4 connectivity to ISP-R1. (ping) | ☐ |

FLG-R3

| Validation Requirement | Verified |
| --- | --- |
| Verify that an OSPF-learned IPv4 default route is installed and directs Internet-bound traffic toward HQ-R1. | ☐ |
| Verify IPv4 connectivity to ISP-R1. (ping) | ☐ |

- Troubleshoot and correct any routing or ISP connectivity issues before proceeding.

- Save the device configuration.

### Phase 2 – Identify Enterprise NAT Interfaces and Traffic

### Background

Port Address Translation requires the enterprise edge router to distinguish between traffic originating from the private enterprise network and traffic traveling toward the public ISP network. Because HQ-R1 receives internal enterprise traffic through multiple routed interfaces and subinterfaces, each applicable enterprise-facing Layer 3 interface must be identified as a NAT inside interface. The ISP-facing interface will be identified as the NAT outside interface.

A standard IPv4 Access Control List will identify the approved private enterprise networks eligible for translation. This ACL is used only to classify traffic for PAT and does not function as an enterprise traffic-filtering security policy.

☐ Identify the enterprise NAT interfaces and approved inside traffic.

#### Step 1 – Configure the NAT Inside Interfaces

☐ Configure the appropriate HQ-R1 enterprise-facing Layer 3 interfaces as NAT inside interfaces.

At a minimum:

Apply the NAT inside designation to each applicable routed interface and subinterface that receives traffic from the private enterprise network.

Confirm that the physical GigabitEthernet0/0 interface is not used in place of its routed subinterfaces.

Verify that all affected interfaces remain operational.

Save the device configuration.

Important:

The ip nat inside designation identifies an interface as being on the private enterprise side of the NAT boundary. It does not, by itself, determine which IPv4 networks will be translated. The NAT traffic-classification ACL configured later in this phase determines which internal networks are eligible for PAT.

#### Step 2 – Configure the NAT Outside Interface

☐ Configure the appropriate HQ-R1 ISP-facing Layer 3 interface as the NAT outside interface.

At a minimum:

Apply the NAT outside designation to the ISP-facing interface.

Verify that the interface remains operational.

Verify connectivity between HQ-R1 and the ISP router.

Important:

NAT inside and NAT outside designations are configured only on the router performing the address translation. In this deployment, HQ-R1 serves as the enterprise NAT boundary. Enterprise-facing interfaces on HQ-R1 are on the inside of this boundary, while the interface connecting HQ-R1 to the ISP is on the outside.

Routers at other enterprise locations continue to route private IPv4 traffic toward HQ-R1 and do not perform NAT as part of this implementation.

#### Step 3 – Identify Enterprise Traffic Eligible for PAT

☐ Configure the named standard IPv4 ACL NAT-INSIDE to identify the approved private IPv4 enterprise networks that require Internet access.

At a minimum:

Include all currently implemented Phoenix Headquarters production user networks that require Internet access.

Include the Tucson Distribution Center user network.

Include the Flagstaff Sales Retail Store user network.

Do not include internal networks that do not require Internet access.

Verify that the ACL identifies only the approved private IPv4 source networks eligible for PAT.

Verify the ACL entries before proceeding.

Save the device configuration.

Important:

The NAT-INSIDE ACL is a traffic-classification ACL used by PAT to identify which private IPv4 source networks are eligible for translation. It does not filter or block traffic like a security ACL.

An interface designated as ip nat inside does not automatically make every network associated with that interface eligible for translation. Only source networks permitted by the NAT-INSIDE ACL will be selected for PAT.

### Phase 3 – Deploy Port Address Translation

### Background

Port Address Translation allows multiple private enterprise devices to share the public IPv4 address assigned to HQ-R1’s ISP-facing interface. When eligible internal traffic exits the enterprise, HQ-R1 translates the private source IPv4 address and uses unique transport-layer port numbers or identifiers to distinguish simultaneous sessions.

This conserves public IPv4 address space while allowing multiple enterprise users to access external IPv4 resources at the same time.

☐ Deploy Port Address Translation on HQ-R1.

### At a minimum:

Configure PAT on HQ-R1 using the public IPv4 address assigned to the ISP-facing interface.

Use the named standard IPv4 ACL NAT-INSIDE to identify traffic eligible for translation.

Configure PAT using interface overload.

Verify that the PAT configuration references the correct:

- NAT traffic-classification ACL

- ISP-facing interface

- Overload option

Verify PAT Operation

☐ Generate IPv4 ping traffic from each enterprise location and verify that PAT translations are created.

From the specified client at each enterprise location, ping the external web server at 198.51.100.10. After generating the traffic, use the appropriate verification command on HQ-R1 to confirm that a PAT translation was created for the source client.

| Source Device | Destination | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | 198.51.100.10 | ☐ | ☐ |
| Tucson PC-WH1 | 198.51.100.10 | ☐ | ☐ |
| Flagstaff PC-SCS1 | 198.51.100.10 | ☐ | ☐ |

At a minimum:

- Generate the required ICMP traffic from each source device.

- Verify that each ping is successful.

- On HQ-R1, use show ip nat translations to verify that a translation is created for each source client.

- Confirm that the Inside Local Address corresponds to the private IPv4 address of the source client.

- Confirm that the Inside Global Address corresponds to the public IPv4 address assigned to HQ-R1's ISP-facing interface.

- Confirm that ICMP identifiers are used to distinguish translated sessions.

- Troubleshoot and correct any unexpected results before proceeding.

Important:

Successful completion of these tests confirms that PAT is translating Internet-bound IPv4 traffic from each enterprise location. Additional application-level testing and evaluation of existing enterprise security controls will be performed in the next phase.

Important:

With interface overload, PAT uses the IPv4 address currently assigned to the specified outside interface as the inside global address. A separate public address pool is not required for this deployment.

### Phase 4 – Evaluate and Adapt the Existing Security Policy

## Background

The deployment of Port Address Translation (PAT) changes how IPv4 traffic is processed as it moves from the private enterprise network toward the public ISP network. Although PAT has been successfully configured and translation entries are being created, the implementation must also be evaluated against security controls that were deployed during earlier Enterprise Change Requests.

Enterprise networks continuously evolve. A configuration that correctly supported business and security requirements when it was originally implemented may need to be modified when new technologies, services, or traffic patterns are introduced. Network engineers must understand how a new configuration interacts with existing controls and make adjustments when necessary.

As part of this phase, you will perform an initial application-level connectivity test, investigate the observed result, determine how the newly implemented PAT configuration interacts with the existing Enterprise Security Policy, and modify the implementation as necessary.

The objective is not to remove or weaken the existing security policy. The objective is to ensure that the existing policy continues to be enforced correctly after PAT is introduced.

#### Step 1 – Perform Initial Post-Implementation Testing

☐ Test HTTPS access following the PAT deployment.

From the specified enterprise client, open the Web Browser and attempt to access the external web server using:

https://198.51.100.10

Record the observed result.

Internet Access & Security Validation

| Source Device | External Resource | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |
| Tucson PC-WH1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |
| Flagstaff PC-SCS1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |

If the HTTPS request is unsuccessful, do not immediately remove or modify the PAT configuration. Continue to the next step and use the available network information to determine where the traffic is being affected.

#### Step 2 – Determine Whether PAT Is Operating

A failed application request does not necessarily mean that PAT has failed. Before modifying the configuration, determine whether HQ-R1 successfully created translations for the HTTPS traffic generated by the tested enterprise clients.

☐ Investigate the PAT translation table on HQ-R1.

Run:

show ip nat translations

Locate the translation associated with the tested clients and examine the following information:

- Protocol

- Inside Local Address

- Inside Global Address

- Outside Local Address

- Outside Global Address

- Port/identifier translations

Use the observed output to answer the following question:

Did HQ-R1 create a PAT translation for the HTTPS request generated by the enterprise clients?

☐ Yes
☐ No

Consider: If the application request was unsuccessful but a valid PAT translation was created, is PAT itself the most likely cause of the failed HTTPS connection?

#### Step 3 – Investigate the Existing Enterprise Security Policy

Earlier Enterprise Change Requests implemented security controls governing access to the external HTTPS resource. Because PAT is successfully translating the traffic, investigate whether an existing security control may be affecting the translated connection.

☐ Review the existing IPv4 security configuration on HQ-R1.

At a minimum:

- Identify the ACL responsible for enforcing access to the external HTTPS resource.

- Review the permit and deny statements contained in the ACL.

- Determine where the ACL is currently applied.

- Determine the direction in which the ACL evaluates traffic.

- Examine ACL match counters for evidence that HTTPS traffic is being denied.

Use appropriate IOS verification commands to complete your investigation.

### Analysis

Compare the IPv4 source address shown as the Inside Local Address in the NAT translation table with the Inside Global Address used after translation.

Then compare those addresses with the source networks referenced by the existing HTTPS security ACL.

Consider the following:

- Which source IPv4 address does the existing ACL expect to evaluate?

- Which source IPv4 address is present after PAT occurs?

- Can multiple enterprise networks be represented by the same translated public IPv4 address?

- At the ACL's current location, can HQ-R1 still distinguish which enterprise network originally generated the traffic?

IMPORTANT – Enterprise Networks Evolve

Network configurations are designed to support the requirements that exist when they are implemented. As an enterprise network grows and new technologies are introduced, previously correct configurations may need to be adapted.

The Enterprise Security Policy has not changed. Authorized enterprise networks must continue to access the approved HTTPS resource, while restricted networks must remain blocked.

However, PAT changes the source IPv4 address of eligible enterprise traffic as it travels toward the ISP. Multiple private enterprise addresses may therefore share the same translated public IPv4 address.

A security control that needs to distinguish traffic by its original private source network must be positioned where those original source addresses can still be evaluated.

Do not remove a required security control simply to restore connectivity. Modify its implementation so that both the connectivity requirement and the security requirement continue to be satisfied.

#### Step 4 – Adapt the Existing Security Policy Implementation

☐ Modify the implementation of the existing HTTPS security ACL so that the Enterprise Security Policy continues to function correctly after PAT deployment.

The existing ACL statements continue to represent the approved Enterprise Security Policy and do not need to be rewritten. Instead, modify where the ACL is applied so that traffic can be evaluated using its original private source IPv4 address before PAT translation occurs.

At a minimum:

- Remove the existing ACL application from the ISP-facing interface on HQ-R1.

- Do not delete or recreate the existing ACL.

- Apply the existing ACL in the appropriate direction on the HQ-R1 enterprise-facing interfaces that receive traffic from:

  - Phoenix Headquarters

  - Tucson Distribution Center

  - Flagstaff Sales Retail Store

- Ensure that the ACL evaluates enterprise traffic before PAT changes the original private source IPv4 address.

- Verify that the ACL is applied to the intended interfaces and in the intended direction.

- Do not modify the underlying Enterprise Security Policy merely to permit connectivity.

Important – IPv4 and IPv6 Security Policy

The ACL placement changes in this step apply only to the IPv4 security ACL affected by PAT. PAT in this deployment translates IPv4 traffic and does not alter the source addresses used by the existing IPv6 security policy.

Do not modify or relocate the existing IPv6 ACL. The IPv6 ACL should remain applied in its current location and direction on HQ-R1.

#### Step 5 – Verify the Adapted Security Policy

☐ Repeat application-level testing after modifying the ACL implementation.

Use the Web Browser on each specified client to access: https://198.51.100.10

Record the observed results.

| Source Device | External Resource | Expected Result | Success | Fail |
| --- | --- | --- | --- | --- |
| Phoenix PC-IT1 | ISP-WEB1 https://198.51.100.10 | HTTPS permitted | ☐ | ☐ |
| Tucson PC-WH1 | ISP-WEB1 https://198.51.100.10 | HTTPS blocked | ☐ | ☐ |
| Flagstaff PC-SCS1 | ISP-WEB1 https://198.51.100.10 | HTTPS permitted | ☐ | ☐ |

Important: For PC-WH1, an HTTPS request that is blocked is the expected successful security outcome. Do not interpret an expected policy denial as a network failure.

Verify that:

- Authorized Phoenix traffic can access the external HTTPS resource.

- Authorized Flagstaff traffic can access the external HTTPS resource.

- Tucson Warehouse & Inventory remains restricted from accessing the external HTTPS resource.

- PAT translations are created for permitted Internet-bound traffic.

- Existing internal enterprise connectivity remains operational.

- The Enterprise Security Policy remains intact following the PAT deployment.

☐ Save the HQ-R1 configuration after all expected results have been confirmed.

### Phase 5 – Confirm Enterprise Internet Access

### Background

Following the deployment of PAT and adaptation of the existing IPv4 security policy, perform a final operational check to confirm that enterprise Internet connectivity, internal communication, and existing security requirements are functioning as expected.

Detailed connectivity, translation, and security testing was completed during the preceding implementation phases. This final check confirms that the enterprise network is ready for the formal Enterprise Verification & Validation procedures.

☐ Confirm enterprise Internet access through PAT.

Enterprise-Wide Validation

| Validation Requirement | Verified |
| --- | --- |
| Confirm that authorized enterprise clients can access the external HTTPS web server through PAT. | ☐ |
| Confirm that the Tucson Warehouse & Inventory security restriction remains operational. | ☐ |
| Verify that multiple enterprise clients can establish translated sessions through HQ-R1. | ☐ |
| Verify that internal IPv4 communication between enterprise locations remains operational. | ☐ |
| Confirm that existing enterprise security requirements remain operational following the PAT deployment. | ☐ |
| Confirm that all connectivity and security requirements are functioning as expected before beginning the Enterprise Verification & Validation procedures. | ☐ |

Troubleshoot and correct any unexpected results before proceeding.

Save all affected device configurations.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

- Include the source device in every screenshot.

- Ensure the destination address or device is visible.

- Include the complete output for all required verification commands.

- If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Validate Enterprise Internet Connectivity

### Background

Following the implementation of Port Address Translation (PAT), formally validate that authorized enterprise users can access the approved external web resource while previously implemented enterprise security policies remain in effect.

☐ Validate enterprise Internet connectivity and security policy enforcement.

At a minimum:

- Perform the required Internet-access tests using the validation table below.

- Use the Web Browser on the specified source client to access the external web server using https://198.51.100.10.

- Record the observed result of each test using the Success or Blocked checkbox, as applicable.

- Capture the required validation evidence showing the source device, destination, and observed result.

- Troubleshoot and resolve any unexpected results before continuing.

Internet Access & Security Validation

| Source Device | External Resource | Success | Blocked |
| --- | --- | --- | --- |
| Phoenix PC-IT1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |
| Phoenix PC-HR1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |
| Phoenix PC-AF1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |
| Phoenix PC-SCS1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |
| Flagstaff PC-SCS1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |
| Tucson PC-WH1 | ISP-WEB1 https://198.51.100.10 | ☐ | ☐ |

Important:

A blocked Tucson HTTPS connection is the expected result and confirms that the previously implemented Enterprise Security Policy remains in effect. An expected blocked result should not be recorded as a failed validation test.

#### Step 2 – Verify Port Address Translation

### Background

Port Address Translation dynamically creates translation entries as approved enterprise clients access external resources. Reviewing the NAT translation table verifies that HQ-R1 is successfully translating private inside local IPv4 addresses to the public inside global IPv4 address assigned to its ISP-facing interface.

☐ Verify Port Address Translation operation.

### Before running the verification command:

- Generate Internet-bound traffic from multiple authorized enterprise clients to create active PAT translations.

- Run the verification command immediately after generating the traffic so that the active dynamic translations are visible.

### Run the following commands on HQ-R1:

show ip nat translations

Highlight the following:

Inside Local Address

Inside Global Address

Outside Global Address

Protocol

Port/identifier translations demonstrating multiple simultaneous sessions

| Device | Required Output - Paste show ip nat translations |
| --- | --- |
| HQ-R1 |  |

### Run the following commands on HQ-R1:

show ip nat statistics

Highlight the following:

Total active translations

Inside interfaces

Outside interfaces

Hits

Misses

| Device | Required Output - Paste show ip nat statistics |
| --- | --- |
| HQ-R1 |  |

#### Step 3 – Verify Final Device Configuration

### Background

Review the final HQ-R1 configuration to verify that Port Address Translation has been correctly implemented and that the enterprise Internet edge configuration reflects the approved Enterprise Change Request.

☐ Verify the final HQ-R1 configuration.

### Run the following command on HQ-R1:

show running-config

Highlight the following:

Named standard IPv4 ACL NAT-INSIDE identifying enterprise networks eligible for translation

PAT interface overload configuration

Updated ip access-group 110 in configuration showing the placement of ACL 110 on the appropriate HQ-R1 enterprise-facing interfaces

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 08/15/2026