ECR-007

## Implement Enterprise Access Control Policies

| Requested By | Network Engineering Department |
| --- | --- |
| Assigned To | Junior Network Engineer |
| Priority | High |
| Change Type | Normal |
| Implementation Window | Friday 11:00 PM – Saturday 2:00 AM (Approved Maintenance Window) |
| Implementation Time | Estimated Implementation Time: 2–4 hours See established due date. |
| Business Impact | Medium – Access control policies may temporarily affect connectivity to protected resources until enterprise security policies are verified. |

## Business Justification

As Sonoran Outdoor Equipment continues to expand its enterprise network, protecting sensitive business resources has become a critical security requirement. While previous Enterprise Change Requests established enterprise connectivity, routing, infrastructure security, and centralized addressing services, all internal departments currently have unrestricted access to enterprise network resources. The Network Engineering Department has approved the implementation of enterprise Access Control Lists (ACLs) to enforce business security policies and protect critical enterprise assets.

This Enterprise Change Request implements standardized IPv4 and IPv6 access control policies that restrict access to confidential enterprise resources, secure remote management of network infrastructure, and control access to external web services. These policies will improve the organization's security posture while ensuring authorized users maintain access to the network resources required to perform their job responsibilities.

The implementation will follow the Enterprise Network Standards Guide to ensure Access Control Lists are deployed consistently throughout the enterprise, provide a scalable security framework for future business growth, and support the ongoing expansion of the Sonoran Outdoor Equipment enterprise network.Top of Form

Bottom of Form

## Objectives

Upon successful completion of this Enterprise Change Request (ECR), you will be able to:

- Implement enterprise security policies using standard, extended, numbered, named, IPv4, and IPv6 Access Control Lists (ACLs).

- Configure Standard IPv4 ACLs to secure remote SSH management access to enterprise network infrastructure.

- Configure Extended IPv4 ACLs to protect confidential enterprise resources and control access to external network services.

- Configure Named IPv6 ACLs to enforce enterprise security policies within a dual-stack network environment.

- Apply IPv4 and IPv6 Access Control Lists according to enterprise ACL placement best practices.

- Verify authorized and unauthorized network access using IPv4 and IPv6 connectivity testing.

- Validate enterprise ACL implementations using Cisco IOS show commands and operational verification.

- Troubleshoot and resolve enterprise access control issues while maintaining authorized network communications.

- Document and submit an enterprise validation package demonstrating compliance with the Enterprise Network Standards Guide and approved enterprise security policies.

## Prerequisites

Before beginning this change request, verify that the following tasks have been completed:

✔ Successfully complete ECR-001 through ECR-006.

✔ The enterprise network is fully operational using both IPv4 and IPv6 addressing.

✔ OSPFv2 and OSPFv3 are fully implemented and enterprise connectivity has been verified.

✔ The Enterprise Network Standards Guide is available for reference.

✔ Update the Enterprise Topology to accurately reflect the current enterprise design and documentation before implementing the requested changes.

## Required Resources

- Enterprise Network Standards Guide

- Enterprise IP Addressing Plan

- Enterprise Topology

- Enterprise Change Request (ECR-007)

- Enterprise Change Request Process & Expectations Guide

## ACL Implementation Plan

### IPv6 ACL Configuration Reference

IPv6 Access Control Lists extend the ACL concepts and skills you have previously learned to the IPv6 environment. Because IPv6 ACL configuration syntax is not emphasized in the NetAcad curriculum used in your previous coursework, the following command structure is provided as a reference for completing the IPv6 portions of this Enterprise Change Request.

Unlike IPv4 ACLs:

- IPv6 ACLs are configured as named ACLs. IPv6 ACLs do not use the numbered standard and extended ACL ranges used by IPv4.

- IPv6 ACLs use extended ACL functionality. They can filter traffic based on source and destination IPv6 addresses, protocols, and transport-layer information such as TCP or UDP port numbers.

- IPv6 ACLs are created using the ipv6 access-list command rather than the IPv4 access-list or ip access-list commands.

Use the requirements in each implementation phase to determine the appropriate ACL name, protocol, source, destination, permit or deny action, interface placement, and direction.

Create a named IPv6 ACL:

ipv6 access-list <ACL-NAME>

permit|deny <protocol> <source> <destination> [port/operator]

exit

Apply an IPv6 ACL to an interface:

interface <interface>

ipv6 traffic-filter <ACL-NAME> in|out

Apply an IPv6 ACL to VTY lines:

line vty <line-range>

ipv6 access-class <ACL-NAME> in

### ⚠ Packet Tracer IPv6 SSH Validation Limitation

Cisco Packet Tracer has limitations when establishing IPv6 SSH sessions to simulated network devices. As a result, IPv6 SSH sessions may not operate as expected even when IPv6 addressing, routing, SSH, and access control configurations are correct.

You are still required to configure and apply SSH-IPV6-ACL to the VTY lines of all enterprise routers and switches as directed in this ECR. This configuration represents the required enterprise security implementation and provides practice implementing equivalent IPv4 and IPv6 remote management security policies.

Due to this Packet Tracer limitation, successful or denied IPv6 SSH sessions are not required as validation evidence. The IPv6 SSH access-control implementation will instead be validated using the required Cisco IOS show commands.

This limitation applies specifically to IPv6 SSH validation. Other IPv6 access-control policies in this ECR must still be implemented and validated using the required connectivity tests.

### ACL Documentation Requirement

All Access Control Lists configured as part of this Enterprise Change Request must include a meaningful remark statement documenting the purpose of the ACL.

ACL remarks are an important part of professional network documentation and help network administrators quickly understand the purpose of an ACL without having to interpret each Access Control Entry individually.

At a minimum, each ACL remark should:

- Clearly identify the business or security purpose of the ACL.

- Be concise and professionally written.

- Provide enough information for another network administrator to understand why the ACL exists.

Network engineers are responsible for creating an appropriate remark based on the security policy being implemented.

Command Reference:

remark <brief description of ACL purpose>

### Phase 1 – Configure Enterprise Device Management Access Control

Background:

The Enterprise Network Standards Guide requires that remote management of enterprise network infrastructure be restricted to authorized Information Technology personnel. Because the Sonoran Outdoor Equipment enterprise operates as a dual-stack IPv4/IPv6 network, remote management access must be protected using equivalent IPv4 and IPv6 access control policies.

☐ Configure enterprise device management access control.

At a minimum:

- Configure standard IPv4 ACL 10 to permit only the Information Technology subnet to remotely manage enterprise routers and switches using IPv4.

- Configure the named IPv6 ACL SSH-IPV6-ACL to permit only the Information Technology subnet to remotely manage enterprise routers and switches using IPv6.

- Apply IPv4 ACL 10 to the VTY lines of all enterprise routers and switches.

- Apply SSH-IPV6-ACL to the VTY lines of all enterprise routers and switches.

- Verify authorized and unauthorized IPv4 SSH management access as specified in the Enterprise Verification & Validation section.

- Verify the IPv6 SSH management access-control configuration using the required Cisco IOS show commands.

- Troubleshoot and correct any access control configuration issues before proceeding.

### Phase 2 – Protect Enterprise Financial Resources

Background:

The Accounting & Finance department contains confidential financial information that must be protected from unauthorized internal access. Enterprise security policy restricts access to Accounting & Finance resources to Executive Management personnel only. Because the enterprise operates as a dual-stack network, this access policy must be enforced for both IPv4 and IPv6 traffic.

When designing an Access Control List to protect a destination network, network engineers must consider both the security policy and the direction of legitimate network communications.

Accounting & Finance workstations must be protected from unauthorized connections initiated by other enterprise departments. However, Accounting & Finance users must still be able to initiate legitimate connections to approved resources outside their own subnet and receive the return traffic associated with those connections.

An ACL that simply permits Executive Management and then denies all other traffic destined for the Accounting & Finance subnet may unintentionally block legitimate return traffic.

### Understanding TCP Return Traffic

TCP communication is bidirectional. When an Accounting & Finance workstation initiates a TCP session, such as an HTTPS connection to a web server, the response traffic must return to the Accounting & Finance workstation.

Cisco extended ACLs can use the established keyword with TCP permit statements to identify TCP traffic that contains the ACK or RST flag associated with an established TCP conversation.

This allows the ACL to distinguish between:

- a new TCP connection attempting to reach the protected Accounting & Finance network, and

- TCP return traffic associated with a connection that was previously initiated by an Accounting & Finance workstation.

The established keyword does not make an ACL stateful. The router is not maintaining a session table. Instead, the ACL examines TCP flags to determine whether the traffic appears to be part of an existing TCP conversation.

### ACL Processing Order

Remember that Access Control Entries are evaluated from top to bottom. As soon as traffic matches an ACE, processing stops and the corresponding permit or deny action is applied.

Therefore, an ACE permitting legitimate TCP return traffic must appear before a broader ACE that denies unauthorized traffic to the Accounting & Finance network.

For example, consider the following logical order:

- Permit authorized Executive Management access to Accounting & Finance.

- Permit required ICMP reply traffic.

- Permit legitimate established TCP return traffic to Accounting & Finance.

- Deny other unauthorized traffic attempting to reach Accounting & Finance.

If the broader deny statement is placed before the established TCP permit statement, the return traffic will match the deny first and will never reach the permit statement.

Important:

ACL sequence and placement are part of the security design. An ACL can contain all of the correct ACEs and still produce an incorrect result if those ACEs are arranged in the wrong order.

☐ Protect enterprise financial resources while maintaining legitimate return communications.

At a minimum:

- Configure the named Extended IPv4 ACL ACCOUNTING-ACL to enforce the enterprise Accounting & Finance access policy.

- Configure the named IPv6 ACL ACCOUNTING-IPV6-ACL to enforce the equivalent IPv6 access policy.

- Permit the Executive Management subnet to initiate communications with Accounting & Finance resources using both IPv4 and IPv6.

- Permit required reply traffic necessary to maintain legitimate communications initiated by Accounting & Finance clients.

- Include an appropriate TCP ACE using the established keyword so that legitimate TCP return traffic can reach Accounting & Finance clients.

- Ensure that the TCP established permit ACE is positioned before the broader ACE that denies unauthorized access to the Accounting & Finance network.

- Deny unauthorized enterprise traffic attempting to access the Accounting & Finance subnet using both IPv4 and IPv6.

- Apply ACCOUNTING-ACL and ACCOUNTING-IPV6-ACL according to enterprise ACL placement best practices.

- Verify the ACL sequence and interface placement using the appropriate Cisco IOS show commands.

- Verify both authorized and unauthorized access to Accounting & Finance resources.

- Verify that an Accounting & Finance workstation can initiate an approved TCP connection and successfully receive the corresponding return traffic.

- Perform the required validation using both IPv4 and IPv6.

- Troubleshoot and correct any access-control issues before proceeding.

### Validate the Accounting & Finance Security Policy

Before proceeding to the next implementation phase, verify that the ACL protects Accounting & Finance resources without disrupting legitimate communications initiated by Accounting & Finance users.

#### Step 1 – Verify Access to Accounting & Finance

Generate ping traffic to PC-AF1 from the specified enterprise clients using both IPv4 and IPv6.

Record the observed result.

### IPv4

| Source Device | Destination Device | Permit | Deny |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 | ☐ | ☐ |
| Phoenix PC-HR1 | Phoenix PC-AF1 | ☐ | ☐ |
| Phoenix PC-IT1 | Phoenix PC-AF1 | ☐ | ☐ |
| Tucson PC-WH1 | Phoenix PC-AF1 | ☐ | ☐ |
| Flagstaff PC-SCS1 | Phoenix PC-AF1 | ☐ | ☐ |

### IPv6

| Source Device | Destination Device | Permit | Deny |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 | ☐ | ☐ |
| Phoenix PC-HR1 | Phoenix PC-AF1 | ☐ | ☐ |
| Phoenix PC-IT1 | Phoenix PC-AF1 | ☐ | ☐ |
| Tucson PC-WH1 | Phoenix PC-AF1 | ☐ | ☐ |
| Flagstaff PC-SCS1 | Phoenix PC-AF1 | ☐ | ☐ |

Expected Security Policy

- Executive Management → Accounting & Finance: Permit

- All other tested enterprise departments → Accounting & Finance: Deny

If the observed results do not match the approved Enterprise Security Policy, troubleshoot the ACL configuration, ACE order, interface placement, and direction before proceeding.

#### Step 2 – Verify Legitimate Traffic Initiated by Accounting & Finance

Protecting Accounting & Finance does not mean that Accounting & Finance clients should be isolated from resources they are authorized to use.

From PC-AF1, initiate an HTTPS connection to ISP-WEB1 using IPv4 and IPv6.

### IPv4

Open the Web Browser on PC-AF1 and access: https://198.51.100.10

| Source Device | Destination Resource | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-AF1 | ISP-WEB1 – 198.51.100.10 | ☐ | ☐ |

### IPv6

Open the Web Browser on PC-AF1 and access: https://[2001:DB8:FFFF:1::10]

Record the observed results.

| Source Device | Destination | Success | Fail |
| --- | --- | --- | --- |
| Phoenix PC-AF1 | ISP-WEB1 – 2001:DB8:FFFF:1::10 | ☐ | ☐ |

Both tests should be successful.

If PC-AF1 can initiate the connection but the application does not successfully receive return traffic, examine the Accounting & Finance ACLs carefully.

Consider:

- Is the required return traffic permitted?

- Is the TCP established ACE present?

- Is the established ACE positioned before the broader deny ACE?

- Are the IPv4 and IPv6 ACLs applied to the correct interface and in the correct direction?

- Does the ACL protect Accounting & Finance without unintentionally blocking legitimate responses?

Use the appropriate Cisco IOS show commands to examine the ACLs and their sequence before making configuration changes.

☐ Confirm that the Accounting & Finance security policy protects confidential resources while preserving legitimate IPv4 and IPv6 communications initiated by Accounting & Finance clients.

### Phase 3 – Secure External Enterprise Web Access

Background:

The Internet Service Provider has deployed a secure HTTPS server for enterprise business applications. The server has been preconfigured and tested. To reduce the enterprise attack surface and support approved business policies, Warehouse & Inventory workstations are not authorized to access the external HTTPS service using either IPv4 or IPv6.

The external HTTPS server is configured with the following network information:

- Server Name: ISP-WEB1

- IPv4 Address: 198.51.100.10

- IPv6 Address: 2001:DB8:FFFF:1::10/64

- Service: HTTPS (TCP port 443)

☐ Secure external enterprise web access.

At a minimum:

- Configure extended IPv4 ACL 110 to enforce the enterprise IPv4 web access policy.

- Configure the named IPv6 ACL WEB-IPV6-ACL to enforce the equivalent enterprise IPv6 web access policy.

- Permit authorized enterprise subnets to access the external HTTPS server.

- Deny HTTPS access from the Warehouse & Inventory subnet using both IPv4 and IPv6.

- Apply IPv4 ACL 110 and WEB-IPV6-ACL according to enterprise ACL placement best practices.

- Verify that authorized enterprise departments can successfully access the external HTTPS server using IPv4 and IPv6.

- Verify that the Warehouse & Inventory subnet is denied HTTPS access using both IPv4 and IPv6 while maintaining all other required enterprise network connectivity.

- Troubleshoot and correct any access control issues before proceeding.

## Enterprise Verification & Validation

After completing all implementation phases, verify that the Enterprise Change Request has been successfully implemented. This section serves as the official validation package submitted to the Director of Network Engineering to demonstrate that the requested changes were implemented successfully.

### Validation Evidence Requirements

Each required validation test must include sufficient evidence for the Director of Network Engineering to verify the results.

Include the source device in every screenshot.

Ensure the destination address or device is visible.

Include the complete command output or ping results.

If a required test is unsuccessful, include a screenshot of the failed test and record the result as Failed. Do not omit failed validation tests.

Provide the following evidence.

#### Step 1 – Verify Enterprise Security Policies

☐ Verify that enterprise security policies have been successfully implemented using IPv4 and IPv6 Access Control Lists.

### Background

Following the implementation of enterprise Access Control Lists, verify that authorized users can access the required network resources while unauthorized access is denied according to the Enterprise Network Standards Guide. Successful validation should demonstrate that enterprise security policies have been implemented without disrupting authorized business communications.

Important:

Record the IPv4 and IPv6 addresses assigned to each enterprise client. This information will be used throughout the remaining validation activities to verify enterprise connectivity and troubleshoot addressing issues if necessary.

### Verify SSH Management Access (IPv4)

Verify that Information Technology can successfully establish SSH connections to enterprise network devices using IPv4. Use PC-IT1 as the source device for the required validation tests. Also verify that unauthorized departments are denied remote management access.

At a minimum, perform and document the following validation tests.

| Destination Device | Expected Result | IPv4 Screenshot |
| --- | --- | --- |
| HQ-R1 | Permit |  |
| HQ-S1 | Permit |  |
| HQ-S2 | Permit |  |
| TUC-R2 | Permit |  |
| TUC-S3 | Permit |  |
| FLG-R3 | Permit |  |
| FLG-S4 | Permit |  |

### Unauthorized SSH Validation

In addition to the successful SSH validation tests above, verify that unauthorized enterprise departments are denied remote management access according to the Enterprise Network Standards Guide.

At a minimum, perform and document the following unsuccessful SSH connection attempts.

#### IPv4 Unauthorized SSH Validation

| Source Department | Destination Device | Expected Result | IPv4 Screenshot |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | HQ-R1 | Deny |  |
| Phoenix PC-AF1 | TUC-R2 | Deny |  |

#### Step 2 – Verify Accounting & Finance Access Protection

## Verify Accounting & Finance Access (IPv4)

Verify that the Accounting & Finance access control policy has been successfully implemented for both IPv4 and IPv6. Executive Management should be permitted to access Accounting & Finance resources, while all other enterprise departments should be denied access.

| Source Department | Destination Device | Expected Result | IPv4 Screenshot |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 | Permit |  |
| Phoenix PC-HR1 | Phoenix PC-AF1 | Deny |  |
| Phoenix PC-IT1 / | Phoenix PC-AF1 | Deny |  |
| Tucson PC-WH1 / | Phoenix PC-AF1 | Deny |  |
| Flagstaff PC-SCS1 | Phoenix PC-AF1 | Deny |  |

### Verify Accounting & Finance Access (IPv6)

Verify that the Accounting & Finance access control policy has been successfully implemented using IPv6. Authorized users should be permitted access while unauthorized enterprise departments should be denied according to the Enterprise Network Standards Guide.

At a minimum, perform and document the following validation tests.

| Source Device | Destination Device | Expected Result | IPv6 Screenshot |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | Phoenix PC-AF1 | Permit |  |
| Phoenix PC-HR1 | Phoenix PC-AF1 | Deny |  |
| Phoenix PC-IT1 | Phoenix PC-AF1 | Deny |  |
| Tucson PC-WH1 | Phoenix PC-AF1 | Deny |  |
| Flagstaff PC-SCS1 | Phoenix PC-AF1 | Deny |  |

#### Step 3 – Verify Enterprise HTTPS Access

Verify that the external HTTPS access control policy has been successfully implemented for both IPv4 and IPv6. Authorized enterprise departments should be permitted to access ISP-WEB1, while Warehouse & Inventory devices should be denied HTTPS access.

At a minimum, perform and document the following validation tests.

#### Verify External HTTPS Access – IPv4

To test external HTTPS access using IPv4, open the Web Browser on the designated source end device and enter the IPv4 address of ISP-WEB1 using the following URL:

https://198.51.100.10

| Source Department | Destination Device | Expected Result | IPv4 Screenshot |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | ISP-WEB1 | Permit |  |
| Phoenix PC-HR1 | ISP-WEB1 | Permit |  |
| Phoenix | ISP-WEB1 | Permit |  |
| PHOENIX PC-SCS1 | ISP-WEB1 | Permit |  |
| PC-IT1 / Information Technology | ISP-WEB1 | Permit |  |
| Flagstaff PC-SLS1 | ISP-WEB1 | Permit |  |
| Tucson PC-WH1 | ISP-WEB1 | Deny |  |

#### Verify External HTTPS Access – IPv6

To test external HTTPS access using IPv6, open the Web Browser on the designated source end device and enter the IPv6 address of ISP-WEB1 using the following URL:

https://[2001:DB8:FFFF:1::10]

When accessing a web server directly by an IPv6 address, the IPv6 address must be enclosed in square brackets [ ] within the URL. The brackets distinguish the colons that are part of the IPv6 address from the colon used to identify a port in a URL.

Verify whether the webpage successfully loads or access is denied according to the expected result shown in the table below.

| Source Department | Destination Device | Expected Result | IPv6 Screenshot |
| --- | --- | --- | --- |
| Phoenix PC-EX1 | ISP-WEB1 | Permit |  |
| Phoenix PC-HR1 | ISP-WEB1 | Permit |  |
| Phoenix PC-AF1 | ISP-WEB1 | Permit |  |
| PHOENIX PC-SCS1 | ISP-WEB1 | Permit |  |
| Phoenix PC-IT1 | ISP-WEB1 | Permit |  |
| Flagstaff PC-SLS1 | ISP-WEB1 | Permit |  |
| Tucson PC-WH1 | ISP-WEB1 | Deny |  |

#### Step 4 – Verify Enterprise Access Control Lists

☐ Verify the enterprise Access Control List (ACL) implementation using the required show commands below.

### Background

Review the output to verify that the required IPv4 and IPv6 ACLs are configured correctly and that ACL match counters, when applicable, reflect the validation traffic generated during the required connectivity and access-control tests.

### Verify Enterprise IPv4 and IPv6 Access Control Lists

Review the output to verify that the required IPv4 and IPv6 ACLs are configured correctly and that ACL match counters reflect the validation traffic generated during the required connectivity and access-control tests.

### show access-lists

### Highlight the following:

- The named IPv4 and IPv6 ACLs implemented in this ECR

- Entries showing hit counts for permitted traffic

- Entries showing hit counts for denied traffic

- The ACCOUNTING-ACL ACE permitting established TCP return traffic and its associated hit count

- Entries demonstrating that the ACLs are actively processing traffic according to the Enterprise Security Policy

- Paste the complete output in the table provided below.

| Device | Required Output - Paste show access-lists |
| --- | --- |
| HQ-R1 |  |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-R2 |  |
| TUC-S3 |  |
| FLG-R3 |  |
| FLG-S4 |  |

#### Step 5 – Verify Final Device Configurations

☐ Verify the final configuration of each enterprise device.

### Background

The final device configurations provide the official implementation record for this Enterprise Change Request. The show running-config output will be used to verify compliance with the Enterprise Network Standards Guide, validate the successful implementation of enterprise IPv4 and IPv6 Access Control Lists (ACLs) , and assist with troubleshooting if necessary.

Important:

Before submitting your show running-config output, highlight the required configuration items identified below. Highlighting these items helps the Director of Network Engineering quickly verify compliance with this Enterprise Change Request and the Enterprise Network Standards Guide.

### Highlight the following:

## Routers (HQ-R1, TUC-R2, FLG-R3)

IPv4 access-class applied to VTY lines

IPv6 access-class applied to VTY lines

IPv4 ip access-group interface applications

IPv6 ipv6 traffic-filter interface applications

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-R1 |  |
| TUC-R2 |  |
| FLG-R3 |  |

### Highlight the following:

## Switches (HQ-S1, HQ-S2, TUC-S3, FLG-S4)

IPv4 ACL applied to the VTY lines using access-class

IPv6 ACL applied to the VTY lines using ipv6 access-class

| Device | Required Output - Paste show running-config |
| --- | --- |
| HQ-S1 |  |
| HQ-S2 |  |
| TUC-S3 |  |
| FLG-S4 |  |

#### Step 6 – Final Validation

☐ Confirm that the Enterprise Change Request has been completed successfully.

Verify that:

- All required configurations have been completed.

- All verification tests have passed.

- All device configurations have been saved.

- The enterprise network is operating according to the Enterprise Network Standards Guide.

## Deliverables

☐  Completed ECR document

☐  Enterprise Verification & Validation evidence

☐  Updated Enterprise Topology

☐  Updated Enterprise Implementation

Version 1: Updated 08/20/2026